3GPP TSG-RAN5 Meeting #101	R5-236271r2

# Chicago, The United States of America, 13th – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4026 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to discussion on R5-236254, there is no test case related check nonCriticalExtension IE in SIB1 for the UE Rel15 and Rel16.Some UE was crashed in comercial NW when they received nonCriticalExtension with including higher release IE in SIB1.To prevent this accident and contribute to more stable UE connectivity, we need to update test case 8.1.1.2.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of test case 8.1.1.2.4 to check whether the UE ignores nonCriticalExtension with including higher release IE in SIB1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be not enough test case to check whether the UE ignores nonCriticalExtension with including higher release IE in SIB1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-2 CR0413 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Modify the description in reason for change to indicate that this test case is for Rel15 and 16 UE Modify the description in test purpose Modify the conformance requirement to follow rel15 not rel17 Modify specific message contents of SIB1 to check some rel17 IE R2: Modify the description about test case number in reason for change |  |  |  |  |  |  |  |  |

[Start of change]

##### 8.1.1.2.4	RRC connection establishment / Extended and spare fields in SI

8.1.1.2.4.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives an optional spare or extended field in system information or non critical extension that it does not comprehend }

then { UE treats system information as if the spare or extended field or non critical extension was absent and system information is not ignored, and UE establishes an RRC connection }

}

8.1.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3.1, 10.1 and 10.3. Unless otherwise stated, these are Rel-15 requirements.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

[TS 38.331, clause 10.1]

The generic error handling defined in the subsequent clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE;

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

[TS 38.331 clause 10.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with clause 10.4.

8.1.1.2.4.3	Test description

8.1.1.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

None.

Preamble:

-	The UE is switched off

8.1.1.2.4.3.2	Test procedure sequence

Table 8.1.1.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. (Note 1) | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCSetupRequest message on NR Cell 1? | --> | RRCSetupRequest | 1 | P |
| 4 | The SS transmits an RRCSetup message on NR Cell 1. | <-- | RRCSetup | - | - |
| 5 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and initiates the registration procedure by including the REGISTRATION REQUEST message? | --> | RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 6-21a1 | Steps 5 to 20a1 of the registration procedure described in TS 38.508 Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| Note 1: The NR-2 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used with SIB1 according to Table 8.1.1.2.4.3.3-2A and SystemInformation according to Table 8.1.1.2.4.3.3-2. |  |  |  |  |  |

8.1.1.2.4.3.3	Specific message contents

Table 8.1.1.2.4.3.3-1: MIB (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MIB ::= SEQUENCE { |  |  |  |  |
| spare | ‘1’B | In Rel-15 and Rel-16, the lowest bit is considered a spare according to TS 38.331 [12]. | Rel-15 to Rel-16 |
| } |  |  |  |

Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.6.1-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| systemInformation SEQUENCE { |  |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  |  |  |  |
| SIB2 |  | SIB2 |  |  |
| SIB21-r17 |  | SIB21 (Table 8.1.1.2.4.3.3-4) | In Rel-15 and Rel-16, SIB21 is considered an extended field according to TS 38.331 [12]. |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.1.2.4.3.3-2A: SIB1 (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| si-SchedulingInfo::= SEQUENCE { |  | SI-SchedulingInfo |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |  |
| schedulingInfo[1] SEQUENCE { |  | entry 1 |  |  |
| si-BroadcastStatus | broadcasting |  |  |  |
| si-Periodicity | rf32 |  |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 2 entries |  |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |  |
| type | sibType2 |  |  |  |
| valueTag | 0 |  |  |  |
| areaScope | Not present |  |  |  |
| } |  |  |  |  |
| SIB-TypeInfo[2] SEQUENCE { |  | entry 2 |  |  |
| type | spare3 | To simulate an extended SI that is undefined for a Rel-15 and Rel-16 UE. |  |  |
| valueTag | 0 |  |  |  |
| areaScope | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | SIB1-v1610-IEs |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | SIB1-v1630-IEs |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | SIB1-v1700-IEs |  |
| hsdn-Cell-r17 |  | true | Handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |  |

Table 8.1.1.2.4.3.3-3: Void

Table 8.1.1.2.4.3.3-4: SIB21 (Table 8.1.1.2.4.3.3-2)

| Derivation Path: TS 38.508-1, Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |  |
| mbs-FSAI-IntraFreq-r17 |  | Not Present |  |  |
| mbs-FSAI-InterFreqList-r17 |  | Not Present |  |  |
| lateNonCriticalExtension |  | Not Present |  |  |
| } |  |  |  |  |

[End of change]