3GPP TSG-RAN5 Meeting #101	R5-236275r1

# Chicago, The United States of America, 13th – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5261 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 22.4.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to discussion on R5-236254, there is no test case related check nonCriticalExtension IE in SIB1 for the UE Rel15 and Rel16.Some UE was crashed in comercial NW when they received nonCriticalExtension with including higher release IE in SIB1.To prevent this accident and contribute to more stable UE connectivity, we need to update test case 22.4.26 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of test case 22.4.26 to check whether the UE ignores nonCriticalExtension with including higher release IE in SIB1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be not enough test case to check whether the UE ignores nonCriticalExtension with including higher release IE in SIB1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-2 CR1426 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Modify the description in reason for change to indicate that this test case is for Rel15 and 16 UE Modify the description in test purpose Modify the conformance requirement to follow rel15 not rel17 Add description of release about nonCriticalExtension IE on comments in specific message contents |  |  |  |  |  |  |  |  |

[Start of change]

### 22.4.26	NB-IoT / RRC connection establishment / Extended and spare fields in SI

22.4.26.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives an optional spare or extended field in system information or non critical extension that it does not comprehend }

then { UE treats system information as if the spare or extended field or non critical extension was absent and system information is not ignored, and UE establishes an RRC connection }

}

22.4.26.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3, 5.7.1 and 5.7.3.

[TS 36.331 clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_INACTIVE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType25;

2>	if in RRC_CONNECTED; and

2>	the UE is not a BL UE; and

2>	the UE is not in CE; and

2>	the UE is not a NB-IoT UE:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT SystemInformationBlockType22-NB;

1>	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1>	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12 and systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

[TS 36.331 clause 5.7.1]

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved field.

[TS 36.331 clause 5.7.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 5.7.4;

22.4.26.3	Test description

22.4.26.3.1	Pre-test conditions

System Simulator:

-	 Ncell 1

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

22.4.26.3.2	Test procedure sequence

Table 22.4.26.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. See NOTE | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCConnectionRequest-NB message on Ncell 1? | --> | RRCConnectionRequest-NB | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup-NB message on Ncell 1. | <-- | RRCConnectionSetup-NB | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-15 | Steps 5 to 14b1 of the registration procedure described in TS 36.508 subclause 8.1.5.2 are performed on Ncell 1 | - | - | - | - |
| NOTE: sib-TypeAndInfo-r13 in SystemInformation-NB is set to values in Rel-17 to check spare and extended fields in UE up to Rel-16. |  |  |  |  |  |

22.4.26.3.3	Specific message contents

Table 22.4.26.3.3-1: Message SystemInformation-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformation-NB::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r13 SEQUENCE { |  |  |  |
| sib-TypeAndInfo-r13 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { | 2 entries |  |  |
| sib2-r13[1] | SystemInformationBlockType2-NB-r13 | SIB2-NB |  |
| sib32-v1700 [2] SEQUENCE { |  | In the ASN.1, sib32-v1700 is considered an extended field in Rel-13 to Rel-16 | Rel-13 to Rel-16 |  |  |
| satelliteInfoList-r17 | Not present |  |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.26.3.3-2: Void

Table 22.4.26.3.3-3: Message MasterInformationBlock-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |  |
| partEARFCN-17 CHOICE { |  |  |  |  |  |
| spare | ‘11’B |  |  |  |  |
| } |  |  |  |  |  |
| spare | ‘111111’B | From Rel-13 to Rel-16, the lowest 6 bits are considered spares according to TS 36.331 [17]. | Rel-13 to Rel-16 |
| } |  |  |  |

Table 22.4.26.3.3-3A: Message MasterInformationBlock-TDD-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-1A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MasterInformationBlock-TDD-NB-r15 ::= SEQUENCE { |  |  |  |  |
| spare | ‘1111 1111’B | In Rel-15 and Rel-16 the lowest 8 bits are considered spares according to TS 36.331 [17]. | TDD-NB Rel-15 to Rel-16 |
| } |  |  |  |

Table 22.4.26.3.3-4: Message SystemInformationBlockType1-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-1A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1350 |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1430 |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1450 |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1530 |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1610 |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-NB-v1700 |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred | Handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]