3GPP TSG-RAN5 Meeting #101	R5-236325r1

# Chicago, Illinois, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0102 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.579-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 24.380 and 24.581 the order of the fields in floor control and transmission control messages is irrelevant. - R5-236602 (TS 36.579-1 CR 0329) introduces default message content for MCPTT-Regroup what requires use of XML schema of 24.379 F.7.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification added to clause 8.1 that codec implementations shall cope with messages having fields in a different order than in TTCN-3 types. - TS24379_preconfiguredRegroup added to Table D.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Codec implementations may not decode messages correctly which have fields in a different order than specified. Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, D.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-1 CR 0329 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 8.1	Codec requirements

In addition to the requirements specified in TS 36.523-3 [27] clause 8.1 and TS 34.229-3 [28] clause 7 the codec requirements of table 8.1-1 shall be applied.

Table 8.1-1: Codec requirements

| Type definitions | Codec requirements | Encoding rule in TTCN-3 |
| --- | --- | --- |
| MIKEY_TypeDefs | RFC 3830 [64] clause 6 with extensions in RFC 6509 [65] and RFC 6043 [66] | "MIKEY-Codec" |
| MIKEY_MCX_Extension_TypeDefs | TS 33.180 [43] Annex E.6 | "Key Parameter Payload Types" |
| MCDataMessage_TypeDefs | TS 24.282 [57] clause 15 | "MCData Messages" |
| MCPTT_MediaControl_TypeDefs |  |  |
| FloorControl_Group | TS 24.380 [10] clause 8.2 | "FloorCtrlCodec" (NOTE 1) |
| PreestablishedSessionControl_Group | TS 24.380 [10] clause 8.3 | "PreestablishedSessionCtrlCodec" (NOTE 1) |
| MCVideo_TransmissionControl_TypeDefs | TS 24.581 [56] clause 9 | "TransmissionCtrlCodec" (NOTE 1) |
| JSON, JSON_AliasDefinitions | ES 201 873-11 [70] annex B, B.2 | "JSON" |
| NOTE 1: The TTCN-3 type definitions of floor control and transmission control messages are record types with the order of the fields being the same as specified in TS 24.380 [10] clause 8 and TS 24.581 [56] clause 9. Nevertheless according to TS 24.380 [10] and TS 24.581 [56] "With the exception of the three first 32-bit words the order of the fields are irrelevant". Therefore codec implementations shall be able to decode messages with fields being in a different order than in the TTCN-3 types and shall encode the messages with the fields being in the order given by the TTCN-3 types. |  |  |

<End of modification>

<Start of modification>

# D.1	XML Schema Definitions (XSD)

Common XML schema definitions according to TS 34.229-3 [28] Table G.0.1-1 are used. In addition there are the MCX specific XML schema definitions as according to table D.1-1.

Table D.1-1: MCX specific definitions

| XML Schema (XSD) | Source | Name space |
| --- | --- | --- |
| RFC4354-poc-settings | RFC 4354 [52] clause 6.1 | urn:oma:params:xml:ns:poc:poc-settings |
| RFC4661-SimpleFilter | RFC 4661 [34] clause 7 | urn:ietf:params:xml:ns:simple-filter |
| RFC5261-patch-ops (NOTE 2) | RFC 5261 [50] clause 8 | (NOTE 1) |
| RFC5874-xcap-diff | RFC 5874 [51] clause 4 | urn:ietf:params:xml:ns:xcap-diff |
| IANA-resource-lists | RFC 4826 [34] clause 3.2 https://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd | urn:ietf:params:xml:ns:resource-lists |
| poc_listService-v1_0 | OMA [37] | urn:oma:xml:poc:list-service |
| xdm_extensions-v1_0 | OMA [39] | urn:oma:xml:xdm:extensions |
| xdm_rsrclst_uriusage-v1_0 | OMA [40] | urn:oma:xml:xdm:resource-list:oma-uriusage |
| xenc-schema | W3C [41] | http://www.w3.org/2001/04/xmlenc#' |
| xmldsig-core-schema | W3C [42] | http://www.w3.org/2000/09/xmldsig# |
| TS24281_mcvideoinfo (NOTE 3) | TS 24.281 [55] Annex F.1.2 | urn:3gpp:ns:mcvideoInfo:1.0 |
| TS24281_mcvideolocationinfo | TS 24.281 [55] Annex F.3.2 | urn:3gpp:ns:mcvideoLocationInfo:1.0 |
| TS24281_mcvideoPresInfo | TS 24.281 [55] Table 8.3.1.2-1 | urn:3gpp:ns:mcvideoPresInfo:1.0 |
| TS24282_mcdatainfo | TS 24.282 [57] Annex D.1.2 | "urn:3gpp:ns:mcdataInfo:1.0" |
| TS24282_mcdatalocationinfo | TS 24.282 [57] Annex D.4.2 | urn:3gpp:ns:mcdataLocationInfo:1.0 |
| TS24282_mcdataPresInfo | TS 24.282 [57] Table 8.4.1.2-1 | "urn:3gpp:ns:mcdataPresInfo:1.0" |
| TS24282_mcdataPresInfoFA | TS 24.282 [57] Table 22.3.1.2-1 | "urn:3gpp:ns:mcdataPresInfoFA:1.0" |
| TS24379_mcpttaff (NOTE 4) | TS 24.379 [9] Annex F.4.2 TS 24.281 [55] Annex F.4.2 TS 24.282 [57] Annex D.3.2 | urn:3gpp:ns:affiliationCommand:1.0 |
| TS24379_mcpttinfo | TS 24.379 [9] Annex F.1.2 | urn:3gpp:ns:mcpttInfo:1.0 |
| TS24379_mcpttlocationinfo | TS 24.379 [9] Annex F.3.2 | urn:3gpp:ns:mcpttLocationInfo:1.0 |
| TS24379_mcpttsigneddoc | TS 24.379 [9] Annex F.6.2 | urn:3gpp:ns:mcpttSignedDoc:1.0 |
| TS24379_preconfiguredRegroup | TS 24.379 [9] Annex F.7.2 | urn:3gpp:ns:preconfiguredRegroup:1.0 |
| TS24379_mcpttPresInfo | TS 24.379 [9] Table 9.3.1.2-1 | urn:3gpp:ns:mcpttPresInfo:1.0 |
| TS24379_mcpttPresInfoFA | TS 24.379 [9] Table 9A.3.1.2-1 | urn:3gpp:ns:mcpttPresInfoFA:1.0 |
| TS24379_poc-settings (NOTE 5) | TS 24.379 [9] Table 7.4.1.2.2-2 TS 24.281 [55] Table 7.4.1.2.2-2 | urn:3gpp:mcsSettings:1.0 |
| TS24481-mcptt-group | TS 24.481 [11] clause 7.2.4.2 | urn:3gpp:ns:mcpttGroupInfo:1.0 |
| TS24481-mcpttGMOP | TS 24.481 [11] clause 7.3.3 | urn:3gpp:ns:mcpttGMOP:1.0 |
| TS24481-mcpttGKTP | TS 24.481 [11] clause 7.7.4.2 | urn:3gpp:ns:mcpttGKTP:1.0 |
| TS24484-ue-init-config | TS 24.484 [14] clause 7.2.2.3 | urn:3gpp:mcptt:mcpttUEinitConfig:1.0 |
| TS24484- mcptt-ue-config | TS 24.484 [14] clause 8.2.2.3 | urn:3gpp:mcptt:mcpttUEConfig:1.0 |
| TS24484-mcvideo-ue-config | TS 24.484 [14] clause 9.2.2.3 | urn:3gpp:mcvideo:mcvideoUEConfig:1.0 |
| TS24484-mcdata-ue-config | TS 24.484 [14] clause 10.2.2.3 | urn:3gpp:mcdata:mcdataUEConfig:1.0 |
| TS24484-mcptt-user-profile | TS 24.484 [14] clause 8.3.2.3 | urn:3gpp:mcptt:user-profile:1.0 |
| TS24484-mcvideo-user-profile | TS 24.484 [14] clause 9.3.2.3 | urn:3gpp:ns:mcvideo:user-profile:1.0 |
| TS24484-mcdata-user-profile | TS 24.484 [14] clause 10.3.2.3 | urn:3gpp:ns:mcdata:user-profile:1.0 |
| TS24484-mcptt-service-config | TS 24.484 [14] clause 8.4.2.3 | urn:3gpp:ns:mcpttServiceConfig:1.0 |
| TS24484-mcvideo-service-config | TS 24.484 [14] clause 9.4.2.3 | urn:3gpp:ns:mcvideoServiceConfig:1.0 |
| TS24484-mcdata-service-config | TS 24.484 [14] clause 10.4.2.3 | urn:3gpp:ns:mcdataServiceConfig:1.0 |
| TS33180-mcsecKMSInterface | TS 33.180 [43] Annex D.3.5.1 | urn:3gpp:ns:mcsecKMSInterface:1.0 |
| TS33180-mcsecKMSKRR | TS 33.180 [43] Annex D.4.4 | urn:3gpp:ns:mcsecKMSKRR:1.0 |
| NOTE 1: The schema does not define any target namespace but is referred to as "urn:ietf:params:xml:schema:patch-ops" e.g. in the original include statement in urn:ietf:params:xml:ns:xcap-diff. NOTE 2: The XML schema in RFC 5261 [50] clause 8 is specified as DTD with ENTITY declarations being referred by the pattern of the schema. These references are resolved in RFC5261-patch-ops in order to get schema definitions which can be processed by TTCN-3 compilers. NOTE 3: The schema specifies import of non-existing namespace urn:3gpp:ns:mcvideoGKTP:1.0. Assumption: urn:3gpp:ns:mcpttGKTP:1.0 to be imported instead NOTE 4: The schemata in TS 24.379 [9] F.4.2, TS 24.281 [55] F.4.2 and TS 24.282 [57] Annex D.1.2 are effectively the same apart from the namespace prefix ("mcpttaff" vs. "mcvideoaff" vs. "mcdataaff"). Assumption: The namespace prefix does not matter in XML documents, in general any prefix can be used. TS24379_mcpttaff is used for MCPTT, MCVideo and MCData. NOTE 5: The schemata in TS 24.379 [9] Table 7.4.1.2.2-2 and TS 24.281 [55] Table 7.4.1.2.2-2 are the same. TS24379_poc-settings is used for MCPTT and MCVideo. |  |  |

In order to avoid ambiguities and unexpected side effects due to tool specific behaviour the schemaLocation in xs:import statements of XSD files are modified to use local references rather than references to internet locations.

The schemaLocations of imported namespaces are shown in table D.1-2. The schemaLocations of included namespaces are shown in table D.1-3.

Table D.1-2: schemaLocation of imported namespaces

| XML Schema | Imported name space | Original schemaLocation | Modified schemaLocation |
| --- | --- | --- | --- |
| RFC4354-poc-settings | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| RFC4661-SimpleFilter | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| IANA-resource-lists | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| poc_listService-v1_0 | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| poc_listService-v1_0 | urn:ietf:params:xml:ns:resource-lists | http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd | IANA-resource-lists.xsd (NOTE 2) |
| xdm_extensions-v1_0 | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/03/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| xenc-schema | http://www.w3.org/2000/09/xmldsig# | xmldsig-core-schema.xsd | (no change needed) |
| TS24282_mcdatainfo.xsd | http://www.w3.org/2001/04/xmlenc# | http://www.w3.org/TR/xmlenc-core/xenc-schema.xsd | xenc-schema.xsd |
| TS24379_mcpttsigneddoc | http://www.w3.org/2000/09/xmldsig# | http://www.w3.org/TR/xmldsig-core/xmldsig-core-schema.xsd | xmldsig-core-schema.xsd |
| TS24484-ue-init-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484- mcptt-ue-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcvideo-ue-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcdata-ue-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcptt-user-profile | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcptt-user-profile | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| TS24484-mcvideo-user-profile | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcvideo-user-profile | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| TS24484-mcdata-user-profile | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcdata-user-profile | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| TS24484-mcptt-service-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcdata-service-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| NOTE 1: The namespace urn:ietf:params:xml:ns:common-policy is specified in RFC 4745 and at http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd with difference in the minOccurs for identityType and validityType. Nevertheless for use in TTCN there shall be only one schema as part 9 of the TTCN-3 language [31] does not support different variants of one and the same namespace. In addition according to DIFF1 for RFC 4745 it seems that the RFC's variant is newer than the IANA's variant. NOTE 2: The namespace urn:ietf:params:xml:ns:resource-lists is specified in RFC 4826 and at http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd with the difference of attribute anchor not being mandatory in the RFC. |  |  |  |

Table D.1-3: schemaLocation of included namespaces

| XML Schema | Included schema | Original schemaLocation | Modified schemaLocation |
| --- | --- | --- | --- |
| RFC5874-xcap-diff | urn:ietf:params:xml:schema:patch-ops | urn:ietf:params:xml:schema:patch-ops | RFC5261-patch-ops.xsd |

<End of modification>