3GPP TSG-RAN5 Meeting #101	R5-236388r1

# Chicago, USA, 13rd – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | n/a | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NTN NB-IoT Maximum input level & ACS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NTN CAT.M1 & NB-IoT Maximum input level & ACS test case has not been completed due to insufficient test points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test configurations for NTN Maximum Input Level & ACS Test Cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 36.905 CR 0260 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct the cover page's release to Rel-18. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

7.4	Maximum input level

7.4A	Maximum input level for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.4A.1	Test purpose

Maximum input level tests the ability of category M1 UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

7.4A.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.4A.3	Minimum conformance requirements

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1) with parameters specified in Table 7.4A.3-1.

Table 7.4A.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: Reference measurement channel is TS 36.101 [7] Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101 [7]. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.4A.

7.4A.4	Test description

7.4A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT M1 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 7.4A.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 7.4A.4.1-1: Test Configuration Table RX test cases UE Cat-M1

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] clause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 4.3.1 |  |  |  | Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] clause 4.3.1 |  |  |  | 1.4MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| Ch BW | Mod'n | RB allocation |  |  | Mod'n | RB allocation |  | UE Category |
|  |  | FDD and HD-FDD | TDD |  |  | FDD and HD-FDD | TDD |  |
| 1.4MHz | 16QAM | 2 | 2 |  | QPSK | 6 | 6 | M1 |
| Note 1: Downlink RB position shall be RBstart = 0 within the narrowband Note 2: The Narrowband index (TS36.211, 5.2.4) shall be set to 0 for all testpoints. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.4A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 4.13 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB31 configuration in TS 36.508 [12].

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.4A.4.3.

7.4A.4.2	Test procedure

1.	SS transmits PDSCH via M-PDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.4A.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.4A.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value defined in Table 7.4A.5-1. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.4A.5-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

7.4A.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exception.

Table 7.4A.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UePUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-uePUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

Table 7.4A.4.3-2: EPDCCH-Config-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| mpdcch-NumRepetition-r13 | r1 |  |  |

7.4A.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.2.2, A.2.3 and A.3.2 with parameters specified in Table 7.4A.5-1.

Table 7.4A.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -402 +TT |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2A.4. NOTE 2: Reference measurement channel is TS36.521-1 [14] Annex A.3.2 64QAM R=3/4variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1. |  |  |

7.4B	Maximum input level for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.4B.1	Test purpose

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

7.4B.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.4B.3	Minimum conformance requirements

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

Category NB1 and NB2 UE maximum input level requirement is -40 dBm. For this input level the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 of TS 36.101 [7].

The normative reference for this requirement is TS 36.102 [11] clause 7.4B.

7.4B.4	Test description

7.4B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the set of E-UTRA operating bands defined for NB-IoT in clause 5.2.F. All of these configurations shall be tested with applicable test parameters, and are shown in table 7.4F.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 7.4B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | QPSK | 12 | BPSK | 1@0 | 15kHz |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.4B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 4.13 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB31 configuration in TS 36.508 [12].

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.4B.4.3.

7.4B.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.4B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send the HARQ feedback based on information contain in DCI format N1.

2.	Set the Downlink signal level to the value defined in Table 7.4B.5-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to TS36.521-1 [14] Annex G.2.

7.4B.4.3	Message contents

Message contents are according to TS 36.508 [12] clause 8.1.6.

7.4B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.3.2 with parameters specified in Table 7.4B.5-1.

Table 7.4B.5-1: Maximum input level for category NB1 and NB2

| Rx Parameter | Units | Maximum input level test requirement |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration | dBm | For carrier frequency f ≤ 3.0GHz: -40 +TT |

7.5	Adjacent Channel Selectivity (ACS)

7.5A	Adjacent Channel Selectivity for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.5A.1	Test purpose

Adjacent channel selectivity tests the ability of category M1 UE to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

7.5A.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.5A.3	Minimum conformance requirements

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirement specified in Table 7.5A.3-1 for all values of an adjacent channel interferer up to –40 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5A.3-2 and Table 7.5A.3-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1). For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.5A.3-1: Adjacent channel selectivity

|  |  | Channel bandwidth |
| --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz |
| ACS | dB | 33.0 |

Table 7.5A.3-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | 1.4 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |
| PInterferer | dBm | REFSENS +45.5dB |  |
| BWInterferer | MHz | 1.4 |  |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.5A.3-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -71.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.5A.

7.5A.4	Test description

7.5A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT M1 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.5A.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS36.521-1 [14] Annex C.2.

Table 7.5A.4.1-1: Test Configuration Table RX test cases UE Cat-M1

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] clause 4.1 |  |  | NC |  |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 4.3.1 |  |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] clause 4.3. |  |  | 1.4 |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD and HD-FDD | TDD |  | FDD and HD-FDD | TDD |
| 1.4MHz | QPSK | 4 | 4 | QPSK | 6 | 6 |
| Note 1: Downlink RB position shall be RBstart = 0 within the narrowband. Note 2: Use narrowband index (TS36.211, 5.2.4) 0 when interferer is below carrier, and max narrowband index when interferer is above carrier. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 4.13 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB31 configuration in TS 36.508 [12].

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 7.5A.4.3.

7.5A.4.2	Test procedure

1.	SS transmits PDSCH via M-PDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.5A.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 7.5A.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.5A.5-2 (Case 1). Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.5A.5-2 (Case 1) for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

4.	Set the Interferer signal level to the value as defined in Table 7.5A.5-2 (Case 1) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D of the present document.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal in Case 1 at step 4.

7.	Set the Downlink signal level to the value as defined in Table 7.5A.5-3 (Case 2). Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.5A.5-3 (Case 2) for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

8.	Set the Interferer signal level to the value as defined in Table 7.5A.5-3 (Case 2) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D of the present document.

9.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

10.	Repeat steps from 7 to 9, using an interfering signal above the wanted signal in Case 2 at step 8.

11.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

7.5A.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exception.

Table 7.5A.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UePUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-uePUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

7.5A.5	Test requirement

The throughput Rav shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.2.2, A.2.3 and A.3.2 with parameters specified in table 7.5A.5-2, and also under the conditions specified in table 7.5A.5-3.

Table 7.5A.5-1: Adjacent channel selectivity

|  |  | Channel bandwidth |
| --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz |
| ACS | dB | 33.0 |

Table 7.5A.5-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | 1.4 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |
| PInterferer | dBm | REFSENS +45.5dB |  |
| BWInterferer | MHz | 1.4 |  |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in clause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in TS36.521-1 [14] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 and 7.3A-2 should be used as REFSENS for the power in Transmission Bandwidth Configuration and PInterferer. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.5A.5-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -71.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L with PCMAX_L as defined in clause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in TS36.521-1 [14] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |

7.5B	Adjacent Channel Selectivity for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.5B.1	Test purpose

Adjacent channel selectivity tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

7.5B.2	Test applicability

This test case applies to all types of E-UTRA HD-FDD UE release 17 and forward of category NB1 and NB2 that support satellite access.

7.5B.3	Minimum conformance requirements

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Category NB1 and NB2 UE shall fulfil the minimum requirement specified in Table 7.5B.3-1 for all values of an adjacent channel interferer up to -40 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5B.3-1 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2.

Table 7.5B.3-1: Adjacent channel selectivity parameters for category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + 42 dB | REFSENS + 47 dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | -68 dBm | -73 dBm |
| Interferer signal power (PInterferer ) / dBm | -40 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 7.5B.

7.5B.4	Test description

7.5B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT NB1 and NB2 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.5B.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS36.521-1 [14] Annex C.2.

Table 7.5B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | QPSK | 12 | BPSK | 1@0 | 15kHz |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5B.4.1-1.

5	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 4.13 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB31 configuration in TS 36.508 [12].

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 7.5B.4.3.

7.5B.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.5B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

2.	Set the Downlink signal level to the value defined for ACS1, GSM in Table 7.5B.5-1. For steps 2 to 5 and 6 to 9, use the default message contents.

3.	Set the Interferer signal level to the value defined for ACS1, GSM in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer bandwidth as defined in Annex D.2 of the present document.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

5.	Repeat steps 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Set the Downlink signal level to the value defined for ACS1, E-UTRA in Table 7.5B.5-1.

7.	Set the Interferer signal level to the value defined for ACS1, E-UTRA in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer bandwidth as defined in Annex D.2 of the present document.

8.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

9.	Repeat steps 6 to 8, using an interfering signal above the wanted signal at step 7.

10.	Release the connection through State 3A-NB.

11.	Modify system information elements according to Table 7.5B.4.3-1 and notify the UE via paging message with SystemInformationModification included.

12.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the new UL power control setting.

13.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.5B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

14.	Set the Downlink signal level to the value defined for ACS2, GSM in Table 7.5B.5-1. For steps 14 to 17 and 18 to 21, use message contents with the exceptions defined in Table 7.5B.4.3-1.

15.	Set the Interferer signal level to the value defined for ACS2, GSM in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer of 5MHz bandwidth defined in Annex D.2 of the present document.

16.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

17.	Repeat steps 14 to 16, using an interfering signal above the wanted signal at step 15.

18.	Set the Downlink signal level to the value defined for ACS2, E-UTRA in Table 7.5B.5-1.

19.	Set the Interferer signal level to the value defined for ACS2, E-UTRA in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer of 5MHz bandwidth defined in Annex D.2 of the present document.

20.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

21.	Repeat steps 18 to 20, using an interfering signal above the wanted signal at step 19.

7.5B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.6 with the following exceptions:

Table 7.5B.4.3-1: UplinkPowerControlCommon-NB-DEFAULT: ACS2

| Derivation Path: TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { p0-NominalNPUSCH-r13 | -70 |  |  |

7.5B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.3.2 with parameters specified in Table 7.5B.5-1.

Table 7.5B.5-1: Test parameters for Adjacent channel selectivity, category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + 42 dB | REFSENS + 47 dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | -68 dBm | -73 dBm |
| Interferer signal power (PInterferer ) / dBm | -40 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

## << End of changes >>