3GPP TSG-RAN5 Meeting #101	R5-236506r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4068 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Huawei, Hisilicon, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose, the Steps between Step 3E and 9A3 have been introduced to accommodate a situation when a UE after PDU Session release can retry to camp on the cell. It is observed that sometimes the UE sends a De-registration REQ immediately after transmission of PDU Session Release Complete. More specifically, sometimes the De-registration REQ arrives before T2 is started. This causes testcase failure. Moreover, the TTCN implementation of the Steps to handle the emergency call release plus the implementation of T2 can also introduce delays, which don’t help the situation. This instability needs to be handled. The Steps 9A22- 23 do not add any value to the testcase. The TP2 gets achieved as soon as the step 9A3 – 9A21a1 are performed (which includes PDU Session Establishment). These can also be voided to optimize the testcase and improve the test case run time. Hence it is proposed to simplify this testcase implementation without compromising on the TPs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that after Step 2C, the SS is configured to stop RAR. This will help the TTCN implementation to handle to optional De-registration and also not compromise the TP2. Hence a new Step 2CA is introduced for this. Also new Step 3EA is introduced for rrc release, followed by new Step 3EB where SS is configured to transmit RAR again. It is proposed to void the Step 3F. Also updated the Note 2 to specify durig which steps the optional De-registration can happen. With this approach, the testcase implementation in TTCN will become stable, and will be able to accommodate all UE behaviours. Steps 9A22- 23 are voided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s230748 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-236506r1: Assigned verdict at Step 9A3 – 9A21a1 |  |  |  |  |  |  |  |  |

### 11.4.1	5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA)

11.4.1.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.NORMAL-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { UE is requested to make an outgoing call using an emergency number stored on the USIM }

then { UE establishes an RRC connection with the RRC establishmentCause set to "emergency", and, sends a SERVICE REQUEST message with Service type IE set to "emergency services", and, establishes a New emergency PDU session by sending an UL NAS TRANSPORT message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST }

}

(2)

with { UE in 5GMM-REGISTERED.NORMAL-SERVICE state and 5GMM-CONNECTED mode having established an Emergency call }

ensure that {

when { UE establishes that the network has failed the authentication check during a 5G AKA based primary authentication and key agreement procedure }

then { UE continues using the current security context, and, releases all non-emergency PDU sessions by initiating UE-requested PDU session release procedure and does not treat the active cell as barred for non-emergency services }

}

11.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [12], subclause 5.3.3.3, TS 24.501 [22], subclauses 5.6.1.2, 6.4.1.2, 5.4.1.3.7, TS 22.101 [42], subclause 10.1.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, subclause 5.3.3.3]

The UE shall set the contents of RRCSetupRequest message as follows:

...

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

[TS 24.501, subclause 5.6.1.2]

The UE initiates the service request procedure by sending a SERVICE REQUEST message to the AMF and starts timer T3517.

If the UE is sending the SERVICE REQUEST message from 5GMM-IDLE mode and the UE needs to send non-cleartext IEs, the UE shall send the SERVICE REQUEST message including the NAS message container IE as described in subclause 4.4.6.

...

For case c) in subclause 5.6.1.1, the Uplink data status IE shall not be included in the SERVICE REQUEST message except if the UE has one or more active always-on PDU sessions associated with the access type over which the SERVICE REQUEST message is sent. If the UE is not a UE configured for high priority access in selected PLMN and:

a)	if the SERVICE REQUEST message is triggered by a request for emergency services from the upper layer, the UE shall set the service type IE in the SERVICE REQUEST message to "emergency services"; or

[TS 24.501, subclause 6.4.1.2]

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

NOTE 0:	When IMS voice is available over either 3GPP access or non-3GPP access, the "voice centric" UE in 5GMM-REGISTERED state will receive a request from upper layers to establish the PDU session for IMS signalling, if the conditions for performing an initial registration with IMS indicated in 3GPP TS 24.229 [14] subclause U.3.1.2 are satisfied.

If the UE requests to establish a new PDU session, the UE shall allocate a PDU session ID which is not currently being used by another PDU session over either 3GPP access or non-3GPP access.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION ESTABLISHMENT REQUEST message to the allocated PTI value.

...

If the UE requests to establish a new emergency PDU session, the UE shall set the SSC mode IE of the PDU SESSION ESTABLISHMENT REQUEST message to "SSC mode 1".

...

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID of the PDU session being established, or being handed over or being transferred;

..

e)	the request type which is set to:

...

3)	"initial emergency request", if the UE requests to establish a new emergency PDU session; and

...

If the request type is set to "initial emergency request" or "existing emergency PDU session", neither DNN nor S-NSSAI is transported by the UE using the NAS transport procedure as specified in subclause 5.4.5.

[TS 22.101, subclause 10.1.1]

The ME shall identify an emergency number dialled by the end user as a valid emergency number and initiate emergency call establishment if it occurs under one or more of the following conditions. If it occurs outside of the following conditions, the ME should not initiate emergency call establishment but normal call establishment. Emergency number identification takes place before and takes precedence over any other (e.g. supplementary service related) number analysis.

...

b)	Any emergency call number stored on a SIM/USIM when the SIM/USIM is present.

[TS 24.501, subclause 5.4.1.3.7]

g)	Network failing the authentication check.

If the UE deems that the network has failed the authentication check, then it shall request RRC to locally release the RRC connection and treat the active cell as barred (see 3GPP TS 38.304 [28]). The UE shall start any retransmission timers (e.g. T3510, T3517 or T3521), if they were running and stopped when the UE received the first AUTHENTICATION REQUEST message containing an incorrect authentication challenge data causing authentication failure.

...

For items c, d, e, and f whether or not the UE is registered for emergency services:

...

The UE shall deem that the network has failed the authentication check or assume that the authentication is not genuine and proceed as described in item g above if any of the following occurs:

-	the timer T3520 expires;

...

For items c, d, e, and f:

...

If a UE has an emergency PDU session established or is establishing an emergency PDU session when timer T3520 expires, the UE shall not deem that the network has failed the authentication check and not behave as described in item g. Instead the UE shall continue using the current security context, if any, release all non-emergency PDU sessions, if any, by initiating UE-requested PDU session release procedure. If there is an ongoing PDU session establishment procedure, the UE shall release all non-emergency PDU sessions upon completion of the PDU session establishment procedure. The UE shall start any retransmission timers (e.g. T3510, T3517 or T3521) if:

-	they were running and stopped when the UE received the AUTHENTICATION REQUEST message and detected an authentication failure;

-	the procedures associated with these timers have not yet been completed.

The UE shall behave as if the UE is registered for emergency services.

11.4.1.3	Test description

11.4.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport..

UE:

-	The UE is equipped with a test USIM with USIM Configuration 20 as defined in TS 38.508-1 [4] Table 6.4.1-20 (USIM contains two Emergency Numbers: 144, 117).

Preamble:

-	Cells power level configuration in accordance with TS 38.508-1 [4], Table 6.2.2.1-3:

-	NR Cell 1 "Serving cell"

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1.

NOTE:	pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection > 0.

11.4.1.3.2	Test procedure sequence

Table 11.4.1.3.2-1: Main behaviour

| St | Procedure |  | Message Sequence |  |  |  |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S |  | Message |  |  |  |  |  |
| - | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise. |  | - |  | - |  |  | - | - |  |
| 1 | Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. (NOTE 1) |  | - |  | - |  |  | - | - |  |
| 2 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? |  | - |  | - |  |  | 1 | - |  |
| 2A | The SS initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message; the included ngKSI value is the same as the one used in the initial message SERVICE REQUEST sent in step 2. |  | <-- |  | AUTHENTICATION REQUEST |  |  | - | - |  |
| 2B | The UE sends an AUTHENTICATION FAILURE message with 5GMM cause #71 "ngKSI already in use" |  | --> |  | AUTHENTICATION FAILURE |  |  | - | - |  |
| 2BA | Wait for T3520 expires. |  | - |  | - |  |  | - | - |  |
| - | EXCEPTION: Depending on the number of non-IMS Emergency relevant PDUs active at this moment of time step 2C is repeated 1 or more times. |  | - |  | - |  |  | - | - |  |
| 2C | FOR i=1 TO i= pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection Repeat the PDU session release procedure specified in Table 11.4.1.3.2-2: Parallel behaviour. Depending on UE implementation the PDU session release procedures can run in parallel or in sequence. |  | - |  | - |  |  | - | - |  |
| 2CA | The SS is configured to Stop RA response |  | - |  | - |  |  | - | - |  |
| 3 | Make the UE release the emergency call. (NOTE 1) |  | - |  | - |  |  | - | - |  |
| 3A | Generic test procedure for MO Release of Voice Call / 5GS, as defined in Annex A.7 of TS 34.229-5 [41], is performed. |  |  |  |  |  |  |  |  |  |
| 3B | SS starts timer T1 = 5 seconds. |  | - |  | - |  |  | - | - |  |
| - | EXCEPTION: Steps 3Ba1 to 3Bb1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. |  | - |  | - |  |  | - | - |  |
| 3Ca1 | The UE transmits a PDU SESSION RELEASE REQUEST message. |  | --> |  | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE REQUEST |  |  | - | - |  |
| 3Ca2 | Stop timer T1. |  | - |  | - |  |  | - | - |  |
| 3Cb1 | Timer T1 expires. |  | - |  | - |  |  | - | - |  |
| 3D | The SS transmits a PDU SESSION RELEASE COMMAND message. |  | <-- |  | NR RRC: DLInformationTransfer NR NAS: PDU SESSION RELEASE COMMAND |  |  | - | - |  |
| 3E | Check: Does the UE transmit a PDU SESSION RELEASE COMPLETE message? |  | --> |  | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE |  |  | - | - |  |
| 3EA | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3EB | SS configures the RA Response. | - | - | - | - |
| 3F – 9A2 | Void | - | - | - | - |
| 9A3 – 9A21a1 | Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. |  | - |  | - |  |  | 2 | P |  |
| 9A22-23 | Void | - | - | - | - |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: The UE may optionally transmit a De-registration REQ during 3E – 3EA. |  |  |  |  |  |  |  |  |  |  |

Table 11.4.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a PDU SESSION RELEASE REQUEST message with PDU session ID equal to one of the IDs of an existing PDU session BUT different to the PDU session ID assigned to the Emergency PDU session in step 2, Table 11.4.1.3.2-1? | - | PDU SESSION RELEASE REQUEST | 2 | P |
| 2 | Check: Does the UE perform PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4]? | - |  | 2 | P |

11.4.1.3.3	Specific message contents

Table 11.4.1.3.3-1: Message AUTHENTICATION REQUEST (step 2A, Table 11.4.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI | The same ng-KSI assigned in the Preamble and indicated in the initial message SERVICE REQUEST sent in step 2 |  |  |

Table 11.4.1.3.3-2: Message AUTHENTICATION FAILURE (step 2B, Table 11.4.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM cause | ‘0100 0111’B | ngKSI already in use |  |