3GPP TSG-RAN5 Meeting #101	R5-236580r1

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5263 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MPS priority indication test case 8.1.3.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on agreed discussion paper R5-233201, new test case for Redirection with MPS Indication in LTE needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.1.3.16 - RRC connection release / Redirection to another E-UTRAN frequency / MPS Priority Indication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage will not be present for L to L MPS priority Indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.16 (New) |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1432 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-236580r1: 1. Fixed editorials. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.3.16	RRC connection release / Redirection to another E-UTRAN frequency / MPS Priority Indication

8.1.3.16.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including IE mpsPriorityIndication-r16 and an IE redirectedCarrierInfo with eutra frequency different from UE was on in RRC_CONNECTED state}

then { UE establishes an RRC connection with establishmentCause set to highPriorityAccess on the new frequency included in IE redirectedCarrierInfo }

}

8.1.3.16.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clause 5.3.3.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	if the UE is connected to EPC:

2>	set the ue-Identity as follows:

3>	if upper layers provide an S-TMSI:

4>	set the ue-Identity to the value received from upper layers;

3>	else:

4>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

2>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

3>	set the establishmentCause to highPriorityAccess

…

8.1.3.16.3	Test description

8.1.3.16.3.1	Pre-test conditions

System Simulator:

-	2 cells on different E-UTRA frequencies

-	Cell 1 is set to ''Serving cell''

-	Cell 3 is set to ''Non-Suitable Off cell''

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18].

8.1.3.16.3.2	Test procedure sequence

Table 8.1.3.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: Cell 3 as a "Suitable Neighbour cell". | - | - | - | - |
| 2 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | <-- | IP packet | - | - |
| 3 | Wait for 1 s after the IP packet has been transmitted in step 2. (Note 1) | - | - | - | - |
| 4 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including eutra of Cell 3 and IE mpsPriorityIndication). | <-- | RRCConnectionRelease | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 3 with establishmentCause set to ‘highPriorityAccess’? | --> | RRCConnectionRequest | 1 | P |
| 6-10 | Steps 4 to 8 of the generic radio bearer establishment procedure (TS 36.508 [18] 4.5.3.3-1) are executed. | - | - | - | - |
| - | EXCEPTION: Steps 11 and 12 can occur in any order | - | - | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3. | --> | RRCConnectionReconfigurationComplete |  |  |
| 12 | The UE loops back the IP packet received in step 2 on the DRB associated with the default EPS bearer context on Cell 3. | --> | IP packet | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE has received and forwarded the IP Packet transmitted by the SS in step 2 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 4 |  |  |  |  |  |

8.1.3.16.3.3	Specific message contents

Table 8.1.3.16.3.3-1: Conditions for specific message contents in tables 8.1.3.16.3.3-2 and 8.1.3.16.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.1.3.16.3.3-2: SystemInformationBlockType5 for cell 1 (preamble and all steps, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-3: SystemInformationBlockType5 for cell 3 (preamble and all steps, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq[n] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[n] | Not present |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[n] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-4: RRCConnectionRelease message (step 4, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| eutra | Downlink EARFCN of cell 3 |  |  |
| } |  |  |  |
| redirectedCarrierInfo {} | Not present |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v9e0 SEQUENCE { |  |  |  |
| eutra-v9e0 | Downlink EARFCN of cell 3 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mpsPriorityIndication-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.16.3.3-5: RRCConnectionRequest (step 5, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.163.3-6: CLOSE UE TEST LOOP (preamble, Table 8.1.3.16.3.2-1)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0010'B | 2 seconds |  |

<End of modified section>