3GPP TSG-RAN5 Meeting #101	R5-236597r2

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5267 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RLC test cases for NTN UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RLC test cases needs to be extended to cover execution with NTN device and the CR captures the changes required. NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. It is proposed to use this definition to define the search space for NTN and TN Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test cases to add a note and use the NPDCCH period bundle to make them applicable for NTN UEs Updated test case 22.3.2.2 with editorial correction and clarification on the timing of Step 27 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-NTN capable devices cannot be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.2, 22.3.2.3, 22.3.2.4, 22.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 : Updated test case 22.3.2.2 with editorial correction and clarification on the timing of Step 27 R2 : Removed changes for test case 22.3.2.1 as it requires more investigation |  |  |  |  |  |  |  |  |

#### 22.3.2.2	NB-IoT / AM RLC / Receiver status triggers

##### 22.3.2.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state, configured for enableStatusReportSN-Gap and using AM RLC }

ensure that {
  when { Reception failure of an RLC data PDU is detected }

then { UE initiates Status Reporting }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and using AM RLC }

ensure that {
  when { Polling from peer AM RLC entity is detected and the sequence number of the PDU that carries the Poll is less than VR(MS) }

then { UE initiates Status Reporting }

}

(3)

Void

(4)

with { UE in E-UTRA RRC_CONNECTED state and using AM RLC }

ensure that {
  when { the UE needs to send a Status Report and the UL grant is not large enough to accommodate the whole report }

then { UE includes as many NACK SNs in the Status Report as allowed by the UL grant }

}

##### 22.3.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.322 clause 5.2.3. Unless otherwise stated these are Rel-13 requirements.

[TS 36.322, clause 5.2.3]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC PDUs (or portions of them).

Except for NB-IoT, RRC configures whether or not the status prohibit function is to be used for an AM RLC entity. For NB-IoT, RRC configures whether or not the status reporting due to detection of reception failure of an RLC data PDU is to be used for an AM RLC entity.

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When a RLC data PDU with SN = x and the P field set to “1” is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the PDU is to be discarded as specified in subclause 5.1.3.2.2; or

-	if x < VR(MS) or x >= VR(MR):

-	trigger a STATUS report;

-	else:

-	delay triggering the STATUS report until x < VR(MS) or x >= VR(MR).

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an RLC data PDU, except for an NB-IoT UE not configured with enableStatusReportSN-Gap:

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reordering expires.

NOTE 2:	The expiry of t-Reordering triggers both VR(MS) to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after VR(MS) is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and deliver it to lower layer;

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and deliver it to lower layer;

When a STATUS PDU has been delivered to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the AMD PDUs with SN such that VR(R) <= SN < VR(MS) that has not been completely received yet, in increasing SN order of PDUs and increasing byte segment order within PDUs, starting with SN = VR(R) up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an AMD PDU for which no byte segments have been received yet::

-	include in the STATUS PDU a NACK_SN which is set to the SN of the AMD PDU;

-	for a continuous sequence of byte segments of a partly received AMD PDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend

-	set the ACK_SN to the SN of the next not received RLC Data PDU which is not indicated as missing in the resulting STATUS PDU.

##### 22.3.2.2.3	Test description

22.3.2.2.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble

-	The UE is in state 2B-NB according to [18] the exceptions listed in table 22.3.2.2.3.1-1 with test loop mode G closed.

Table 22.3.2.2.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit-r13 | ms4000 |
| enableStatusReportSN-Gap-r13 | true |

22.3.2.2.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number  ‘n’  in test procedure maps as :

UL SQN n maps to SQN X+n MOD 1024 &

DL SQN n maps to SQN Y+n MOD 1024

Table 22.3.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. Size of each RLC UL SDU #1 - #6 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. | - | - | - | - |
| 1 | The SS transmits 3 AMD PDUs with SN=0, 1, 2. The SS sets the P field of all the AMD PDUs to 0 | <-- | AMD PDU (SN=0, P=0) AMD PDU (SN=1, P=0) AMD PDU (SN=2, P=0) | - | - |
| 2 | In the search space of the 4th NPDCCH period bundle after the first transmission at step 1 the SS schedules 3 consecutive UL grants with a time spacing of 3 (1st for NTN) NPDCCH period bundles of size 328 bits. (Note 1, Note 6) | <-- | (UL grants, 328 bits) | - | - |
| 2A | The UE transmits RLC UL SDU#1. | --> | (RLC UL SDU#1) | - | - |
| 2B | The UE transmits RLC UL SDU#2. | --> | (RLC UL SDU#2) | - | - |
| 2C | The UE transmits RLC UL SDU#3. | --> | (RLC UL SDU#3) | - | - |
| 3 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 4 | Void. | - | - | - | - |
| 5 | The SS starts the UL periodic grant transmission of size 40 bits. (Note 2) | - | - | - | - |
| 6 | The SS transmits 1 AMD PDUs with SN= 4. The SS sets the P field to 0 | <-- | AMD PDU (SN=4, P=0) | - | - |
| 7 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? | --> | STATUS PDU | 1 | P |
| 8 | The SS stops to allocate any uplink grant. | - | - | - | - |
| 9 | In the search space of the 6th NPDCCH period bundle after step 8 the SS transmits 1 AMD PDU with SN=3. The SS sets the P field to 1 (Note 6). | <-- | AMD PDU (SN=3, P=1) | - | - |
| 10 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 9 the SS schedules 1 UL grant of size 40 bits. (Note 2, Note 6) | <-- | (UL grant, 40 bits) | - | - |
| 11 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN=5? | --> | STATUS PDU | 2 | P |
| 12 | In the search space of the next NPDCCH period bundle after the transmission at step 10 the SS schedules 2 UL grants of size 328 bits. (Note 1, Note 6) | <-- | (UL grant, 328 bits) | - | - |
| 13 | The UE transmits RLC UL SDU#4. | --> | (RLC UL SDU#4) | - | - |
| 14 | The UE transmits RLC UL SDU#5. | --> | (RLC UL SDU#5) | - | - |
| 15 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 16-23 | Void | - | - | - | - |
| 24 | The SS transmits an AMD PDU with SN=5 and P=0, and an AMD PDU with SN=11 and P=1. | <-- | AMD PDU (SN=5, P=0) AMD PDU (SN=11, P=1) | - | - |
| 25 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 24 the SS schedules an UL grant of size 72 bits. (Note 3, Note 6) | <-- | (UL Grant) | - | - |
| - | Steps 26a1 and 26b1 depend on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens | - | - | - | - |
| 26a1 | Check: Does the UE transmit a Status Report with ACK_SN=8 and 2 NACK_SNs: 6 and 7? | --> | STATUS PDU | 4 | P |
| 26b1 | Check: Does the UE transmit a Status Report with ACK_SN=10 and 4 NACK_SNs: 6, 7, 8 and 9? | --> | STATUS PDU | 4 | P |
| 27 | In the search space of the 6th NPDCCH period bundle from step 25 SS transmits an AMD PDU with SN=6 and P=1. (Note 6) | <-- | AMD PDU (SN=6, P=1) | - | - |
| 28 | In the search space of the 3rd NPDCCH period bundle after step 27 the SS schedules an UL grant of size 72 bits. (Note 4, Note 6) | <-- | (UL Grant) | - | - |
| 29 | Check: Does the UE transmit a Status Report with ACK_SN=12 and 4 NACK_SNs: 7, 8, 9 and 10? | --> | STATUS PDU | 4 | P |
| 30 | The SS transmits 4 AMD PDU with SN=7, 8, 9, 10. NOTE: AMD PDUs with SN 5 to 11 carry RLC DL SDU #6. It is segmented in 6*5 + L-(6*5) octets. | <-- | AMD PDU (SN=7, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=0) AMD PDU (SN=10, P=0) | - | - |
| 31 | In the search space of the 3rd NPDCCH period bundle after step 30 the SS schedules 1 UL grant of size 392 bits. (Note 5, Note 6) | <-- | (UL grant, 328 bits) | - | - |
| 32 | The UE loopbacks a STATUS PDU with SN_ACK=12 and the complete RLC UL SDU. | --> | (RLC UL SDU#6) | - | - |
| 33 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Short BSR subheader + 1 byte MAC PDU subheader, 1 byte Short BSR + 38 bytes MAC SDU(16-bit RLC AMD PDU header + 288-bit UL RLC SDU)). Note 2: UL grant of 40 bits (ITBS=3, =0, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and (8-bit short BSR + 2x 8-bit MAC PDU subheader + 4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 1bit padding). Note 3: UL grant of 72 bits (ITBS=2, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit (a Status Report with ACK_SN and 2 NACK_SNs (3x 8-bit MAC PDU subheader +8-bit Short BSR + 4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2 x (12-bit NACK_SN/E1/E2) + 1-bit Padding) ) or (a Status Report with ACK_SN and 4 NACK_SNs (8-bit MAC PDU subheader +4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 4 x (12-bit NACK_SN/E1/E2) +1-bit padding)). Note 4: UL grant of 72 bits (ITBS=2, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and 4 NACK_SNs (8-bit MAC PDU subheader + 4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 4 x (12-bit NACK_SN/E1/E2) +1-bit padding). Note 5: UL grant of 392 bits (ITBS=8, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Short BSR subheader + 4 bytes MAC PDU subheader + 1 byte Padding subheader, 1 byte Short BSR + 2 bytes MAC SDU(2 bytes RLC STATUS PDU) + 38 bytes MAC SDU(16-bit RLC AMD PDU header + 288-bit UL RLC SDU) + 2 bytes MAC Padding). Note 6: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2 |  |  |  |  |  |

22.3.2.2.3.3	Specific message contents

None.

#### 22.3.2.3	NB-IoT / AM RLC / In sequence delivery of upper layers PDUs/ Different numbers of length indicators

##### 22.3.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives duplicate AMD PDUs }

then { UE discards the duplicate AMD PDUs }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU with a SN gap }

then { UE sends STATUS PDU to request retransmissions of PDUs in the SN gap}

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives PDUs within a SN gap }

then { RLC reassembles and reorders the AMD PDUs and deliver them to the upper layer in sequence }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU or an AMD PDU segment with no LI field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU or an AMD PDU segment more than one LI field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

##### 22.3.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.322, clause 4.2.1.3.3. Unless otherwise stated these are Rel-13 requirements.

[TS 36.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives RLC data PDUs, it shall:

-	detect whether or not the RLC data PDUs have been received in duplication, and discard duplicated RLC data PDUs;

-	reorder the RLC data PDUs if they are received out of sequence;

-	detect the loss of RLC data PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the reordered RLC data PDUs and deliver the RLC SDUs to upper layer in sequence.

At the time of RLC re-establishment, the receiving side of an AM RLC entity shall:

-	if possible, reassemble RLC SDUs from the RLC data PDUs that are received out of sequence and deliver them to upper layer;

-	discard any remaining RLC data PDUs that could not be reassembled into RLC SDUs;

-	initialize relevant state variables and stop relevant timers.

[TS 36.322, clause 6.2.2.5]

Length: 11 bits for RLC UM, 11 bits or 15 bits for RLC AM. The length of the LI field for RLC AM is configured by upper layers.

The LI field indicates the length in bytes of the corresponding Data field element present in the RLC data PDU delivered/received by an UM or an AM RLC entity. The first LI present in the RLC data PDU header corresponds to the first Data field element present in the Data field of the RLC data PDU, the second LI present in the RLC data PDU header corresponds to the second Data field element present in the Data field of the RLC data PDU, and so on. The value 0 is reserved.

##### 22.3.2.3.3	Test description

22.3.2.3.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

-	None.

Preamble:

-	The UE is in state 2B-NB according to [18] the exceptions listed in table 22.3.2.3.3.1-1 with test loop mode G closed.

Table 22.3.2.3.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit-r13 | ms4000 |
| enableStatusReportSN-Gap-r13 | true |

22.3.2.3.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number  ‘n’  in test procedure maps as :

- UL SQN n maps to SQN X+n MOD 1024 &

- DL SQN n maps  to SQN Y+n MOD 1024

Table 22.3.2.3.3.2-1: Main Behaviour

| St |  | Procedure | Message Sequence |  | TP |  | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |  |
| - |  | The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. Size of each RLC UL SDU #1 - #3 is PRBS of 256 bits (32 octets), size of each RLC UL SDU #4 - #16 is PRBS of 80 bits (10 octets). The size of the RLC SDUs used in downlink is L. | - | - | - |  | - |
| 1 |  | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#1 without LI field. | <-- | AMD PDU#1 (SN=0) | - |  | - |
| 2 |  | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#1 without LI field. | <-- | AMD PDU#1 (SN=0) | - |  | - |
| 2A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 2 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) |  | <-- | (UL Grant) | - |  | - |  |
| 3 |  | Check: Does the UE transmit RLC UL SDU#1? | --> | (RLC UL SDU#1) | 1,4 |  | P |
| 4 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=1) | - |  | - |
| 5 |  | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#2, and the 1st part of RLC DL SDU#3. | <-- | AMD PDU#2 (SN=1) | - |  | - |
| 5A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 5 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) |  | <-- | (UL Grant) | - | - |  |  |
| 6 |  | Check: Does the UE transmit RLC UL SDU#2 with the poll bit set? | --> | (RLC UL SDU#2) | 1 |  | P |
| 7 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=2) | - |  | - |
| 8 |  | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#2, and the 1st part of RLC DL SDU#3. | <-- | AMD PDU#2 (SN=1) | - |  | - |
| 8A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 8 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) |  | <-- | (UL Grant) | - | - |  |  |
| 9 |  | Check: Does the UE transmit RLC UL SDU#2? | --> | (RLC UL SDU#2) | 1 |  | F |
| 10 |  | The SS transmits an AMD PDU to the UE. This PDU contains the 2nd part of RLC DL SDU#3. | <-- | AMD PDU#3 (SN=2) | - |  | - |
| 10A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 10 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) |  | <-- | (UL Grant) | - | - |  |  |
| 11 |  | Check: Does the UE transmit RLC UL SDU#3? | --> | (RLC UL SDU#3) | 1 |  | P |
| 12 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=3) | - |  | - |
| 13 |  | The SS transmits an AMD PDU to the UE. This PDU contains the last part of RLC DL SDU#6. | <-- | AMD PDU#6 (SN=5) | - |  | - |
| 14 |  | The SS transmits an AMD PDU to the UE. This PDU contains the 2nd part of RLC DL SDU#5, and the 1st part of RLC DL SDU#6. | <-- | AMD PDU#5 (SN=4) | - |  | - |
| 15 |  | The SS does not allocate any uplink grant. | - | - | - |  | - |
| 16 |  | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#4 and the 1st part of RLC DL SDU#5. | <-- | AMD PDU#4 (SN=3) | - |  | - |
| 17 |  | In the search space of the 3rd NPDCCH period bundle after step 16 the SS schedules an UL grant of size 504 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#4, RLC DL SDU#5 and RLC DL SDU#6 and report RLC Status PDU. (Note 2, Note 7) | <-- | (UL grant) | - |  | - |
| 18 |  | Check: Does the UE transmit a STATUS PDU with ACK_SN=6 and an AMD PDU containing RLC UL SDU#4, RLC UL SDU#5 and RLC UL SDU#6 in its data field? | --> | STATUS PDU and AMD PDU (RLC UL SDU#4, RLC UL SDU#5, RLC UL SDU#6) | 3 |  | P |
| 19 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=4) | - |  | - |
| 20 |  | The SS transmits an AMD RLC PDU to the UE. This PDU contains the last part of RLC DL SDU#9. | <-- | AMD PDU#9 (SN=8, P=1) | - |  | - |
| 20A | In the search space of the 3rd NPDCCH period bundle after step 20 the SS schedules an UL grant of size 88 bits. (Note 3, Note 7) |  | <-- | (UL Grant) | - | - |  |  |
| 21 |  | Check: Does the UE transmit a STATUS PDU NACK_SN/E1/E2 fields set correctly to inform SS of missing PDUs #7, #8, (ACK_SN=9, NACK_SN= 6, NACK_SN= 7)? | --> | STATUS PDU | 2 |  | P |
| 22 |  | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#8, and the 1st part of RLC DL SDU#9. | <-- | AMD PDU#8 (SN=7) | - |  | - |
| 23 |  | The SS does not allocate any uplink grant. | - | - | - |  | - |
| 24 |  | In the search space of the 3rd NPDCCH period bundle after step 24 the SS schedules The SS transmits an AMD PDU to the UE. This PDU carries SDU#7. (Note 7) | <-- | AMD PDU#7 (SN=6) | - |  | - |
| 24A |  | an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#7, RLC DL SDU#8 and RLC DL SDU#9 and report RLC Status PDU. (Note 4) | <-- | (UL grants) | - |  | - |
| 25 |  | Check: Does the UE transmit a STATUS PDU with ACK_SN=9 and an AMD PDU containing RLC UL SDU#7, RLC UL SDU#8 and RLC UL SDU#9 in its data field? | --> | STATUS PDU and AMD PDU (RLC UL SDU#7, RLC UL SDU#8, RLC UL SDU#9) | 3 |  | P |
| 26 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=5) | - |  | - |
| 27 |  | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#10, RLC DL SDU#11 and RLC DL SDU#12 with two LI fields. | <-- | AMD PDU#10 (SN=9) | - |  | - |
| 28 |  | In the search space of the 3rd NPDCCH period bundle after step 27 the SS schedules an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#10, RLC DL SDU#11 and RLC DL SDU#12. (Note 5, Note 7) | <-- | (UL grant) | - |  | - |
| 29 |  | Check: Does the UE transmit an AMD PDU containing RLC UL SDU#10, RLC UL SDU#11 and RLC UL SDU#12 in its data field? | --> | AMD PDU (RLC UL SDU#10, RLC UL SDU#11, RLC UL SDU#12) | 5 |  | P |
| 30 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=6) | - |  | - |
| 31 |  | The t-PollRetransmit-r13 timer for AMD PDU#11 expires and SS assumes that the transmission of AMD PDU#11 containing RLC DL SDU#13, RLC DL SDU#14, RLC DL SDU#15 and RLC DL SDU#16 is failed and considers AMD PDU#11 for re-transmission. | - | - | - |  | - |
| 32 |  | The SS transmits an AMD PDU segment to the UE. This PDU segment contains RLC DL SDU#13 without LI field. | <-- | AMD PDU segment (SM=10) | - |  | - |
| 33 |  | In the search space of the 3rd NPDCCH period bundle after step 32 the SS schedules an uplink grant of size 208 bits allowing the UE to transmit 1 RLC SDU. (Note 6, Note 7) | <-- | (UL grant) | - |  | - |
| 34 |  | Check: Does the UE transmit a STATUS PDU with NACK_SN=10, SOStart, SOEnd and ACK_SN=11 and an AMD PDU containing RLC UL SDU#13 in its data field? | --> | AMD PDU (RLC UL SDU#13) | 4 |  | P |
| 35 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=7) | - |  | - |
| 36 |  | The SS transmits AMD PDU segment to the UE. This PDU segment contains SDU#14, SDU#15 and SDU#16 with two LI fields. | <-- | AMD PDU segment | - |  | - |
| 37 |  | In the search space of the 3rd NPDCCH period bundle after step 32 the SS schedules an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#14, RLC DL SDU#15 and RLC DL SDU#16. (Note 5, Note 7) | <-- | (UL grant) | - |  | - |
| 38 |  | Check: Does the UE transmit an AMD PDU containing RLC UL SDU#14, RLC UL SDU#15 and RLC UL SDU#16 in its data field? | --> | AMD PDU (RLC UL SDU#14, RLC UL SDU#15, RLC UL SDU#16) | 5 |  | P |
| 39 |  | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=8) | - |  | - |
| NOTE 1: UL grant of 296 bits (ITBS=9, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Short BSR subheader + 1 byte MAC PDU subheader, 1 byte Short BSR + 34 bytes MAC SDU(16-bit RLC AMD PDU header + 256 bit UL RLC SDU)) NOTE 2: UL grant of 504 bits (ITBS=10, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report and a AMD PDU (a 4 byte Status Report with ACK_SN and 1 NACK_SN (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + (12-bit NACK_SN/E1/E2) + 5-bit Padding) or ACK_SN and 2 NACK_SN (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2*(12-bit NACK_SN/E1/E2) + 1-bit Padding) + a 35 bytes RLC AMD PDU(5 bytes RLC AMD PDU header + 3*10 bytes UL RLC SDU) + 6 bytes MAC PDU subheader + 1 byte Short BSR + n bytes MAC Padding). NOTE 3: UL grant of 88 bits (ITBS=6, =0, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and 2 NACK_SNs (4 bytes MAC PDU subheader + 1 byte Short BSR + 5 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2 x (12-bit NACK_SN/E1/E2) + 1-bit Padding) + 1 byte MAC Padding ). NOTE 4: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report and a AMD PDU. NOTE 5: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a AMD PDU (4 bytes MAC PDU subheader + a 35 bytes RLC AMD PDU(5 bytes RLC AMP PDU header + 3*10 bytes UL RLC SDU) + 1 byte Short BSR + 1 byte MAC Padding). NOTE 6: UL grant of 208 bits (ITBS=4, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Padding subheader + 1 byte Short BSR subheader + 2 bytes MAC PDU subheader + 1 byte MAC PDU subheader, 1 byte Short BSR + 12 bytes MAC SDU(2 bytes RLC AMD PDU header + 10 bytes bit UL RLC SDU) + 8 bytes MAC SDU (8 bytes of RLC STATUS PDU with NACK_SN and SOstart+SOend fields) NOTE 7: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2 |  |  |  |  |  |  |  |

22.3.2.3.3.3	Specific message contents

None.

#### 22.3.2.4	NB-IoT / AM RLC / Re-segmentation RLC PDU / SO, FI, LSF / Re-transmission of RLC PDU

##### 22.3.2.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {
  when { AMD PDU to be retransmitted does not fit in new allocated TBS }

then { UE segments AMD PDU into AMD PDU segments }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {
  when { AMD PDU segment to be retransmitted does not fit in new allocated TBS }

then { UE resegments AMD PDU segment to fit TBS }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives a STATUS PDU including a NACK_SN for missing AMD PDUs, RETX_COUNT < maxRetxThreshold }

then { UE successfully retransmits missing AMD PDUs }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {
  when { an AMD PDU or a portion of an AMD PDU is considered for retransmission and if RETX_COUNT = maxRetxThreshold }

then { UE indicates to upper layers that max retransmission has been reached }

}

##### 22.3.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.322 clause 4.2.1.3.2, 5.2.1, 6.2.1.4 and 6.2.1.5. Unless otherwise stated these are Rel-13 requirements.

[TS 36.322, clause 4.2.1.3.2]

When the transmitting side of an AM RLC entity forms AMD PDUs from RLC SDUs, it shall:

-	segment and/or concatenate the RLC SDUs so that the AMD PDUs fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity notified by lower layer.

The transmitting side of an AM RLC entity supports retransmission of RLC data PDUs (ARQ):

-	if the RLC data PDU to be retransmitted does not fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity notified by lower layer, the AM RLC entity can re-segment the RLC data PDU into AMD PDU segments;

-	the number of re-segmentation is not limited.

When the transmitting side of an AM RLC entity forms AMD PDUs from RLC SDUs received from upper layer or AMD PDU segments from RLC data PDUs to be retransmitted, it shall:

-	include relevant RLC headers in the RLC data PDU.

[TS 36.322, clause 5.2.1]

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an AMD PDU or a portion of an AMD PDU by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an AMD PDU or a portion of an AMD PDU by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding AMD PDU falls within the range VT(A) <= SN < VT(S):

-	consider the AMD PDU or the portion of the AMD PDU for which a negative acknowledgement was received for retransmission.

When an AMD PDU or a portion of an AMD PDU is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the AMD PDU is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the AMD PDU to zero;

-	else, if it (the AMD PDU or the portion of the AMD PDU that is considered for retransmission) is not pending for retransmission already, or a portion of it is not pending for retransmission already:

-	increment the RETX_COUNT;

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

When retransmitting an AMD PDU, the transmitting side of an AM RLC entity shall:

-	if the AMD PDU can entirely fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity:

-	deliver the AMD PDU as it is except for the P field (the P field should be set according to sub clause 5.2.2) to lower layer;

-	otherwise:

-	segment the AMD PDU, form a new AMD PDU segment which will fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity and deliver the new AMD PDU segment to lower layer.

When retransmitting a portion of an AMD PDU, the transmitting side of an AM RLC entity shall:

-	segment the portion of the AMD PDU as necessary, form a new AMD PDU segment which will fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity and deliver the new AMD PDU segment to lower layer.

When forming a new AMD PDU segment, the transmitting side of an AM RLC entity shall:

-	only map the Data field of the original AMD PDU to the Data field of the new AMD PDU segment;

-	set the header of the new AMD PDU segment in accordance with the description in sub clause 6.;

-	set the P field according to sub clause 5.2.2.

[TS 36.322, clause 6.2.1.4]

AMD PDU consists of a Data field and an AMD PDU header.

AMD PDU header consists of a fixed part (fields that are present for every AMD PDU) and an extension part (fields that are present for an AMD PDU when necessary). The fixed part of the AMD PDU header itself is byte aligned and consists of a D/C, a RF, a P, a FI, an E and a SN. The extension part of the AMD PDU header itself is byte aligned and consists of E(s) and LI(s).

An AM RLC entity is configured by RRC to use either a 10 bit SN or a 16 bit SN. The length of the fixed part of the AMD PDU header is two and three bytes respectively. The default values for SN field length used by an AM RLC entity is 10 bits.

An AMD PDU header consists of an extension part only when more than one Data field elements are present in the AMD PDU, in which case an E and a LI are present for every Data field element except the last. Furthermore, when an AMD PDU header consists of an odd number of LI(s) and the length of the LI field is 11 bits, four padding bits follow after the last LI. The default value for LI field length used by an AM RLC entity is 11 bits.

Figure 6.2.1.4-1: AMD PDU with 10 bit SN (length of LI field is 11 bits) (No LI)

Figure 6.2.1.4-2: AMD PDU with 10 bit SN (length of LI field is 11 bits) (Odd number of LIs, i.e. K = 1, 3, 5, …)

Figure 6.2.1.4-3: AMD PDU with 10 bit SN (length of LI field is 11 bits) (Even number of LIs, i.e. K = 2, 4, 6, …)

...

Figure 6.2.1.4-4: AMD PDU with 10 bit SN (length of LI field is 15 bits)

[TS 36.322, clause 6.2.1.5]

AMD PDU segment consists of a Data field and an AMD PDU segment header.

AMD PDU segment header consists of a fixed part (fields that are present for every AMD PDU segment) and an extension part (fields that are present for an AMD PDU segment when necessary). The fixed part of the AMD PDU segment header itself is byte aligned and consists of a D/C, a RF, a P, a FI, an E, a SN, a LSF and a SO. The extension part of the AMD PDU segment header itself is byte aligned and consists of E(s) and LI(s).

AM RLC entity is configured by RRC to use either a 10 bit SN or a 16 bit SN. When a 10 bit SN is used, the SO field is 15 bits, and when a 16 bit SN is used, the SO field is 16 bits. The length of the fixed part of the AMD PDU segment header is four and five bytes respectively. The default values for SN field length and SO field length used by an AM RLC entity are 10 bits and 15 bits, respectively.

An AMD PDU segment header consists of an extension part only when more than one Data field elements are present in the AMD PDU segment, in which case an E and a LI are present for every Data field element except the last. Furthermore, when an AMD PDU segment header consists of an odd number of LI(s) and the length of the LI field is 11 bits, four padding bits follow after the last LI. The default value for LI field length used by an AM RLC entity is 11 bits.

Figure 6.2.1.5-1: AMD PDU segment with 10 bit SN (No LI)

Figure 6.2.1.5-2: AMD PDU segment with 10 bit SN (length of LI field is 11 bits) (Odd number of LIs, i.e. K = 1, 3, 5, …)

Figure 6.2.1.5-3: AMD PDU segment with 10 bit SN (length of LI field is 11 bits) (Even number of LIs, i.e. K = 2, 4, 6, …)

Figure 6.2.1.5-4: AMD PDU segment with 10 bit SN (length of LI field is 15 bits)

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached for an SRB or for an MCG or split DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

2>	if AS security has not been activated:

3>	if the UE is a NB-IoT UE:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.331, clause 5.3.12]

Upon leaving RRC_CONNECTED, the UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320, T322, T325, T330;

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

…

1>	else:

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was triggered neither by reception of the MobilityFromEUTRACommand message nor by selecting an inter-RAT cell while T311 was running:

…

2>	enter RRC_IDLE and perform procedures as specified in TS 36.304 [4, 5.2.7];

##### 22.3.2.4.3	Test description

22.3.2.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 11.

-	System information combination 2 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

UE:

None.

Preamble

-	The UE is in state 2B-NB on Ncell 1 according to [18] the exceptions listed in table 22.3.2.4.3.1-1 with test loop mode G closed.

Table 22.3.2.4.3.1-1: RLC Settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit-r13 | ms4000 |

22.3.2.4.3.2	Test procedure sequence

Table 22.3.2.4.3.2-0 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. The configuration marked “T1” is applied at the points indicated in the Main behaviour description in Table 22.3.2.4.3.2-1.

Table 22.3.2.4.3.2-0: Cell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -85 | -79 |  |

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number ‘n’  in test procedure maps as:

UL SQN n maps to SQN X+n MOD 1024 &

DL SQN n maps to SQN Y+n MOD 1024

Table 22.3.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 0 | The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. | - | - | - | - |
| 1 | The SS transmits one AMD PDU containing RLC DL SDU#1 in its data field. The SDU includes a PRBS of 400 bits. | <-- | AMD PDU#1 | - | - |
| 2 | In the search space of the 3rd NPDCCH period bundle after the first transmission at step 1 the SS schedules one UL grant (Note 6, Note 8) | <-- | (UL grant, 440 bits) | - | - |
| 3 | The UE transmits an AMD PDU with the same data contents as received in the corresponding PRBS part of DL PDU#1. | --> | AMD PDU#1 (SN=0) | - | - |
| 4 | In the 5th NPDCCH period bundle after the transmission at step 1the SS transmits one AMD PDU containing RLC DL SDU#2 in its data field. The SDU includes a PRBS of 400 bits. (Note 8) | <-- | AMD PDU#2 | - | - |
| 5 | In the search space of the 8th NPDCCH period bundle after the first transmission at step 1 the SS schedules one UL grant (Note 6, Note 8) | <-- | (UL grant, 440 bits) | - | - |
| 6 | The UE transmits an AMD PDU with the same data contents as received in the corresponding PRBS part of DL PDU#2? | --> | AMD PDU#2 (SN=1) | - | - |
| 7 | The SS transmits a STATUS PDU. This PDU nacks the AMD PDU with SN=0. NACK_SN=0 and ACK_SN=2. | <-- | STATUS PDU | - | - |
| 8 | In the 3rd and 4th NPDCCH period bundle after the transmission at step 7 the SS schedules 1 UL grant of size 296 bits (Note 1, Note 4, Note 8) | <-- | (UL grants, 296 bits) | - | - |
| 9 | Check: Does the UE transmit an AMD PDU segment with SO=0, LSF=0 and the same data contents at the corresponding received positions as in the original AMD PDU? | --> | AMD PDU#1 segment 1 (SN=0) | 1,3 | P |
| 10 | Check: Does the UE transmit an AMD PDU segment with SO=<x>, LSF=1 and the same data contents at the corresponding received positions as in the original AMD PDU? (Note 3, Note 8) | --> | AMD PDU#1 segment 2 (SN=0) | 1,3 | P |
| 11 | The SS transmits a STATUS PDU. This PDU nacks the AMD PDU with SN=0. NACK_SN=0, SOStart=0, SOEnd=<x-1> and ACK_SN=2. (Note 3, Note 5) | <-- | STATUS PDU | - | - |
| 12 | In the 3rd and 4th NPDCCH period bundle after the transmission at step 11 the SS schedules 1 UL grant of size 208 bits (Note 2) (Note 4, Note 8) | <-- | (UL grants, 208 bits) | - | - |
| 13 | Check: Does the UE transmit an AMD PDU segment with SO=0, LSF=0 and the same data contents at the corresponding received positions as in the original AMD PDU? | --> | AMD PDU#1 segment 1, 1st part(SN=0) | 2,3 | P |
| 14 | Check: Does the UE transmit an AMD PDU segment with SO=<y>, LSF=0 and the same data contents at the corresponding received positions as in the original AMD PDU? (Note 3) | --> | AMD PDU#1 segment 1, 2nd part (SN=0) | 2,3 | P |
| 15 | The SS transmits a STATUS PDU. This PDU acks the AMD PDUs with SN=0 and SN=1. ACK_SN=2. | <-- | STATUS PDU | - | - |
| 16 | The SS transmits one AMD PDU containing RLC DL SDU#3 in its data field. The SDU includes a PRBS of 400 bits. | <-- | AMD PDU#3 | - | - |
| 17 | In the search space of the 5th NPDCCH period bundle after the transmission at step 16 the SS schedules one UL grant (Note 6, Note 8) | <-- | (UL grant, 440 bits) | - | - |
| 18 | The UE transmits an AMD PDU containing the corresponding PRBS part of DL PDU#3 in its data field. | --> | AMD PDU#3 (SN=2) | - | - |
| - | EXCEPTION: Step 19 to 20 shall be repeated maxRetxThreshold times (let i be the loop counter, i=1,…, maxRetxThreshold) | - | - | - | - |
| 19 | The SS transmits an RLC STATUS PDU. ACK_SN=3 and NACK_SN=2. | <-- | STATUS PDU | - | - |
| 19A | In the search space of the ((i +1) * 5)th NPDCCH period bundle after the transmission at step 16 the SS schedules one UL grant (Note 6, Note 8). | <-- | (UL grant, 440 bits) | - | - |
| 20 | Check: Does the UE retransmit the AMD PDU not yet acknowledged? | --> | AMD PDU#3 (SN=2) | 3 | P |
| 21 | The SS transmits an RLC STATUS PDU. ACK_SN=3 and NACK_SN=2. | <-- | STATUS PDU | - | - |
| 21A | The SS changes Ncell 11 levels according to row "T1" in Table 22.3.2.4.3.2-0 |  |  |  |  |
| 22 | Check: Does UE transmit an RRCConnectionRequest-NB message ? | --> | RRC: RRCConnectionRequest-NB | 4- | P - |
| 23 - 26 | UE performs TAU procedure based on steps 2 to 5 of Generic test procedure in TS 36.508 subclause 8.1.5A.5 to takes place on Ncell 11. |  |  | - | - |
| 27 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| Note 1: UL grant of 296 bits (ITBS=9, =1, see TS 36.213 Table 16.5.1.2-2) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 296 bits=39 bytes fits an AMD PDU payload of >= 25 bytes + 2 bytes AMD PDU header + 2 bytes of segment header + X bytes spare for MAC header and possible RLC STATUS PDU and BSR report. Note 2: UL grant of 208 bits (ITBS=4, =2, see TS 36.213 Table 16.5.1.2-2) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 208 bits=26 bytes fits an AMD PDU payload of >= 13 bytes + 2 bytes AMD PDU header + 2 bytes of segment header + x bytes spare for MAC header and possible RLC STATUS PDU and BSR report. Note 3: The values x and y depend upon the need of the UE to add RLC STATUS PDU and BSR report. The TBS has been chosen to ensure that the PDUs to be resegmented can be carried in 2 segments. Note 4: 40 ms gap between transmissions both in DL and UL respectively allows for possible repetitions. Note 5: As <x> becomes available in step 8 only the transmission in step 10 can only be scheduled afterwards. This requires a 100 ms activation time. Note 6: UL grant of 440 bits (ITBS=3, =6, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. Note 7: Void. Note 8: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2 |  |  |  |  |  |

22.3.2.4.3.3	Specific message contents

None.

#### 22.3.2.5	NB-IoT / AM RLC / Segmentation and Reassembly / AMD PDU reassembly from AMD PDU segments / Re-ordering of RLC PDU segments

##### 22.3.2.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives an AMD PDU or an AMD PDU segment containing a FI field set to 00 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives an AMD PDU or an AMD PDU segment containing a FI field set to 01 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives an AMD PDU or an AMD PDU segment containing a FI field set to 11 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment}

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives an AMD PDU or an AMD PDU segment containing a FI field set to 10 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives AM PDU segments }

then { UE delivers reassembled RLC SDU to upper layer}

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives RLC AM PDU segments without segment header extension part }

then { UE correctly reassembles RLC AMD PDU segments into RLC AMD PDUs }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives RLC AM PDU segments with segment header extension part }

then { UE correctly reassembles RLC AMD PDU segments into RLC AMD PDUs }

}

(8)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives duplicate RLC AM PDU segments }

then { UE discards duplicate RLC AMD PDU segments }

}

(9)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives RLC AM PDU segments out of sequence }

then { UE delivers reassembled RLC SDU to upper layer }

}

(10)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives RLC AMD PDU segments with segments lost }

then { UE transmits STATUS PDU to request retransmission of missing segments }

}

(11)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives overlapping RLC AMD PDU segments }

then { UE discards duplicate RLC AMD PDU byte segments }

}

(12)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives RLC AM PDU segments }

then { UE reorders RLC AMD PDU segments received out of sequence }

}

(13)

with { UE in RRC_CONNECTED state }

ensure that {
  when { enableStatusReportSN-Gap-r13 }

then { Set VR(MS) to SN of the first AMD PDU with SN >= VR(X) for which not all byte segments have been received }

}

##### 22.3.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.322 clauses 4.2.1.3.3, 5.1.3.2.1, 5.1.3.2.2, 5.1.3.2.3, 6.2.1.4, 6.2.1.5 and 6.2.2.6.

[TS 36.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives RLC data PDUs, it shall:

-	reorder the RLC data PDUs if they are received out of sequence;

-	detect the loss of RLC data PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the reordered RLC data PDUs and deliver the RLC SDUs to upper layer in sequence.

...

[TS 36.322, clause 5.1.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to state variables VR(R) and VR(MR) as follows:

-	a SN falls within the receiving window if VR(R) <= SN < VR(MR);

-	a SN falls outside of the receiving window otherwise.

When receiving a RLC data PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received RLC data PDU or place it in the reception buffer (see sub clause 5.1.3.2.2);

-	if the received RLC data PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reordering as needed (see sub clause 5.1.3.2.3).

When t-Reordering expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reordering as needed (see sub clause 5.1.3.2.4).

For NB-IoT;

-	The receiving side of an RLC entity shall behave such that the timer values of t-Reordering and t-StatusProhibit are 0.

[TS 36.322, clause 5.1.3.2.2]

When a RLC data PDU is received from lower layer, where the RLC data PDU contains byte segment numbers y to z of an AMD PDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the AMD PDU with SN = x have been received before:

-	discard the received RLC data PDU;

-	else:

-	place the received RLC data PDU in the reception buffer;

-	if some byte segments of the AMD PDU contained in the RLC data PDU have been received before:

-	discard the duplicate byte segments.

[TS 36.322, clause 5.1.3.2.3]

When a RLC data PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= VR(H)

-	update VR(H) to x+ 1;

-	if all byte segments of the AMD PDU with SN = VR(MS) are received:

-	update VR(MS) to the SN of the first AMD PDU with SN > current VR(MS) for which not all byte segments have been received;

-	if x = VR(R):

-	if all byte segments of the AMD PDU with SN = VR(R) are received:

-	update VR(R) to the SN of the first AMD PDU with SN > current VR(R) for which not all byte segments have been received;

-	update VR(MR) to the updated VR(R) + AM_Window_Size;

-	reassemble RLC SDUs from any byte segments of AMD PDUs with SN that falls outside of the receiving window and in-sequence byte segments of the AMD PDU with SN = VR(R), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in sequence if not delivered before;

-	if t-Reordering is running:

-	if VR(X) = VR(R); or

-	if VR(X) falls outside of the receiving window and VR(X) is not equal to VR(MR):

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case t-Reordering is stopped due to actions above):

-	if VR (H) > VR(R):

-	start t-Reordering;

-	set VR(X) to VR(H).

[TS 36.322, clause 6.2.1.4]

AMD PDU consists of a Data field and an AMD PDU header.

AMD PDU header consists of a fixed part (fields that are present for every AMD PDU) and an extension part (fields that are present for an AMD PDU when necessary). The fixed part of the AMD PDU header itself is byte aligned and consists of a D/C, a RF, a P, a FI, an E and a SN. The extension part of the AMD PDU header itself is byte aligned and consists of E(s) and LI(s).

…

An AMD PDU header consists of an extension part only when more than one Data field elements are present in the AMD PDU, in which case an E and a LI are present for every Data field element except the last. Furthermore, when an AMD PDU header consists of an odd number of LI(s) and the length of the LI field is 11 bits, four padding bits follow after the last LI. The default value for LI field length used by an AM RLC entity is 11 bits.

…

[TS 36.322, clause 6.2.1.5]

AMD PDU segment consists of a Data field and an AMD PDU segment header.

AMD PDU segment header consists of a fixed part (fields that are present for every AMD PDU segment) and an extension part (fields that are present for an AMD PDU segment when necessary). The fixed part of the AMD PDU segment header itself is byte aligned and consists of a D/C, a RF, a P, a FI, an E, a SN, a LSF and a SO. The extension part of the AMD PDU segment header itself is byte aligned and consists of E(s) and LI(s).

...

An AMD PDU segment header consists of an extension part only when more than one Data field elements are present in the AMD PDU segment, in which case an E and a LI are present for every Data field element except the last. Furthermore, when an AMD PDU segment header consists of an odd number of LI(s) and the length of the LI field is 11 bits, four padding bits follow after the last LI. The default value for LI field length used by an AM RLC entity is 11 bits.

…

[TS 36.322, clause 6.2.2.6]

Length: 2 bits.

The FI field indicates whether a RLC SDU is segmented at the beginning and/or at the end of the Data field. Specifically, the FI field indicates whether the first byte of the Data field corresponds to the first byte of a RLC SDU, and whether the last byte of the Data field corresponds to the last byte of a RLC SDU. The interpretation of the FI field is provided in Table 6.2.2.6-1.

Table 6.2.2.6-1: FI field interpretation

| Value | Description |
| --- | --- |
| 00 | First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. |
| 01 | First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. |
| 10 | First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. |
| 11 | First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. |

##### 22.3.2.5.3	Test description

22.3.2.5.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 2B-NB) according to [18] and the exceptions listed in table 22.3.2.5.3.1-1 with test loop mode G closed.

Table 22.3.2.5.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit-r13 | ms4000 |
| enableStatusReportSN-Gap-r13 | true |

22.3.2.5.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number ‘n’ in test procedure maps as:

- UL SQN n maps to SQN X+n MOD 1024 &

- DL SQN n maps to SQN Y+n MOD 1024

Table 22.3.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |  |
| - | Note: In steps 0 to 17 the size of the RLC SDUs used in uplink will be 33 octets. The size of the RLC SDUs used in downlink is L. | - | - |  | - | - |  |
| 0 | The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence | - |  | - | - |  | - |
| 1 | The SS transmits AMD PDU#1 containing a complete RLC DL SDU#1 (FI field = 00). | <-- | AMD PDU#1(SN=0) |  | - | - |  |
| 1A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 1 the SS schedules one UL Grant of size 328 bits, sufficient for one RLC UL SDU to be looped back (Note 1, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 2 | Check: Does the UE transmit RLC UL SDU#1? | --> | (RLC UL SDU#1) (SN=0) |  | 1 | P |  |
| 3 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=1) |  | - | - |  |
| 4 | The SS transmits AMD PDU#2 containing the first segment of RLC DL SDU#2 (FI field = 01). | <-- | AMD PDU#2(SN=1) |  | - | - |  |
| 5 | The SS transmits AMD PDU#3 containing the second segment of RLC DL SDU#2 (FI field = 11). | <-- | AMD PDU#3(SN=2) |  | - | - |  |
| 6 | The SS transmits AMD PDU#4 containing the last segment of RLC DL SDU#2 (FI field = 10). | <-- | AMD PDU#4(SN=3) |  | - | - |  |
| 6A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 6 the SS schedules one UL Grant of size 328 bits, sufficient for one RLC UL SDU to be looped back (Note 1, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 7 | Check: Does the UE transmit RLC UL SDU#2? | --> | (RLC UL SDU#2) (SN=1) |  | 2,3,4 | P |  |
| 8 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=2) |  | - | - |  |
| 9 | The t-PollRetransmit-r13 timer for RLC DL PDU#5 expires and SS assumes that the transmission of AMD PDU#5 containing a complete RLC DL SDU#3 and a complete RLC DL SDU#4 is failed and consider RLC DL PDU#5 for re-transmission | - | - |  | - | - |  |
| 10 | The SS transmits AMD PDU segment containing a complete RLC DL SDU#3 (FI field = 00). | <-- | AMD PDU#5 segment(SN=4) |  | - | - |  |
| 10A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 10 the SS schedules one UL Grant of size 392 bits, sufficient for one STATUS PDU and one RLC UL SDU to be looped back (Note 11, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 11 | Check: Does the UE transmit a STATUS PDU with NACK_SN=4 with SOStart=L and SOEnd=32767 (special SOEnd value) and ACK_SN=5 and an AMD PDU containing RLC SDU#3? | --> | (RLC UL SDU#3) (SN=2) STATUS PDU |  | 1 | P |  |
| 12 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=3) |  | - | - |  |
| 13 | The SS transmits AMD PDU segment containing the first segment of RLC DL SDU#4 (FI field = 01). | <-- | AMD PDU#5 segment(SN=4) |  | - | - |  |
| 14 | The SS transmits AMD PDU segment containing the second segment of RLC DL SDU#4 (FI field = 11). | <-- | AMD PDU#5 segment(SN=4) |  | - | - |  |
| 15 | The SS transmits AMD PDU segment containing the last segment of RLC DL SDU#4 (FI field = 10). | <-- | AMD PDU#5 segment(SN=4) |  | - | - |  |
| 15A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 15 the SS schedules one UL Grant of size 392 bits, sufficient for one STATUS PDU and one RLC UL SDU to be looped back (Note 12, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 16 | Check: Does the UE transmit a STATUS PDU with ACK_SN=5 and an AMD PDU containing RLC UL SDU#4? | --> | (RLC UL SDU#4) (SN=3) STATUS PDU |  | 2,3,4 | P |  |
| 17 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=4) |  | - | - |  |
| - | Note: In steps 18 to 51 the size of the RLC SDUs used in uplink will be 16 octets. The size of the RLC SDUs used in downlink is L. | - | - |  | - | - |  |
| 18 | The SS transmits an AMD PDU containing the first part (8 bytes) of RLC DL SDU#5 in its data field. This PDU is in error (SN falls outside of the receiving window) and is to be discarded by the UE. | <-- | AMD PDU#6 (SN=WindowSize+3) |  | - | - |  |
| 19 | The SS transmits an AMD PDU containing RLC DL SDU#6 (L bytes) in its data field with the P-bit set. | <-- | AMD PDU#7 (SN=6, P=1) |  | - | - |  |
| 20 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 19 the SS schedules one UL Grant of size 72 bits, sufficient for one RLC STATUS PDU (Note 2, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 21 | The UE transmits a STATUS PDU with NACK_SN field indicating missing PDU#5. ACK_SN=7, NACK_SN=5. | --> | STATUS PDU |  | - | - |  |
| 22 | The SS transmits an AMD PDU segment of AMD PDU#6 (AMD PDU#6 carries RLC DL SDU#5) containing the first 8 bytes of RLC DL SDU#5 in its data field. SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#6 (SN=5) segment 1 |  | - | - |  |
| 23 | The SS transmits an AMD PDU segment of AMD PDU#6 (AMD PDU#6 carries RLC DL SDU#5) containing the last L-8 bytes of RLC DL SDU#5 in its data field with the P-bit set. SO=8 and LSF=1. No header extension part is provided. | <-- | AMD PDU #6 (SN=5, P=1) segment 2 |  | - | - |  |
| 24 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 23 the SS schedules one UL Grant of size 56 bits, sufficient for one RLC STATUS PDU (Note 3, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 25 | Check: Does the UE transmit a STATUS PDU with ACK_SN=7, thus acknowledging the reception of PDUs with SN=5 and SN=6, and no NACK_SN provided? | --> | STATUS PDU |  | 6 | P |  |
| 25A | In the search space of the next NPDCCH period bundle after the transmission at step 24 the SS schedules one UL Grant of size 328 bits, sufficient for two RLC UL SDUs to be looped back (Note 4, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 26 | Check: Does the UE transmit RLC UL SDU#5 and RLC UL SDU#6? | --> | AMD PDU (RLC UL SDU#5, RLC UL SDU#6) (SN=4) |  | 5 | P |  |
| 27 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=5) |  | - | - |  |
| 28 | The SS transmits an AMD PDU segment of AMD PDU#8 (AMD PDU#8 carries RLC DL SDU#7 and RLC DL SDU#8) containing the last L-8 bytes of RLC DL SDU#8 in its data field, with the P-bit set. FI=10, SO=(L+8) and LSF=1. No header extension part is provided. | <-- | AMD PDU#8 (SN=7, P=1) segment 2 |  | - | - |  |
| 29 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 28 the SS schedules one UL Grant of size 104 bits, sufficient for one RLC STATUS PDU (Note 5, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 30 | The UE transmits a STATUS PDU NACK_SN field for receipt of PDU#8. ACK_SN=8, NACK_SN=7, SOStart=0/SOEnd=L+7. | --> | STATUS PDU |  | - | - |  |
| 31 | The SS transmits an AMD PDU segment of AMD PDU#8 (AMD PDU#8 carries RLC DL SDU#7 and RLC DL SDU#8) containing RLC DL SDU#7 (L bytes) and the first 8 bytes of SDU#8 in its data field, with the P-bit set. FI=01, SO=0 and LSF=0. Header extension part present: E in fixed part header=1, E in extension part header=0, LI=L. | <-- | AMD PDU#8 (SN=7, P=1) segment 1 |  | - | - |  |
| 32 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 31 the SS allocates one UL Grant of size 56 bits, sufficient for one RLC STATUS PDU (Note 3, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 33 | Check: Does the UE transmit a STATUS PDU with ACK_SN=8? | --> | STATUS PDU |  | 7 | P |  |
| 33A | In the search space of the next NPDCCH period bundle after the transmission at step 32 the SS schedules one UL Grant of size 328 bits, sufficient for one RLC UL SDU to be looped back (Note 4, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 34 | Check: Does the UE transmit RLC UL SDU#7 and RLC UL SDU#8? | --> | AMD PDU (RLC UL SDU#7, RLC UL SDU#8) (SN=5) |  | 5,9 | P |  |
| 35 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=6) |  | - | - |  |
| 36 | The SS transmits an AMD PDU segment of AMD PDU#9 (AMD PDU#9 carries RLC DL SDU#9) containing the first 8 bytes of RLC DL SDU#9 in its data field. SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#9 (SN=8) segment 1 |  | - | - |  |
| 37 | The SS transmits an AMD PDU segment of AMD PDU#9 (AMD PDU#9 carries RLC DL SDU#9) containing the first 8 bytes of RLC DL SDU#9 in its data field. SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#9 (SN=8) segment 1 |  | - | - |  |
| 38 | The SS transmits an AMD PDU segment of AMD PDU#9 (AMD PDU#9 carries RLC DL SDU#9) containing the last L-8 bytes of RLC DL SDU#9 in its data field, with the P-bit set. SO=8 and LSF=1. No header extension part is provided. | <-- | AMD PDU#9 (SN=8, P=1) segment 2 |  | - | - |  |
| 39 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 38 the SS schedules one UL Grant of size 56 bits, sufficient for one RLC STATUS PDU (Note 3, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 40 | Check: Does the UE transmit a STATUS PDU with ACK_SN=9, thus acknowledging the reception of PDUs with SN=4 to SN=8, and no NACK_SN provided? | --> | STATUS PDU |  | 8 | P |  |
| 40A | In the search space of the next NPDCCH period bundle after the transmission at step 39 the SS schedules one UL Grant of size 176 bits, sufficient for one RLC UL SDU to be looped back (Note 6, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 41 | Check: Does the UE transmit RLC UL SDU#9? | --> | (RLC UL SDU#9) (SN=6) |  | 5 | P |  |
| 42 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=7) |  | - | - |  |
| 43 | The SS transmits an AMD PDU segment of AMD PDU#11 (AMD PDU#11 carries RLC DL SDU#11) containing the last L-8 bytes of RLC DL SDU#11 in its data field, with the P-bit set. This AMD PDU segment is sent with SN=10. SO=8 and LSF=1. No header extension part is provided. | <-- | AMD PDU#11 (SN=10, P=1) segment 2 |  | - | - |  |
| 44 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 43 the SS schedules one UL Grant of size 144 bits, sufficient for one RLC STATUS PDU (Note 7, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 45 | Check: Does the UE transmit a STATUS PDU with ACK_SN=11, thus acknowledging the reception of PDUs with SN=4 to SN=10, and NACK_SN=9, E1/E2 field for receipt of PDU#9 and NACK_SN=10, SOStart=0/SOEnd=7 for segment 1 of PDU#11? | --> | STATUS PDU |  | 10 | P |  |
| 46 | The SS transmits an AMD PDU segment of AMD PDU#11 (AMD PDU#11 carries RLC DL SDU#11) containing the first 8 bytes of RLC DL SDU#11 in its data field. SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#11 (SN=10) segment 1 |  | - | - |  |
| 47 | The SS transmits one AMD PDU containing RLC DL SDU#10 (L bytes) in its data field, with the P-bit set. | <-- | AMD PDU#10 (SN=9, P=1) |  | - | - |  |
| 48 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 47 the SS schedules one UL Grant of size 56 bits, sufficient for one RLC STATUS PDU (Note 3, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 49 | The UE transmits a STATUS PDU with ACK_SN=11, thus acknowledging the reception of PDUs with SN=4 to SN=10, and no NACK_SN provided. | --> | STATUS PDU |  | - | - |  |
| 49A | In the search space of the next NPDCCH period bundle after the transmission at step 48 the SS schedules one UL Grant of size 328 bits, sufficient for two RLC UL SDUs to be looped back (Note 4, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 50 | Check: Does the UE transmit RLC UL SDU#10 and RLC UL SDU#11? | --> | AMD PDU (RLC UL SDU#10, RLC UL SDU#11) (SN=7) |  | 6,9 | P |  |
| 51 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=8) |  | - | - |  |
| - | Note: In steps 52 to 62 the size of the RLC SDUs used in uplink will be 10 octets. The size of the RLC SDUs used in downlink is L. | - | - |  | - | - |  |
| 52 | The SS transmits an AMD PDU segment of AMD PDU#12 (AMD PDU#12 carries RLC DL SDU#12, RLC DL SDU#13 and RLC DL SDU#14) containing the last L-6 bytes of RLC DL SDU#13 and the complete RLC DL SDU#14 (L bytes) in its data field, with the P-bit set. FI=10, SO= L+6 and LSF=1. Header extension part present: E in fixed part header=1, E in extension part header=0, LI=L-6. | <-- | AMD PDU#12 (SN=11, P=1) segment 3 |  | - | - |  |
| 53 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 52 the SS schedules one UL Grant of size 104 bits, sufficient for one RLC STATUS PDU (Note 5, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 54 | The UE transmits a STATUS PDU NACK_SN field for receipt of PDU#12. ACK_SN=12, NACK_SN=11, SOStart=0/SOEnd= L+5. | --> | STATUS PDU |  | - | - |  |
| 55 | The SS transmits an AMD PDU segment of AMD PDU#12 (AMD PDU#12 carries RLC DL SDU#12, RLC DL SDU#13 and RLC DL SDU#14) containing the last L-6 bytes of RLC DL SDU#12 and the first 6 bytes of RLC DL SDU#13 in its data field, with the P-bit set. FI=11, SO=6 and LSF=0. Header extension part present: E in fixed part header=1, E in extension part header=0, LI=L-6. | <-- | AMD PDU#12 (SN=11, P=1) segment 2 |  | - | - |  |
| 56 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 55 the SS schedules one UL Grant of size 104 bits, sufficient for one RLC STATUS PDU (Note 5, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 57 | The UE transmits a STATUS PDU NACK_SN field for receipt of PDU#12. ACK_SN=12, NACK_SN=11, SOStart=0/SOEnd=5. | --> | STATUS PDU |  | 11 | P |  |
| 58 | The SS transmits an AMD PDU segment of AMD PDU#12 (AMD PDU#12 carries RLC DL SDU#12, RLC DL SDU#13 and SDU#14) containing the first 6 bytes of SDU#12 in its data field, with the P-bit set. FI=01, SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#12 (SN=11, P=1) segment 1 |  | - | - |  |
| 59 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 58 the SS schedules one UL Grant of size 56 bits, sufficient for one RLC STATUS PDU (Note 3, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 60 | Check: Does the UE transmit a STATUS PDU with ACK_SN=12, thus acknowledging the reception of PDUs with SN=4 to SN=11, and no NACK_SN provided? | --> | STATUS PDU |  | 11 | P |  |
| 60A | In the search space of the next NPDCCH period bundle after the transmission at step 59 the SS schedules one UL Grant of size 328 bits, sufficient for three RLC UL SDUs to be looped back (Note 8, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 61 | Check: Does the UE transmit RLC UL SDU#12, RLC UL SDU#13 and RLC UL SDU#14? | --> | AMD PDU (RLC UL SDU#12, RLC UL SDU#13, RLC UL SDU#14) (SN=8) |  | 11 | P |  |
| 62 | The SS transmits a STATUS PDU. | <-- |  | STATUS PDU (ACK SN=9) | - |  | - |
| - | Note: In steps 63 to 93 the size of the RLC SDUs used in uplink will be 32 octets. The size of the RLC SDUs used in downlink is L. | - | - |  | - | - |  |
| 63 | The SS transmits one AMD PDU containing RLC DL SDU#22 (L bytes) in its data field to the UE. SN=19 indicates the loss of 7 PDUs. | <-- | AMD PDU#20 (SN=19) |  | - | - |  |
| 64 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#15 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#13, which contained RLC DL SDU#15 (L bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#13 (SN=12) segment 1 |  | - | - |  |
| 65 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#16 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#14, which contained RLC DL SDU#16 (L bytes) in its data field. SO=16 and LSF=1. | <-- | AMD PDU#14 (SN=13) segment 2 |  | - | - |  |
| 66 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#17 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#15, which contained RLC DL SDU#17 (L bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#15 (SN=14) segment 1 |  | - | - |  |
| 67 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#18 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#16, which contained RLC DL SDU#18 (L bytes) in its data field. SO=16 and LSF=1. | <-- | AMD PDU#16 (SN=15) segment 2 |  | - | - |  |
| 68 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#18 in its data field to the UE. This AMD PDU segment carries part 1of AMD PDU#16, which contained RLC DL SDU#18 (L bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#16 (SN=15) segment 1 |  | - | - |  |
| 69 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#15 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#13, which contained RLC DL SDU#15 (L bytes) in its data field. SO=16 and LSF=1. | <-- | AMD PDU#13 (SN=12) segment 2 |  | - | - |  |
| 70 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#16 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#14, which contained RLC DL SDU#16 (L bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#14 (SN=13) segment 1 |  | - | - |  |
| 71 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#17 in its data field to the UE. This AMD PDU segment carries part 2 of PDU#15, which contained RLC DL SDU#17 (L bytes) in its data field. SO=16 and LSF=1. | <-- | AMD PDU#15 (SN=14) segment 2 |  | - | - |  |
| 72 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#21 in its data field to the UE. This AMD PDU segment carries part 1 of PDU #19, which contained RLC DL SDU#21 (L bytes) in its data field. SO=0 and LSF=0. | <-- | AMD PDU#19 (SN=18) segment 1 |  | - | - |  |
| 73 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#20 in its data field to the UE. This AMD PDU segment carries segment 2 of AMD PDU#18, which contained RLC DL SDU#20 (L bytes) in its data field. SO=16 and LSF=1. | <-- | AMD PDU#18 (SN=17) segment 2 |  | - | - |  |
| 73A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 73 the SS allocates one UL Grant of size 144 bits, sufficient for one RLC STATUS PDU (Note 9, Note 13). | <-- | Uplink Grant |  | - |  | - |
| 73B | Check: Does the UE transmit a Status Report with NACK_SN=16, NACK_SN=17 with SOStart=0 and SOEnd=15, and NACK_SN=18 with SOStart=16 and SOEnd=32767 (special SOEnd value), and ACK_SN=20? | --> | STATUS PDU |  | 13 |  | P |
| 74 | In the search space of the next NPDCCH period bundle after the transmission at step 73A the SS schedules four UL Grants of size 296 bits, each one sufficient for one RLC UL SDU to be looped back (Note 10, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 75 | Check: Does the UE transmit an RLC UL SDU containing SDU#15 in its data field? | --> | (RLC UL SDU#15) (SN=9) |  | 12 | P |  |
| 76 | Check: Does the UE transmit an RLC UL SDU containing SDU#16 in its data field? | --> | (RLC UL SDU#16) (SN=10) |  | 12 | P |  |
| 77 | Check: Does the UE transmit an RLC UL SDU containing SDU#17 in its data field? | --> | (RLC UL SDU#17) (SN=11) |  | 12 | P |  |
| 78 | Check: Does the UE transmit an RLC UL SDU containing SDU#18 in its data field? | --> | (RLC UL SDU#18) (SN=12) |  | 12 | P |  |
| 79 | The SS transmits an RLC STATUS PDU to the UE. This PDU acks PDUs up to those including SDU#18. ACK_SN=13. | <-- | STATUS PDU |  | - | - |  |
| 80 | Void | - |  |  | - | - |  |
| 81 | Void | - |  |  | - |  | - |
| 82 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#21 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#19, which contained RLC DL SDU#21 (L bytes) in its data field. SO=16 and LSF=1. | <-- |  | AMD PDU#19 (SN=18) segment 2 | - |  | - |
| 83 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#20 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#18, which contained RLC DL SDU#20 (L bytes) in its data field. SO=0 and LSF=0. | <-- |  | AMD PDU#18 (SN=17) segment 1 | - |  | - |
| 84 | The SS transmits one AMD PDU segment containing 16 bytes of RLC DL SDU#19 in its data field to the UE. This AMD PDU segment carries part 1 of AMD PDU#17, which contained RLC DL SDU#19 (L bytes) in its data field. SO=0 and LSF=0. | <-- |  | AMD PDU#17 (SN=16) segment 1 | - |  | - |
| 85 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 84 the SS schedules one UL Grant of size 104 bits, sufficient for one RLC STATUS PDU (Note 5, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 86 | Check: Does the UE transmit a Status Report with NACK_SN=16 with SOStart=16 and SOEnd=32767 (special SOEnd value), and ACK_SN=20? | --> |  | STATUS PDU | 13 |  | P |
| 87 | The SS transmits one AMD PDU segment containing L-16 bytes of RLC DL SDU#19 in its data field to the UE. This AMD PDU segment carries part 2 of AMD PDU#17, which contained RLC DL SDU#19 (L bytes)in its data field. SO=16 and LSF=1. | <-- |  | AMD PDU#17 (SN=16) segment 2 | - |  | - |
| 88 | In the search space of the 3rd NPDCCH period bundle after the transmission at step 87 the SS schedules four UL Grants of size 296 bits, each one sufficient for one RLC UL SDU to be looped back (Note 10, Note 13). | <-- | Uplink Grant |  | - | - |  |
| 89 | Check: Does the UE transmit an RLC UL SDU containing SDU#19 in its data field? | --> |  | (RLC UL SDU#19) (SN=13) | 12 |  | P |
| 90 | Check: Does the UE transmit an RLC UL SDU containing SDU#20 in its data field? | --> |  | (RLC UL SDU#20) (SN=14) | 12 |  | P |
| 91 | Check: Does the UE transmit an RLC UL SDU containing SDU#21 in its data field? | --> |  | (RLC UL SDU#21) (SN=15) | 12 |  | P |
| 92 | Check: Does the UE transmit an RLC UL SDU containing SDU#22 in its data field? | --> |  | (RLC UL SDU#22) (SN=16) | 12 |  | P |
| 93 | The SS transmits an RLC STATUS PDU to the UE. This PDU acks PDUs up to those including SDU#21. ACK_SN=17. | <-- |  | STATUS PDU | - |  | - |
| Note 1: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Short BSR subheader + 2 bytes MAC SDU subheader + 1 byte Padding subheader, 1 byte Short BSR + 35 bytes MAC SDU(2 bytes RLC AMD PDU header + 33 bytes UL RLC SDU) + 1 byte Padding). Note 2: UL grant of 72 bits (ITBS=2, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and NACK_SN (2 bytes Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader + 1 byte Short BSR + 4 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 12-bit NACK_SN/E1/E2 + 5-bit Padding)). Note 3: UL grant of 56 bits (ITBS=1, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN (2 bytes Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader + 1 byte Short BSR + 2 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 1bit padding)). Note 4: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 2 bytes Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader, 1 byte Short BSR + 36 bytes MAC SDU (4 bytes RLC AMD PDU header + 2*16 UL RLC SDU)). Note 5: UL grant of 104 bits (ITBS=3, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and NACK_SN (2 bytes Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader + 1 byte Short BSR + 8 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 12-bit NACK_SN/E1/E2 + 30-bit SOstart/SOend + 7-bit Padding)). Note 6: UL grant of 176 bits (ITBS=1, =4, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader, 1 byte Short BSR + 18 bytes MAC SDU(2 bytes RLC AMD PDU header + 16 UL RLC SDU)). Note 7: UL grant of 144 bits (ITBS=5, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and 2 NACK_SNs ( (1 byte Short BSR subheader + 2 byte MAC SDU subheader + 1 byte Padding subheader + 1 byte Short BSR + 9 bytes RLC Status PDU ( 4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 12-bit NACK_SN/E1/E2 + (12-bit NACK_SN/E1/E2 + 30-bit SOstart/SOend) + 3-bit Padding) + 4 byte Padding) or (2 bytes Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader + 1 byte Short BSR + 13 bytes RLC Status PDU ( 4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2 x (12-bit NACK_SN/E1/E2 + 30-bit SOstart/SOend) + 5-bit Padding)) ). Note 8: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Short BSR subheader + 2 bytes MAC SDU subheader + 1 byte Padding subheader, 1 byte Short BSR + 35 bytes MAC SDU(5 bytes RLC AMD PDU header + 3*10 UL RLC SDU) + 1 byte Padding). Note 9: UL grant of 144 bits (ITBS=5, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and 3 NACK_SNs (1 byte Padding subheader + 1 byte Short BSR subheader + 1 byte MAC SDU subheader + 1 byte Short BSR + 14 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 12-bit NACK_SN/E1/E2 +2*(12-bit NACK_SN/E1/E2 + 30-bit SOstart/SOend) + 1-bit Padding)). Note 10: UL grant of 296 bits (ITBS=9, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Short BSR subheader + 1 byte MAC SDU subheader, 1 byte Short BSR + 34 bytes MAC SDU(2 bytes RLC AMD PDU header + 32 bytes UL RLC SDU)). Note 11: UL grant of 392 bits (ITBS=8, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Padding subheader + 1 byte Short BSR subheader + 2 bytes MAC SDU subheader + 1 byte MAC SDU subheader, 1 byte Short BSR + 35 bytes MAC SDU(2 bytes RLC AMD PDU header + 33 bytes UL RLC SDU) + 8 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 12-bit NACK_SN/E1/E2 + 30-bit SOstart/SOend + 7-bit Padding)). Note 12: UL grant of 392 bits (ITBS=8, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time (MAC subheader: 1 byte Short BSR subheader + 2 bytes MAC SDU subheader + 2 byte MAC SDU subheader + 1 byte Padding subheader, 1 byte Short BSR + 35 bytes MAC SDU(2 bytes RLC AMD PDU header + 33 bytes UL RLC SDU) + 2 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 1bit padding) + 5 bytes Padding). Note 13: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2 |  |  |  |  |  |  |  |

22.3.2.5.3.3	Specific message contents

N