3GPP TSG-RAN5 Meeting #101	R5-236610r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0051 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.2.14 FD Using HTTP Active Functional Alias |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no test case for an FD Using HTTP call with an Active Functional Alias |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TC 6.2.14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT Clients could fail in the field without conformance verification of an FD Using HTTP call with an Active Functional Alias |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removed text before Table 6.2.14.3.3-1 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.2.14	On-network / File Distribution (FD) / FD Using HTTP / One-to-one Standalone FD / Non-Mandatory Download / Active functional alias / Client Originated (CO)

6.2.14.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { MCDATA User wants to send a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE", and, the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message using the active functional alias }

}

(2)

with { MCDATA User having requesting the sending of a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE" and the UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE with disposition notification type of "FILE DOWNLOAD DEFERRED", followed by a SIP MESSAGE with disposition notification type of "FILE DOWNLOAD REQUEST ACCEPTED", followed by SIP MESSAGE with disposition notification type of "FILE DOWNLOAD COMPLETED" }

then { UE (MCDATA Client) responds to each SIP MESSAGE with a SIP 200 (OK) message and delivers suitable notification on the respective remote Client action to the MCDATA User }

}

6.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.2.1, 10.2.4.2.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282, clause 10.2.2.1]

The media storage client shall determine the value of the absolute URI associated with the media storage function of the MCData content server from the <MCDataContentServerURI> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.484 [50]).

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function on the MCData content server, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	a <request-type> element set to a value of "one-to-one-fd"; and

ii)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias;

3)	if a group standalone FD message is to be sent:

a)	if the "/<x>/<x>/Common/MCData/AllowedFD" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request for FD and not continue with the rest of the steps in this subclause; and

b)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	the <request-type> element set to a value of "group-fd";

ii)	the <mcdata-request-uri> element set to the MCData group identity;

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

iv)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias;

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.2.14.3	Test description

6.2.14.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 2 for CO FD as specified in aAnnex A.2.2 are available at the UE for upload.

Preamble:

-	In the <on-network> element of the MCData Service Configuration document the <max-data-size-auto-recv-bytes> element of the <tx-and-rx-control> element shall be set to 0 to indicate non-mandatory download independent from the file size.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	The UE has performed procedure 'UE initiated MCX functional alias status determination and subscription' as specified in TS 36.579-1 [2] clause 5.3.36.

-	The UE has performed procedure 'UE initiated MCX functional alias status change' as specified in TS 36.579-1 [2] clause 5.3.37.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.14.3.2	Test procedure sequence

Table 6.2.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 2 (TS 36.579-7 A.2.2) for CO one-to-one FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE" using the active functional alias. (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 1 | P |
| 3 | Check: Is the content of the uploaded file the same as specified in annex A.2.2? | - | - | 1 | P |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD DEFERRED" for the FD message sent at step 24? | - | - | 2 | P |
| 5 | Check: Does the UE (MCData client) notify the user that the remote client has deferred the acceptance of the download? (NOTE 1) | - | - | 2 | P |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED" for the FD message sent at step 24? | - | - | 2 | P |
| 7 | Check: Does the UE (MCData client) notify the user that the remote client has accepted the download? (NOTE 1) | - | - | 2 | P |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED" for the FD message sent at step 24? | - | - | 2 | P |
| 9 | Check: Does the UE (MCData client) notify the user that the remote client has completed the download? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 2 for CO FD as specified in annex A.2.2. |  |  |  |  |  |

6.2.14.3.3	Specific message contents

Table 6.2.14.3.3-1..6: Void

Table 6.2.14.3.3-1: HTTP POST from the UE (step 2, Table 6.2.14.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.14.3.3-2 |  |  |  |

Table 6.2.14.3.3-2: MCData-Info (Table 6.2.14.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A | NOTE: the element is not encrypted |  |  |

Table 6.2.14.3.3-3: HTTP 201 Created from the SS (step 2, Table 6.2.14.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| uri | tsc_MCData_MSF_URI & "/file-location-2" |  |  |  |

Table 6.2.14.3.3-4: SIP MESSAGE from the UE (step 2, Table 6.2.14.3.2-1; 
step 4, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.14.3.3-5 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.14.3.3-6 |  |  |  |

Table 6.2.14.3.3-5: MCData-Info (Table 6.2.14.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition FUNCTIONAL_ALIAS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.14.3.3-6: FD SIGNALLING PAYLOAD (Table 6.2.14.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP

Table 6.2.14.3.3-7: SIP MESSAGE from the SS (step 4, Table 6.2.14.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.14.3.3-8 |  |  |  |

Table 6.2.14.3.3-8: FD NOTIFICATION (Table 6.2.14.3.3-7)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_DEFERRED

Table 6.2.14.3.3-9: SIP MESSAGE from the SS (step 6, Table 6.2.14.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.14.3.3-10 |  |  |  |

Table 6.2.14.3.3-10: FD NOTIFICATION (Table 6.2.14.3.3-9)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_ACCEPTED

Table 6.2.14.3.3-11: SIP MESSAGE from the SS (step 8, Table 6.2.14.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.14.3.3-12 |  |  |  |

Table 6.2.14.3.3-12: FD NOTIFICATION (Table 6.2.14.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

<END OF CHANGES>