3GPP TSG-RAN5 Meeting #101	R5-236701r1

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2728 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addtion of RedCap RRM TC 16.7.4.4_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add NR Redcap RRM TC - 16.7.4.4_2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR Redcap RRM TC - 16.7.4.4_2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.4.4_2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0347 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To add missing editor’s note that this TC is still incomplete. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.7.4.4_2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.4.4_2.1	Test purpose

To verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.4.4_2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.4_2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.4.

16.7.4.4_2.4	Test description

16.7.4.4_2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.2.4.1 with following exceptions:

-	Table 6.7.4.2.2.4.1-1 is replaced by Table 16.7.4.4_2.4.1-1;

-	Table 6.7.4.2.2.4.1-2 is replaced by Table 16.7.4.4_2.4.1-2;

Table 16.7.4.4_2.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.4_2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.4_2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.4_2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.4_2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.4_2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.4_2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.4_2.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.2.4.2.

16.7.4.4_2.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.2.4.3.

16.7.4.4_2.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.2.5 with following exceptions:

-	Table 6.7.4.2.2.5-1 is replaced by Table 16.7.4.4_2.5-1;

-	Table 6.7.4.2.2.5-2 is replaced by Table 16.7.4.4_2.5-2;

Table 16.7.4.4_2.5-1: Same as Table 16.7.4.4_2_1.5-1

Table 16.7.4.4_2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0+TT | 0+TT |
| Highest DIFF RSRP reported value | 2+TT | 2+TT |

<End of modified section 2>