3GPP TSG-RAN5 Meeting #101	R5-236703r1

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2730 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addtion of RedCap RRM TC 16.7.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add NR Redcap RRM TC - 16.7.5.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements for LTE RSRP accuracy test cases are added. - NR Redcap RRM TC - 16.7.5.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.5.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0347 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Wrong test config is corrected. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.7.5.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE

Editor’s Note: this test case is incomplete in following aspects:

-	TT analysis is still missing.

16.7.5.2.1	Test purpose

To verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 10.2A.2 for SA inter-RAT E-UTRAN RSRP measurements.

16.7.5.2.2	Test applicability

This test applies to all types of NR RedCap UEs with 2Rx from Release 17 onwards and supporting E-UTRA.

16.7.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.5.2.

16.7.5.2.4	Test description

16.7.5.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.5.1.4.1 with following exceptions:

-	Table 6.7.5.1.4.1-1 is replaced by Table 16.7.5.2.4.1-1;

-	Table 6.7.5.1.4.1-2 is replaced by Table 16.7.5.2.4.1-2;

Table 16.7.5.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.7.5.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 16.7.5.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 16.7.5.2-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN FDD |
| 16.7.5.2-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode. E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.5.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.5.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.5.1.4.2.

16.7.5.2.4.3	Message contents

Same as the message contents given in 6.7.5.1.4.3.

16.7.5.2.5	Test requirement

Same as the test requirements given in clause 6.7.5.1.5 with following exceptions:

-	Table 6.7.5.1.5-1 is replaced by Table 16.7.5.2.5-1;

-	Table 6.7.5.1.5-2 is replaced by Table 16.7.5.2.5-2;

-	Table 6.7.5.1.5-3 is replaced by Table 16.7.5.2.5-3;

Table 16.7.5.2.5-1: NR Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
|  | Config 7,8 |  | HD-FDD |
| TDD Configuration | Config 1,4,7,8 |  | N/A |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4,5,7,8 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 20: NRB,c = 51 |
| Gap pattern Id |  |  | 0 |
| PDSCH RMC | Config 1,4,7,8 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| RMSI CORSET RMC | Config 1,4,7,8 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORSET RMC | Config 1,4,7,8 |  | CCR.1.1 FDD |
|  | Config 2,5 |  | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1,4,7,8 |  | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD |
| Initial DL BWP |  |  | DLBWP.0.1 |
| Dedicated DL BWP |  |  | DLBWP.1.1 |
| Initial UL BWP |  |  | ULBWP.0.1 |
| Dedicated UL BWP |  |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.1 RedCap FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1,2,4,5,7,8 | dBm/SCS | -104 |
|  | Config 3,6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| SSB_RPNote3 | Config 1,2,4,5,7,8 | dBm/SCS | -87 |
|  | Config 3,6 |  | -84 |
| IoNote3 | Config 1,2,4,5,7,8 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/18.36 MHz | -56.03 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 16.7.5.2.5-2: E-UTRAN Cell specific test parameters (Cell 2)

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 2 |  |
| Duplex mode | Config 1,2,3,7 |  | FDD |  |
|  | Config 4,5,6,8 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1,2,3,7 |  | N/A |  |
|  | Config 4,5,6,8 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1,2,3,7 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1,2,3,7 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4,5,6,8 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 |  | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| SCH_RPNote5 |  | dBm/15kHz | -81.65 | -121 + ΔBG_offset |
| IoNote5 |  | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.76 + 10log(NRB,c/50) + ΔBG_offset |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table 16.7.5.2.5-3: E-UTRA RSRP absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |  |
|  |  | FDD_B | 19+TT |  |
|  |  | TDD_C | 19+TT |  |
|  |  | FDD_D, TDD_D | 19+TT |  |
|  |  | FDD_E, TDD_E | 19+TT |  |
|  |  | FDD_G | 19+TT |  |
|  |  | FDD_H | 19+TT |  |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |  |
|  |  | FDD_B | 19+TT |  |
|  |  | TDD_C | 19+TT |  |
|  |  | FDD_D, TDD_D | 19+TT |  |
|  |  | FDD_E, TDD_E | 19+TT |  |
|  |  | FDD_G | 19+TT |  |
|  |  | FDD_H | 19+TT |  |
| Extreme Conditions | Test 1 All bands | Test 2 |  |  |
| Lowest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |  |
|  |  | FDD_B | 19+TT |  |
|  |  | TDD_C | 19+TT |  |
|  |  | FDD_D, TDD_D | 19+TT |  |
|  |  | FDD_E, TDD_E | 19+TT |  |
|  |  | FDD_G | 19+TT |  |
|  |  | FDD_H | 19+TT |  |
| Highest reported value (Cell 2) | 59+TT | FDD_A, TDD_A | 19+TT |  |
|  |  | FDD_B | 19+TT |  |
|  |  | TDD_C | 19+TT |  |
|  |  | FDD_D, TDD_D | 19+TT |  |
|  |  | FDD_E, TDD_E | 19+TT |  |
|  |  | FDD_G | 19+TT |  |
|  |  | FDD_H | 19+TT |  |
| Note 1: E-UTRA operating band groups are as defined in TS 36.521-3 [26] clause 3.5.1 |  |  |  |  |

<End of modified section 2>