3GPP TSG-RAN5 Meeting #101	R5-236772r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2967 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5G ProSe information elements for UE policy part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G ProSe information elements for UE policy part were missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding 5G ProSe information elements for UE policy part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink relay will be not applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7C.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Added missing Spare IEs. Fixed the IEs into correct order. Deleted not Rel-17 IEs and Tables. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.7C.1.1	5G ProSe information elements for UE policy part

–	UE policy part when UE policy part type = {ProSeP}

Table 4.7C.1.1-1: UE policy part when UE policy part type = {ProSeP}

| Derivation Path: TS 24.555 [59] Figure 5.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE policy part contents length | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| UE policy part type | ‘0100’B | UE policy part type={ProSeP} |  |
| Spare | ‘0000’B |  |  |
| UE policy part contents={ProSeP contents} | See Table 4.7C.1.1-2 |  |  |

–	ProSeP contents

Table 4.7C.1.1-2: ProSeP contents

| Derivation Path: TS 24.555 [59] Figure 5.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSeP info #1 | ProSeP info with condition DirectDiscovery |  |  |
| ProSeP info #2 | ProSeP info with condition DirectCommunications |  |  |
| ProSeP info #3 | ProSeP info with condition UEtoNetRelayUE |  |  |
| ProSeP info #4 | ProSeP info with condition RemoteUE |  |  |

–	ProSeP info

Table 4.7C.1.1-3: ProSeP info

| Derivation Path: TS 24.555 [59] Figure 5.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSeP info type | ‘0001’B |  | DirectDiscovery |
|  | ‘0010’B |  | DirectCommunications |
|  | ‘0011’B |  | UEtoNetRelayUE |
|  | ‘0100’B |  | RemoteUE |
|  | ‘0101’B |  | UsageInformationReporting |
| Spare | ‘0000’B |  |  |
| Length of ProSeP info contents | Set to the actual length of 'ProSeP info contents’ in bytes |  |  |
| ProSeP info contents | Set according to specific message content |  |  |

| Condition | Explanation |
| --- | --- |
| DirectDiscovery | UE policies for 5G ProSe direct discovery |
| DirectCommunications | UE policies for 5G ProSe direct communications |
| UEtoNetRelayUE | UE policies for 5G ProSe UE-to-network relay UE |
| RemoteUE | UE policies for 5G ProSe remote UE |
| UsageInformationReporting | UE policies for 5G ProSe usage information reporting |

4.7C.1.2	5G ProSe information elements of UE policies for 5G ProSe direct discovery

–	ProSeP info = { UE policies for 5G ProSe direct discovery }

Table 4.7C.1.2-1: ProSeP info = { UE policies for 5G ProSe direct discovery }

| Derivation Path: TS 24.555 [59] Figure 5.3.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ProSeP info type |  | ‘0001’B | UE policies for 5G ProSe direct discovery |  |
| Spare |  | ‘0000’B |  |  |
| Length of ProSeP info contents |  | Set to the actual length of ' ProSeP info contents' in bytes |  |  |
| Validity timer |  | 'FF FF FF FF FF'H | 5 bytes, Expiration UTC time of validity of the UE policies, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Served by NG-RAN |  | See Table 4.7C.1.2-2 with condition Announcing_Open_Model_A |  | Announcing_Open_Model_A |
|  |  | See Table 4.7C.1.2-2 with condition Announcing_Restricted_Model_A |  | Announcing_Restricted_Model_A |
|  |  | See Table 4.7C.1.2-2 with condition Monitoring_Open_Model_A |  | Monitoring_Open_Model_A |
|  |  | See Table 4.7C.1.2-2 with condition Monitoring_Restricted_Model_A |  | Monitoring_Restricted_Model_A |
|  |  | See Table 4.7C.1.2-2 with condition Discoveree_Restricted_Model_B |  | Discoveree_Restricted_Model_B |
|  |  | See Table 4.7C.1.2-2 with condition Discoverer_Restricted_Model_B |  | Discoverer_Restricted_Model_B |
| Not served by NG-RAN |  | See Table 4.7C.1.2-6 |  |  |
| ProSe direct discovery UE ID |  | ‘000002’O |  |  |
| Group member discovery parameters |  | See Table 4.7C.1.2-12 |  |  |
| ProSe identifiers |  | See Table 4.7C.1.2-14 |  |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules |  | See Table 4.7C.1.2-15 |  |  |
| H5DAI |  | ‘000’B | HPLMN 5G DDNMF address information is absent |  |
| Spare |  | ‘0000 0’B |  |  |
| HPLMN 5G DDNMF address information |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| Announcing_Open_Model_A | Announce request procedure for open 5G ProSe direct discovery model A |
| Announcing_Restricted_Model_A | Announce request procedure for restricted 5G ProSe direct discovery model A |
| Monitoring_Open_Model_A | Monitor request procedure for open 5G ProSe direct discovery model A |
| Monitoring_Restricted_Model_A | Monitor request procedure for restricted 5G ProSe direct discovery model A |
| Discoveree_Restricted_Model_B | Discoveree request procedure for restricted 5G ProSe direct discovery model B |
| Discoverer_Restricted_Model_B | Discoverer request procedure for restricted 5G ProSe direct discovery model B |

–	Served by NG-RAN

Table 4.7C.1.2-2: Served by NG-RAN

| Derivation Path: TS 24.555 [59] Figure 5.3.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of served by NG-RAN contents |  | Set to the actual length of 'served by NG-RAN ' in bytes |  |  |
| Authorization for direct discovery info 1 |  | Table 4.7C.1.2-3 with condition Announcing and Open and Model_A |  | Announcing_Open_Model_A |
|  |  | Table 4.7C.1.2-3 with condition Announcing and Restricted and Model_A |  | Announcing_Restricted_Model_A |
|  |  | Table 4.7C.1.2-3 with condition Monitoring and Open and Model_A |  | Monitoring_Open_Model_A |
|  |  | Table 4.7C.1.2-3 with condition Monitoring and Restricted and Model_A |  | Monitoring_Restricted_Model_A |
|  |  | Table 4.7C.1.2-3 with condition Discoveree and Restricted and Model_B |  | Discoveree_Restricted_Model_B |
|  |  | Table 4.7C.1.2-3 with condition Discoverer and Restricted and Model_B |  | Discoverer_Restricted_Model_B |

| Condition | Explanation |
| --- | --- |
| Announcing_Open_Model_A | Announce request procedure for open 5G ProSe direct discovery model A |
| Announcing_Restricted_Model_A | Announce request procedure for restricted 5G ProSe direct discovery model A |
| Monitoring_Open_Model_A | Monitor request procedure for open 5G ProSe direct discovery model A |
| Monitoring_Restricted_Model_A | Monitor request procedure for restricted 5G ProSe direct discovery model A |
| Discoveree_Restricted_Model_B | Discoveree request procedure for restricted 5G ProSe direct discovery model B |
| Discoverer_Restricted_Model_B | Discoverer request procedure for restricted 5G ProSe direct discovery model B |

–	Authorization for direct discovery info

Table 4.7C.1.2-3: Authorization for direct discovery info

| Derivation Path: TS 24.555 [59] Figure 5.3.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of authorization for direct discovery info contents |  | Set to the actual length of ' authorization for direct discovery info ' in bytes |  |  |
| DDT |  | ‘0’B | Direct discovery type is Open | Open |
|  |  | ‘1’B | Direct discovery type is Restricted | Restricted |
| Model |  | ‘0’B | Model A | Model_A |
|  |  | ‘1’B | Model B | Model_B |
| Role |  | ‘0’B | Role is Announcing when Model is A, Role is Discoverer when Model is B | Announcing, Discoverer |
|  |  | ‘1’B | Role is Monitoring when Model is A, Role is Discoveree when Model is B | Monitoring, Discoveree |
| Spare |  | ‘0000 0’B |  |  |
| Authorized PLMN info |  | See Table 4.7C.1.2-4 |  |  |

| Condition | Explanation |
| --- | --- |
| Announcing | Direct discovery role is set to announcing. |
| Monitoring | Direct discovery role is set to monitoring. |
| Discoverer | Direct discovery role is set to discoverer. |
| Discoveree | Direct discovery role is set to discoveree. |
| Model_A | Direct discovery model A is used. |
| Model_B | Direct discovery model B is used. |
| Open | Direct discovery type is set to open. |
| Restricted | Direct discovery type is set to restricted. |

–	Authorized PLMN info

Table 4.7C.1.2-4: Authorized PLMN info

| Derivation Path: TS 24.555 [59] Figure 5.3.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of authorized PLMN info contents |  | Set to the actual length of 'authorized PLMN info ' in bytes |  |  |
| Authorized PLMN 1 |  | See Table 4.7C.1.2-5 |  |  |

–	PLMN ID

Table 4.7C.1.2-5: PLMN ID

| Derivation Path: TS 24.555 [59] Figure 5.3.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCC digit 1 |  | Set according to parameter given in test case |  |  |
| MCC digit 2 |  | Set according to parameter given in test case |  |  |
| MNC digit 3 |  | Set according to parameter given in test case |  |  |
| MCC digit 3 |  | Set according to parameter given in test case |  |  |
| MNC digit 1 |  | Set according to parameter given in test case |  |  |
| MNC digit 2 |  | Set according to parameter given in test case |  |  |

–	Not served by NG-RAN

Table 4.7C.1.2-6: Not served by NG-RAN

| Derivation Path: TS 24.555 [59] Figure 5.3.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of not served by NG-RAN contents |  | Set to the actual length of 'not served by NG-RAN contents' in bytes |  |  |
| PDNNI |  | ‘1’B | UE is authorized to perform 5G ProSe direct discovery when not served by NG-RAN |  |
| Spare |  | ‘0000 000’B |  |  |
| NR radio parameters per geographical area list |  | See Table 4.7C.1.2-7 |  |  |
| Default PC5 DRX configuration |  | See Table 4.7C.1.2-11A |  |  |

–	NR radio parameters per geographical area list

Table 4.7C.1.2-7: NR radio parameters per geographical area list

| Derivation Path: TS 24.555 [59] Figure 5.3.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents |  | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 |  | See Table 4.7C.1.2-8 |  |  |

–	Radio parameters per geographical area info

Table 4.7C.1.2-8: Radio parameters per geographical area info

| Derivation Path: TS 24.555 [59] Figure 5.3.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents |  | Set to the actual length of 'Length of radio parameters per geographical area contents' in bytes |  |  |
| Geographical area |  | See Table 4.7C.1.2-9 |  |  |
| Radio parameters |  | See Table 4.7C.1.2-11 |  |  |
| Spare |  | ‘000 0000’B |  |  |
| MI |  | ‘1’B | Operator managed |  |

–	Geographical area

Table 4.7C.1.2-9: Geographical area

| Derivation Path: TS 24.555 [59] Figure 5.3.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical area contents |  | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 |  | Table 4.7C.1.2-10 with condition Coordinate 1 |  |  |
| Coordinate 2 |  | Table 4.7C.1.2-10 with condition Coordinate 2 |  |  |
| Coordinate 3 |  | Table 4.7C.1.2-10 with condition Coordinate 3 |  |  |

–	Coordinate area

Table 4.7C.1.2-10: Coordinate area

| Derivation Path: TS 24.555 [59] Figure 5.3.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Latitude |  | 35.753056 |  | Coordinate 1 |
|  |  | 35.735278 |  | Coordinate 2 |
|  |  | 35.744167 |  | Coordinate 3 |
| Longitude |  | 139.689167 |  | Coordinate 1 |
|  |  | 139.689167 |  | Coordinate 2 |
|  |  | 139.709167 |  | Coordinate 3 |

–	Radio parameters

Table 4.7C.1.2-11: Radio parameters

| Derivation Path: TS 24.555 [59] Figure 5.3.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters contents |  | Set to the actual length of 'radio parameters contents ' in bytes |  |  |
| Radio parameters contents |  | Table 4.10.1-1: SL-PreconfigurationNR with condition L2RemoteUE |  |  |

–	Default PC5 DRX configuration

Table 4.7C.1.2-11A: Default PC5 DRX configuration

| Derivation Path: TS 24.555 [59] Figure 5.3.2.11a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of default PC5 DRX configuration contents |  | Set to the actual length of 'default PC5 DRX configuration contents' in bytes |  |  |
| Default PC5 DRX configuration contents |  | See Table 4.6.6-38 |  |  |

–	Groupcast parameters

Table 4.7C.1.2-12: Groupcast parameters

| Derivation Path: TS 24.555 [59] Figure 5.3.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of groupcast parameters contents |  | Set to the actual length of 'groupcast parameters contents' in bytes |  |  |
| Application layer group info 1 |  | See Table 4.7C.1.2-13 |  |  |

–	Application layer group info

Table 4.7C.1.2-13: Application layer group info

| Derivation Path: TS 24.555 [59] Figure 5.3.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of application layer group info contents |  | Set to the actual length of 'application layer group info contents' in bytes |  |  |
| Application layer group identifier |  | '01 01'H |  |  |
| ProSe layer-2 group identifier |  | '00 00 10'H |  |  |
| User info ID |  | ‘000000000001’H |  |  |

–	ProSe identifiers

Table 4.7C.1.2-14: ProSe identifiers

| Derivation Path: TS 24.555 [59] Figure 5.3.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of ProSe identifiers contents |  | Set to the actual length of 'ProSe identifiers contents' in bytes |  |  |
| ProSe identifier 1 |  | '00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 01'H |  |  |

–	ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules

Table 4.7C.1.2-15: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.3.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules contents |  | Set to the actual length of 'ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules contents' in bytes |  |  |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule 1 |  | See Table 4.7C.1.2-16 |  |  |

–	ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule

Table 4.7C.1.2-16: ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.3.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule contents |  | Set to the actual length of 'ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rule contents' in bytes |  |  |
| ProSe identifiers |  | See Table 4.7C.1.2-14 |  |  |
| Destination layer-2 ID for initial discovery signalling |  | '00 00 41'H |  |  |

4.7C.1.3	5G ProSe information elements of UE policies for 5G ProSe direct communications

–	ProSeP info = {UE policies for 5G ProSe direct communications}

Table 4.7C.1.3-1: UE policies for 5G ProSe direct communications

| Derivation Path: TS 24.555 [59] Figure 5.4.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSeP info type | ‘0010’B | UE policies for 5G ProSe direct communications |  |
| Spare | ‘0000’B |  |  |
| Length of ProSeP info contents | Set to the actual length of 'ProSeP info contents' in bytes |  |  |
| Validity timer | 'FF FF FF FF FF'H | Expiration UTC time of validity of the UE policies, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Served by NG-RAN | See Table 4.7C.1.3-2 |  |  |
| Not served by NG-RAN | See Table 4.7C.1.3-5 |  |  |
| Privacy config | See Table 4.7C.1.3-11 |  |  |
| 5G ProSe direct communication in NR-PC5 | See Table 4.7C.1.3-16 |  |  |
| ProSe application to path preference mapping rules | See Table 4.7C.1.3-41 |  |  |
| ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules | See Table 4.7C.1.3-43 |  |  |

–	Served by NG-RAN

Table 4.7C.1.3-2: Served by NG-RAN

| Derivation Path: TS 24.555 [59] Figure 5.4.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of served by NG-RAN contents | Set to the actual length of 'served by NG-RAN contents' in bytes |  |  |
| Authorized PLMN | See Table 4.7C.1.3-3 |  |  |

–	Authorized PLMN

Table 4.7C.1.3-3: Authorized PLMN

| Derivation Path: TS 24.555 [59] Figure 5.4.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of authorized PLMN contents | Set to the actual length of 'authorized PLMN contents' in bytes |  |  |
| PLMN ID 1 | See Table 4.7C.1.3-4 |  |  |

–	PLMN ID

Table 4.7C.1.3-4: PLMN ID

| Derivation Path: TS 24.555 [59] Figure 5.4.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCC digit 1 | Set according to parameter given in test case |  |  |
| MCC digit 2 | Set according to parameter given in test case |  |  |
| MCC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 1 | Set according to parameter given in test case |  |  |
| MNC digit 2 | Set according to parameter given in test case |  |  |

–	Not served by NG-RAN

Table 4.7C.1.3-5: Not served by NG-RAN

| Derivation Path: TS 24.555 [59] Figure 5.4.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of not served by NG-RAN contents | Set to the actual length of 'not served by NG-RAN contents' in bytes |  |  |
| PNNI | '1'B | UE is authorized to use 5G ProSe direct communication when not served by NG-RAN |  |
| Spare | '0000 000'B |  |  |
| NR radio parameters per geographical area list | See Table 4.7C.1.3-6 |  |  |
| PC5 DRX configuration for broadcast, groupcast and initial signalling of 5G ProSe direct link establishment | See Table 4.7C.1.3-7 |  |  |

–	Radio parameters per geographical area list

Table 4.7C.1.3-6: Radio parameters per geographical area list

| Derivation Path: TS 24.555 [59] Figure 5.4.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 | See Table 4.7C.1.3-7 |  |  |

–	Radio parameters per geographical area info

Table 4.7C.1.3-7: Radio parameters per geographical area info

| Derivation TS 24.555 [59] Figure 5.4.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area | See Table 4.7C.1.3-8 |  |  |
| Radio parameters | See Table 4.7C.1.3-10 |  |  |
| Spare | '000 0000'B |  |  |
| MI | '1'B | Operator managed |  |

–	Geographical area

Table 4.7C.1.3-8: Geographical area

| Derivation Path: TS 24.555 [59] Figure 5.4.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of geographical area contents | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 | See Table 4.7C.1.3-9 with Condition Coordinate 1 |  |  |
| Coordinate 2 | See Table 4.7C.1.3-9 with Condition Coordinate 2 |  |  |
| Coordinate 3 | See Table 4.7C.1.3-9 with Condition Coordinate 3 |  |  |

–	Coordinate area

Table 4.7C.1.3-9: Coordinate area

| Derivation Path: TS 24.555 [59] Figure 5.4.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Latitude | 35.753056 |  | Coordinate 1 |
|  | 35.735278 |  | Coordinate 2 |
|  | 35.744167 |  | Coordinate 3 |
| Longitude | 139.689167 |  | Coordinate 1 |
|  | 139.689167 |  | Coordinate 2 |
|  | 139.709167 |  | Coordinate 3 |

–	Radio parameters

Table 4.7C.1.3-10: Radio parameters

| Derivation Path: TS 24.555 [59] Figure 5.4.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters contents | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents | See Table 4.10.1-1: SL-PreconfigurationNR |  |  |

–	Privacy config

Table 4.7C.1.3-11: Privacy config

| Derivation Path: TS 24.555 [59] Figure 5.4.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of privacy config contents | Set to the actual length of 'privacy config contents' in bytes |  |  |
| ProSe applications requiring privacy | See Table 4.7C.1.3-12 |  |  |
| Privacy timer | 'FF FF'H | Expressed in units of seconds, after which the UE shall change the source Layer-2 ID self-assigned by the UE |  |

–	ProSe applications requiring privacy

Table 4.7C.1.3-12: ProSe applications requiring privacy

| Derivation Path: TS 24.555 [59] Figure 5.4.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe applications requiring privacy contents | Set to the actual length of ' ProSe applications requiring privacy contents' in bytes |  |  |
| ProSe applications requiring privacy 1 | See Table 4.7C.1.3-13 |  |  |

–	ProSe applications requiring privacy

Table 4.7C.1.3-13: ProSe applications requiring privacy

| Derivation Path: TS 24.555 [59] Figure 5.4.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe applications requiring privacy contents | Set to the actual length of ' ProSe applications requiring privacy contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Geographical areas | See Table 4.7C.1.3-15 |  |  |

–	ProSe identifiers

Table 4.7C.1.3-14: ProSe identifiers

| Derivation Path: TS 24.555 [59] Figure 5.4.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifiers contents | Set to the actual length of ' ProSe identifiers' in bytes |  |  |
| ProSe identifiers 1 | '00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 01'H |  |  |
| ProSe identifiers 2 | '00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 02'H |  |  |

–	Geographical areas

Table 4.7C.1.3-15: Geographical areas

| Derivation Path: TS 24.555 [59] Figure 5.4.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of geographical areas contents | Set to the actual length of 'geographical areas contents' in bytes |  |  |
| Geographical area 1 | See Table 4.7C.1.3-8 |  |  |

–	5G ProSe direct communication in NR-PC5

Table 4.7C.1.3-16: 5G ProSe direct communication in NR-PC5

| Derivation Path: TS 24.555 [59] Figure 5.4.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of 5G ProSe direct communication in NR-PC5 contents | Set to the actual length of '5G ProSe direct communication in NR-PC5 contents' in bytes |  |  |
| Spare | '00 0000'B |  |  |
| PINFMRI | '1'B | ProSe identifier to ProSe NR frequency mapping rules field is present |  |
| Spare | '0'B |  |  |
| ProSe identifier to ProSe NR frequency mapping rules | See Table 4.7C.1.3-17 |  |  |
| ProSe identifier to destination layer-2 ID for broadcast mapping rules | See Table 4.7C.1.3-22 |  |  |
| Groupcast parameters | See Table 4.7C.1.3-24 |  |  |
| ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rules | See Table 4.7C.1.3-26 |  |  |
| ProSe identifier to PC5 QoS parameters mapping rules | See Table 4.7C.1.3-28 |  |  |
| AS configuration | See Table 4.7C.1.3-30 |  |  |
| NR-PC5 unicast security policies | See Table 4.7C.1.3-34 |  |  |
| ProSe identifier to default mode of communication mapping rules | See Table 4.7C.1.3-37 |  |  |
| ProSe identifier to destination layer-2 ID for groupcast mapping rules | See Table 4.7C.1.3-39 |  |  |

–	ProSe identifier to ProSe NR frequency mapping rules

Table 4.7C.1.3-17: ProSe identifier to ProSe NR frequency mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to ProSe NR frequency mapping rules contents | Set to the actual length of 'ProSe identifier to ProSe NR frequency mapping rules contents' in bytes |  |  |
| ProSe identifier to ProSe NR frequency mapping rule 1 | See Table 4.7C.1.3-18 |  |  |

–	ProSe identifier to ProSe NR frequency mapping rule

Table 4.7C.1.3-18: ProSe identifier to ProSe NR frequency mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to ProSe NR frequency mapping rule contents | Set to the actual length of 'ProSe identifier to ProSe NR frequency mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| ProSe NR frequencies with geographical areas list | See Table 4.7C.1.3-19 |  |  |

–	ProSe NR frequencies with geographical areas list

Table 4.7C.1.3-19: ProSe NR frequencies with geographical areas list

| Derivation Path: TS 24.555 [59] Figure 5.4.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe NR frequencies with geographical areas list contents | Set to the actual length of 'ProSe NR frequencies with geographical areas list contents' in bytes |  |  |
| ProSe NR frequencies with geographical areas info 1 | See Table 4.7C.1.3-20 |  |  |

–	ProSe NR frequencies with geographical areas info

Table 4.7C.1.3-20: ProSe NR frequencies with geographical areas info

| Derivation Path: TS 24.555 [59] Figure 5.4.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe NR frequencies with geographical areas info contents | Set to the actual length of 'ProSe NR frequencies with geographical areas info contents' in bytes |  |  |
| ProSe NR frequencies | See Table 4.7C.1.3-21 |  |  |
| Geographical areas | See Table 4.7C.1.3-15 |  |  |

–	ProSe NR frequencies

Table 4.7C.1.3-21: ProSe NR frequencies

| Derivation Path: TS 24.555 [59] Figure 5.4.2.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe NR frequencies contents | Set to the actual length of 'ProSe NR frequencies contents' in bytes |  |  |
| ProSe NR frequency 1 | ARFCN-ValueNR with condition SL_SSB in Table 4.6.3-5 |  |  |

–	ProSe identifier to destination layer-2 ID for broadcast mapping rules

Table 4.7C.1.3-22: ProSe identifier to destination layer-2 ID for broadcast mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for broadcast mapping rules contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for broadcast mapping rules contents' in bytes |  |  |
| ProSe identifier to destination layer-2 ID for broadcast mapping rule 1 | See Table 4.7C.1.3-23 |  |  |

–	ProSe identifier to destination layer-2 ID for broadcast mapping rule

Table 4.7C.1.3-23: ProSe identifier to destination layer-2 ID for broadcast mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for broadcast mapping rule contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for broadcast mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Destination layer-2 ID for broadcast | '00 00 51'H |  |  |

–	Groupcast parameters

Table 4.7C.1.3-24: Groupcast parameters

| Derivation Path: TS 24.555 [59] Figure 5.4.2.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of groupcast parameters contents | Set to the actual length of 'groupcast parameters contents' in bytes |  |  |
| Application layer group info 1 | See Table 4.7C.1.3-25 |  |  |

–	Application layer group info

Table 4.7C.1.3-25: Application layer group info

| Derivation Path: TS 24.555 [59] Figure 5.4.2.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of application layer group info contents | Set to the actual length of 'application layer group info contents' in bytes |  |  |
| Application layer group identifier | '01 01'H |  |  |
| Spare | '0000 0'B |  |  |
| IPv6 | '0'B | IPv6 is not authorized |  |
|  | '1'B | IPv6 is authorized |  |
| IPv4AI | '0'B | IPv4 address is absent |  |
|  | '1'B | IPv4 address is present |  |
| IPv4 | '0'B | IPv4 is not authorized |  |
|  | '1'B | IPv4 is authorized |  |
| ProSe layer-2 group identifier | '00 00 10'H |  |  |
| ProSe group IP multicast address | 10.10.10.2 |  |  |
| IPv4 address | 10.10.10.3 |  |  |

–	ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 4.7C.1.3-26: ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rules contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rules contents' in bytes |  |  |
| ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 | See Table 4.7C.1.3-27 |  |  |

–	ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 4.7C.1.3-27: ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rule contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for unicast initial signalling mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Destination layer-2 ID for unicast initial signalling | '00 00 41'H |  |  |

–	ProSe identifier to PC5 QoS parameters mapping rules

Table 4.7C.1.3-28: ProSe identifier to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to PC5 QoS parameters mapping rules contents | Set to the actual length of 'ProSe identifier to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| ProSe identifier to PC5 QoS parameters mapping rule 1 | See Table 4.7C.1.3-29 |  |  |

–	ProSe identifier to PC5 QoS parameters mapping rule

Table 4.7C.1.3-29: ProSe identifier to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to PC5 QoS parameters mapping rule contents | Set to the actual length of 'ProSe identifier to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Spare | '0000'B |  |  |
| RI | '1'B | Range field is present |  |
| PLAMBRI | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI | '1'B | Maximum flow bit rate field is present |  |
| GFBRI | '1'B | Guaranteed flow bit rate field is present |  |
| PQI | 24 | See Table 5.6.1-1 in TS 23.304 |  |
| Guaranteed flow bit rate | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range | '0000 0001 1111 0100'B | 500 meters |  |

–	AS configuration

Table 4.7C.1.3-30: AS configuration

| Derivation Path: TS 24.555 [59] Figure 5.4.2.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of AS configuration contents | Set to the actual length of 'AS configuration contents' in bytes |  |  |
| SLRB mapping rules | See Table 4.7C.1.3-31 |  |  |

–	SLRB mapping rules

Table 4.7C.1.3-31: SLRB mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of SLRB mapping rules contents | Set to the actual length of 'SLRB mapping rules contents' in bytes |  |  |
| SLRB mapping rule 1 | See Table 4.7C.1.3-32 |  |  |

–	SLRB mapping rule

Table 4.7C.1.3-32: SLRB mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of SLRB mapping rule contents | Set to the actual length of 'SLRB mapping rule contents' in bytes |  |  |
| PC5 QoS profile | See Table 4.7C.1.3-33 |  |  |
| Length of SLRB | Set to the actual length of 'SLRB' in bytes |  |  |
| SLRB | See Table 4.10.1-1 |  |  |

–	PC5 QoS profile

Table 4.7C.1.3-33: PC5 QoS profile

| Derivation Path: TS 24.555 [59] Figure 5.4.2.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of PC5 QoS profile contents | Set to the actual length of 'PC5 QoS profile contents' in bytes |  |  |
| Spare | '0'B |  |  |
| MDBVI | '0'B | Maximum data burst volume field is absent |  |
| AWI | '1'B | Averaging window field is present |  |
| PLOI | '1'B | The octet of the priority level is present |  |
| RI | '1'B | Range field is present |  |
| PLAMBRI | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI | '1'B | Maximum flow bit rate field is present |  |
| GFBRI | '1'B | Guaranteed flow bit rate field is present |  |
| PQI | 24 | See Table 5.6.1-1 in TS 23.304 |  |
| Guaranteed flow bit rate | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range | '0000 0001 1111 0100'B | 500 meters |  |
| Priority level | 4 |  |  |
| Spare | '0000 0'B |  |  |
| Averaging window | '0000 0111 1101 0000'B | 2000ms |  |
| Maximum data burst volume | Not Present |  |  |

–	NR-PC5 unicast security policies

Table 4.7C.1.3-34: NR-PC5 unicast security policies

| Derivation Path: TS 24.555 [59] Figure 5.4.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 | See Table 4.7C.1.3-35 |  |  |

–	NR-PC5 unicast security policy

Table 4.7C.1.3-35: NR-PC5 unicast security policy

| Derivation Path: TS 24.555 [59] Figure 5.4.2.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policy contents | Set to the actual length of 'NR-PC5 unicast security policy contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Security policy | See Table 4.7C.1.3-36 |  |  |
| Geographical areas | See Table 4.7C.1.3-15 |  |  |

–	Security policy

Table 4.7C.1.3-36: Security policy

| Derivation Path: TS 24.555 [59] Figure 5.4.2.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Signalling integrity protection policy | '000'B | Signalling integrity protection not needed |  |
| Spare | ‘0’B |  |  |
| Signalling ciphering policy | '000'B | Signalling ciphering not needed |  |
| Spare | ‘0’B |  |  |
| User plane integrity protection policy | '000'B | User plane integrity protection not needed |  |
| Spare | ‘0’B |  |  |
| User plane ciphering policy | '000'B | User plane ciphering not needed |  |
| Spare | ‘0’B |  |  |

–	ProSe identifier to default mode of communication mapping rules

Table 4.7C.1.3-37: ProSe identifier to default mode of communication mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to default mode of communication mapping rules contents | ‘Set to the actual length of 'ProSe identifier to default mode of communication mapping rules contents' in bytes |  |  |
| ProSe identifier to default mode of communication mapping rule 1 | See Table 4.7C.1.3-38 |  |  |

–	ProSe identifier to default mode of communication mapping rule

Table 4.7C.1.3-38: ProSe identifier to default mode of communication mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to default mode of communication mapping rule contents | ‘Set to the actual length of 'ProSe identifier to default mode of communication mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| DMC | '00'B | Default mode of communication is set to unicast |  |
| Spare | ‘0000 00’B |  |  |

–	ProSe identifier to destination layer-2 ID for groupcast mapping rules

Table 4.7C.1.3-39: ProSe identifier to destination layer-2 ID for groupcast mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for groupcast mapping rules contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for groupcast mapping rules contents' in bytes |  |  |
| ProSe identifier to destination layer-2 ID for groupcast mapping rule 1 | See Table 4.7C.1.3-40 |  |  |

–	ProSe identifier to destination layer-2 ID for groupcast mapping rule

Table 4.7C.1.3-40: ProSe identifier to destination layer-2 ID for groupcast mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to destination layer-2 ID for groupcast mapping rule contents | Set to the actual length of 'ProSe identifier to destination layer-2 ID for groupcast mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| Destination layer-2 ID for groupcast | '00 00 52'H |  |  |

–	ProSe application to path preference mapping rules

Table 4.7C.1.3-41: ProSe application to path preference mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe application to path preference mapping rules contents | Set to the actual length of 'ProSe application to path preference mapping rules contents' in bytes |  |  |
| ProSe application to path preference mapping rule 1 | See Table 4.7C.1.3-42 |  |  |

–	ProSe application to path preference mapping rule

Table 4.7C.1.3-42: ProSe application to path preference mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe application to path preference mapping rule contents | Set to the actual length of 'ProSe application to path preference mapping rule contents' in bytes |  |  |
| PP | '00'B | PC5 preferred |  |
| SI | '1'B | For all ProSe services |  |
| Spare | '0000 0'B |  |  |

–	ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules

Table 4.7C.1.3-43: ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules

| Derivation Path: TS 24.555 [59] Figure 5.4.2.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules contents | Set to the actual length of 'ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules contents' in bytes |  |  |
| ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rule 1 | See Table 4.7C.1.3-44 |  |  |

–	ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rule

Table 4.7C.1.3-44: ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rule

| Derivation Path: TS 24.555 [59] Figure 5.4.2.44 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rule contents | Set to the actual length of 'ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.3-14 |  |  |
| NR Tx profile | FFS |  |  |

4.7C.1.4	5G ProSe information elements of UE policies for 5G ProSe UE-to-network relay UE

–	ProSeP Info = {UE policies for 5G ProSe UE-to-network relay UE}

Table 4.7C.1.4-1: ProSeP Info = {UE policies for 5G ProSe UE-to-network relay UE}

| Derivation Path: TS 24.555[59] Figure 5.5.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSeP info type | ‘0011’B | UE policies for 5G ProSe UE-to-network relay UE |  |
| PAI | ‘0’B |  |  |
| Spare | ‘000’B |  |  |
| Length of ProSeP info contents | Set to the actual length of ProSeP info contents |  |  |
| Validity timer | 'FF FF FF FF FF'O | 5 octets, Expiration UTC time of validity of the UE policies, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Served by NG-RAN | See Table 4.7C.1.4-2 |  |  |
| Not served by NG-RAN | See Table 4.7C.1.4-5 |  |  |
| Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation | See Table 4.7C.1.4-13 |  |  |
| User info ID for discovery | ‘000001’O |  |  |
| RSC info list | See Table 4.7C.1.4-14 |  |  |
| 5QI to PC5 QoS parameters mapping rules | See Table 4.7C.1.4-22 |  |  |
| ProSe identifier to ProSe application server address mapping rules | See Table 4.7C.1.4-24 |  |  |
| 5G PKMF address information | See Table 4.7C.1.4-28 |  |  |
| Privacy timer | 'FF FF FF FF FF'O |  |  |

–	Served by NG-RAN

Table 4.7C.1.4-2: Served by NG-RAN

| Derivation Path: TS 24.555[59] Figure 5.5.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of served by NG-RAN contents | Set to the actual length of 'NG-RAN contents' in bytes |  |  |
| Authorized PLMN list for layer-3 relay UE | See Table 4.7C.1.4-3 |  |  |
| Authorized PLMN list for layer-2 relay UE | See Table 4.7C.1.4-3 |  |  |

–	Authorized PLMN list

Table 4.7C.1.4-3: Authorized PLMN list

| Derivation Path: TS 24.555[59] Figure 5.5.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of authorized PLMN list contents | Set to the actual length of 'authorized PLMN list contents ' in bytes |  |  |
| Authorized PLMN 1 | See Table 4.7C.1.4-4 |  |  |

–	PLMN ID

Table 4.7C.1.4-4: PLMN ID

| Derivation Path: TS 24.555[59] Figure 5.5.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCC digit 1 | Set according to parameter given in test case |  |  |
| MCC digit 2 | Set according to parameter given in test case |  |  |
| MCC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 1 | Set according to parameter given in test case |  |  |
| MNC digit 2 | Set according to parameter given in test case |  |  |

–	Not served by NG-RAN

Table 4.7C.1.4-5: Not served by NG-RAN

| Derivation Path: TS 24.555[59] Figure 5.5.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of not served by NG-RAN contents | Set to the actual length of 'not served by NG-RAN contents' in bytes |  |  |
| NR radio parameters per geographical area list for UE-to-network relay discovery | See Table 4.7C.1.4-6 |  |  |
| NR radio parameters per geographical area list for UE-to-network relay communication | See Table 4.7C.1.4-7 |  |  |
| Default PC5 DRX configuration for UE-to-network relay discovery | See Table 4.7C.1.4-12 |  |  |

–	NR radio parameters per geographical area list for UE-to-network relay discovery

Table 4.7C.1.4-6: NR radio parameters per geographical area list for UE-to-network relay discovery

| Derivation Path: TS 24.555[59] Figure 5.5.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR radio parameters per geographical area list for UE-to-network relay discovery contents | Set to the actual length of 'NR radio parameters per geographical area list for UE-to-network relay discovery contents' in bytes |  |  |
| Radio parameters per geographical area info 1 | See Table 4.7C.1.4-8 |  |  |

–	NR radio parameters per geographical area list for UE-to-network relay communication

Table 4.7C.1.4-7: NR radio parameters per geographical area list for UE-to-network relay communication

| Derivation Path: TS 24.555[59] Figure 5.5.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR radio parameters per geographical area list for UE-to-network relay communication contents | Set to the actual length of 'NR radio parameters per geographical area list for UE-to-network relay communication contents' in bytes |  |  |
| Radio parameters per geographical area info 1 | See Table 4.7C.1.4-8 |  |  |

–	Radio parameters per geographical area info

Table 4.7C.1.4-8: Radio parameters per geographical area info

| Derivation TS 24.555[59] Figure 5.5.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area | See Table 4.7C.1.4-9 |  |  |
| Radio parameters | See Table 4.7C.1.4-11 |  |  |
| Spare | ‘0000000’B |  |  |
| MI | '1'B | Operator managed |  |

–	Geographical area

Table 4.7C.1.4-9: Geographical area

| Derivation Path: TS 24.555[59] Figure 5.5.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of geographical area contents | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 | See Table 4.7C.1.4-10 |  |  |
| Coordinate 2 | See Table 4.7C.1.4-10 |  |  |
| Coordinate 3 | See Table 4.7C.1.4-10 |  |  |

–	Coordinate area

Table 4.7C.1.4-10: Coordinate area

| Derivation Path: TS 24.555[59] Figure 5.5.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Latitude | 35.753056 |  | Coordinate 1 |
|  | 35.735278 |  | Coordinate 2 |
|  | 35.744167 |  | Coordinate 3 |
| Longitude | 139.689167 |  | Coordinate 1 |
|  | 139.689167 |  | Coordinate 2 |
|  | 139.709167 |  | Coordinate 3 |

–	Radio parameters

Table 4.7C.1.4-11: Radio parameters

| Derivation Path: TS 24.555[59] Figure 5.5.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters contents | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents | See Table 4.10.1-1: SL-PreconfigurationNR |  |  |

–	Default PC5 DRX configuration for UE-to-network relay discovery

Table 4.7C.1.4-12: Default PC5 DRX configuration for UE-to-network relay discovery

| Derivation Path: TS 24.555[59] Figure 5.5.2.11a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of default PC5 DRX configuration for UE-to-network relay discovery contents | Set to the actual length of 'default PC5 DRX configuration for UE-to-network relay discovery contents' in bytes |  |  |
| Default PC5 DRX configuration for UE-to-network relay discovery contents | See Table 4.6.6-38 |  |  |

–	Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation

Table 4.7C.1.4-13: Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation

| Derivation Path: TS 24.555[59] Figure 5.5.2.11b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation contents | Set to the actual length of 'default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation contents' in bytes |  |  |
| Default destination layer-2 ID 1 | ‘000040’O |  |  |

–	RSC info list

Table 4.7C.1.4-14: RSC info list

| Derivation Path: TS 24.555[59] Figure 5.5.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC info list contents | Set to the actual length of 'RSC info list contents' in bytes |  |  |
| RSC info 1 | See Table 4.7C.1.4-15 |  |  |

–	RSC info

Table 4.7C.1.4-15: RSC info

| Derivation Path: TS 24.555[59] Figure 5.5.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC info contents | Set to the actual length of 'RSC info contents' in bytes |  |  |
| RSC list | See Table 4.7C.1.4-16 |  |  |
| Security related parameters for discovery | See Table 4.7C.1.4-17 |  |  |
| LI | '10'B | Layer 2 |  |
| CPSI | '0'B | security procedure over control plane is not used |  |
| Spare | '00000'B |  |  |
| NR-PC5 UE-to-network relay security policies | See Table 4.7C.1.4-20 |  |  |
| PDU session parameters of layer-3 relay | Not Present |  |  |

–	RSC list

Table 4.7C.1.4-16: RSC list

| Derivation Path: TS 24.555[59] Figure 5.5.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC list contents | Set to the actual length of 'RSC list contents' in bytes |  |  |
| RSC 1 | '000010'O |  |  |

–	Security related parameters for discovery

Table 4.7C.1.4-17: Security related parameters for discovery

| Derivation Path: TS 24.555[59] Figure 5.5.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Security related parameters validity timer | 'FF FF FF FF FF'O | 5 octets, Expiration UTC time of validity, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Code-sending security parameters | See Table 4.7C.1.4-18 |  |  |
| Code-receiving security parameters | See Table 4.7C.1.4-19 |  |  |

–	Code-sending security parameters

Table 4.7C.1.4-18: Code-sending security parameters

| Derivation Path: TS 24.555[59] Figure 5.5.2.15a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDUSK | '0'B | DUSK field is not included |  |
| PDUIK | '0'B | DUIK field is not included |  |
| PDUCK | '0'B | DUCK and encrypted bitmask fields are not included |  |
| Spare | ‘00000’B |  |  |
| DUSK | Not Present |  |  |
| DUIK | Not Present |  |  |
| DUCK | Not Present |  |  |
| Encrypted bitmask | Not Present |  |  |

–	Code-receiving security parameters

Table 4.7C.1.4-19: Code-receiving security parameters

| Derivation Path: TS 24.555[59] Figure 5.5.2.15b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDUSK | '0'B | DUSK field is not included |  |
| PDUIK | '0'B | DUIK field is not included |  |
| PDUCK | '0'B | DUCK and encrypted bitmask fields are not included |  |
| Spare | ‘00000’B |  |  |
| DUSK | Not Present |  |  |
| DUIK | Not Present |  |  |
| DUCK | Not Present |  |  |
| Encrypted bitmask | Not Present |  |  |

–	NR-PC5 UE-to-network relay security policies

Table 4.7C.1.4-20: NR-PC5 UE-to-network relay security policies

| Derivation Path: TS 24.555[59] Figure 5.4.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 | See Table 4.7C.1.3-35 |  |  |

–	PDU session parameters for layer-3 relay UE

Table 4.7C.1.4-21: PDU session parameters for layer-3 relay UE

| Derivation Path: TS 24.555[59] Figure 5.5.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of PDU session parameters for layer-3 relay UE contents | Set to the actual length of 'PDU session parameters for layer-3 relay UE contents' in bytes |  |  |
| PDU session type | '001'B |  | IPv4 |
|  | '010'B |  | IPv6 |
|  | '011'B |  | IPv4v6 |
| PDNN | '0'B |  |  |
| PSNSSAI | '0'B |  |  |
| PSSCM | '0'B |  |  |
| PATP | '0'B |  |  |
| Spare | '0'B |  |  |
| DNN | Not Present |  |  |
| S-NSSAI | Not Present |  |  |
| SSC mode | Not Present |  |  |
| Access type preference | Not Present |  |  |
| Spare | Not Present |  |  |

–	5QI to PC5 QoS parameters mapping rules

Table 4.7C.1.4-22: 5QI to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.555[59] Figure 5.5.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of 5QI to PC5 QoS parameters mapping rules contents | Set to the actual length of '5QI to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| 5QI to PC5 QoS parameters mapping rule 1 | See Table 4.7C.1.4-23 |  |  |

–	5QI to PC5 QoS parameters mapping rule

Table 4.7C.1.4-23: 5QI to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.555[59] Figure 5.5.2.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of 5QI to PC5 QoS parameters mapping rule contents | Set to the actual length of '5QI to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| 5QI | '00000001'B | 5QI 1 |  |
| PQI | '00010110'B | PQI 22 |  |
| PDB adjustment factor | '00000000'B |  |  |
| RSC list | Not Present |  |  |

–	ProSe identifier to ProSe application server address mapping rules

Table 4.7C.1.4-24: ProSe identifier to ProSe application server address mapping rules

| Derivation Path: TS 24.555[59] Figure 5.5.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to ProSe application server address mapping rules contents | Set to the actual length of 'ProSe identifier to ProSe application server address mapping rules contents' in bytes |  |  |
| ProSe identifier to ProSe application server address mapping rule 1 | See Table 4.7C.1.4-25 |  |  |

–	ProSe identifier to ProSe application server address mapping rule

Table 4.7C.1.4-25: ProSe identifier to ProSe application server address mapping rule

| Derivation Path: TS 24.555[59] Figure 5.5.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifier to ProSe application server address mapping rule contents | Set to the actual length of 'ProSe identifier to ProSe application server address mapping rule contents' in bytes |  |  |
| ProSe identifiers | See Table 4.7C.1.4-26 |  |  |
| AT | '001'B |  | IPv4 |
|  | '010'B |  | IPv6 |
|  | '011'B |  | FQDN |
| Spare | ‘00000’B |  |  |
| ProSe application server address | IPv4 address | 4 octets | IPv4 |
|  | IPv6 address | 16 octets | IPv6 |
|  | FQDN length field and FQDN value |  | FQDN |

–	ProSe identifiers

Table 4.7C.1.4-26: ProSe identifiers

| Derivation Path: TS 24.555[59] Figure 5.3.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of ProSe identifiers contents | Set to the actual length of 'ProSe identifiers contents' in bytes |  |  |
| ProSe identifier 1 | See Table 4.7C.1.4-27 | a sixteen octet OS Id field, a one octet OS App Id length field, and an OS App Id field |  |

–	ProSe identifier

Table 4.7C.1.4-27: ProSe identifier

| Derivation Path: TS 24.555[59] Table 5.3.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OS Id | Set according to parameter given in test case | sixteen octets |  |
| OS App Id length | Set to the actual length of 'OS App Id' in bytes | one octet |  |
| OS App Id | Set according to parameter given in test case |  |  |

–	5G PKMF address information

Table 4.7C.1.4-28: 5G PKMF address information

| Derivation Path: TS 24.555[59] Figure 5.5.2.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of 5G PKMF address information | Set to the actual length of '5G PKMF address information' in bytes |  |  |
| IPv4add | '0'B |  |  |
|  | '1'B |  | IPv4 |
| IPv6add | '0'B |  |  |
|  | '1'B |  | IPv6 |
| FQDN | '0'B |  |  |
|  | '1'B |  | FQDN |
| Spare | ‘00000’B |  |  |
| IPv4 address list | See Table 4.7C.1.4-29 |  | IPv4 |
| IPv6 address list | See Table 4.7C.1.4-30 |  | IPv6 |
| FQDN | FQDN length field and FQDN value |  | FQDN |

–	IPv4 address list

Table 4.7C.1.4-29: IPv4 address list

| Derivation Path: TS 24.555[59] Figure 5.5.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Number of IPv4 addresses | '01'O |  |  |
| IPv4 address 1 | IPv4 address |  |  |

–	IPv6 address list

Table 4.7C.1.4-30: IPv6 address list

| Derivation Path: TS 24.555[59] Figure 5.5.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Number of IPv6 addresses | '01'O |  |  |
| IPv6 address 1 | IPv6 address |  |  |

4.7C.1.5	5G ProSe information elements of UE policies for 5G ProSe remote UE

FFS

4.7C.1.6	5G ProSe information elements of UE policies for 5G ProSe usage information reporting

FFS

4.7C.1.7	5G ProSe information elements of UE policies for 5G ProSe UE-to-UE relay UE

FFS

4.7C.1.8	5G ProSe information elements of UE policies for 5G ProSe end UE

FFS

<End of Changed Section>