3GPP TSG-RAN5 Meeting #101	R5-236789r1

# Chicago, USA, 13th Nov – 17th Nov 2023

Title:							MU discussion on FR2c

Source:			Anritsu

Agenda Item:			5.4.17

Document for:			Endorsement

# 1.	Introduction

There is an action point to define MTSU of FR2 RF test cases for FR2c (n259). In the last RAN5 meeting, we agreed on MTSU/TT values with [ ] for MOP (EIRP, TRP), Frequency error, REFSENS, EIS spherical coverage [1][2][3]. This paper repeats the proposals of [3] for remaining test cases and provides further analysis for spurious test cases to proceed MU discussion on FR2c. Unless otherwise commented, observations and proposals in this paper are for PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Table 1 Summary of the current agreement on FR2c MU

| Test case | MTSU | Relaxation |
| --- | --- | --- |
| MOP (EIRP) | [5.91] dB | - |
| MOP (TRP) | [5.44] dB | - |
| MOP (Spherical coverage) | No agreement | No agreement |
| MPR | No agreement | No agreement |
| Minimum output power | No agreement | No agreement |
| Frequency error | 0.01 ppm | - |
| OBW | No agreement | No agreement |
| SEM | No agreement | No agreement |
| ACLR | No agreement | No agreement |
| REFSENS | [5.95] dB | - |
| EIS spherical coverage | [5.66] dB | - |
| ACS | No agreement | No agreement |
| In-band blocking | No agreement | No agreement |
| Tx Spurious | No agreement | No agreement |
| Rx Spurious | No agreement | No agreement |

# 2.	Discussion

## 2.1.	Testability

Our analysis on testability of each test case which was not agreed in the last meeting has not changed from [3] and summarized in the tables below. Please refer to [4] for the detailed analysis.

Table 2 Summary of testability analysis for FR2c

| Test case | Testability |
| --- | --- |
| MOP (Spherical coverage) | No testability issues SNR = 5.9 dB |
| MPR | No testability issues SNR = 8.7 dB |
| Minimum output power | Relaxation of 1 dB larger than FR2b is required due to low UL power. SNR is the same as FR2b, when the required relaxation is applied. |
| OBW | MTSU for CBW ≤ 200 MHz is the same as FR2b. CBW = 400 MHz is not testable due to low UL power. |
| SEM | No testability issues SNR is the same as FR2b. |
| ACLR | The red marked test points in Table 3 are not testable due to low UL power. SNR is the same as FR2b at the green marked test points in Table 2. |
| ACS | For case 1, the interferer level needs to be relaxed by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz due to high DL power. Case 1 with CBW > 100 MHz and case 2 are not testable. |
| In-band blocking | Same as ACS (case 1) |

Table 3 Testability of ACLR test points for FR2c

| Test ID | Adjacent channel level [dBm] |  | SNR [dB] |  |  |  | Influence of noise [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz – 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 2 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 3 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 4 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 5 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 6 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 7 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 8 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 9 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 10 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 11 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 12 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 13 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 14 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 15 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |

For spurious test cases, testability was discussed up to 80 GHz. To test the second harmonic of the upper edge of n259 (39.5 GHz – 43.5 GHz), it is required to measure up to 87 GHz. We assume that the same testability as f = 80 GHz is applicable to the frequency range 80 GHz – 87 GHz.

According to Table 2 and the above analysis on spurious test cases, we propose the following relaxations/test points for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 1: Do not relax the test requirement of MOP (Spherical coverage) for FR2c.

Proposal 2: Do not relax the test requirement of MPR for FR2c.

Proposal 3: Relax the test requirement of Minimum output power for FR2c by 5.5 dB for CBW = 50 MHz, 8.5 dB for CBW = 100 MHz, 11.5 dB for CBW = 200 MHz, and 14.5 dB for CBW = 400 MHz.

Proposal 4: For FR2c OBW, do not relax the test requirement for CBW  200 MHz. For CBW = 400 MHz, further study is needed to solve testability issue.

Proposal 5: Do not relax the test requirement of SEM for FR2c.

Proposal 6: For FR2c ACLR, do not relax the test requirement of the green marked test points in Table 3 and do not test others.

Proposal 7: For FR2c ACS, relax the test requirement of case 1 by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz, and do not test case 1 for CBW > 100 MHz and case 2.

Proposal 8: For FR2c In-band blocking, relax the test requirement by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz, and do not test CBW > 100 MHz.

Proposal 9: Do not relax the test requirement of General Tx Spurious for FR2c.

Proposal 10: For Rx Spurious, apply the same relaxation as f = 80 GHz to the frequency range 80 GHz – 87 GHz.

## 2.2.	MU element

Uncertainty of the RF power measurement equipment, gNB emulator uncertainty

[ ] is remaining for the uncertainty values for FR2c. We assume that [ ] can be removed.

MU elements for spurious test cases

Following MU elements have specific values for spurious test cases up to f = 80 GHz. We assume that the same value as f = 80 GHz is applicable to the frequency range 80 GHz – 87 GHz.

- Quality of Quiet Zone

- Mismatch

- Uncertainty of the RF power measurement equipment

- Amplifier Uncertainties

- Influence of XPD

- Uncertainty of the Network Analyzer

- Uncertainty of the absolute gain of the calibration antenna

- Quality of the Quiet Zone for Calibration Process

- Influence of the calibration antenna feed cable (Flexing cables, adapters, attenuators, connector repeatability)

Influence of noise

Based on the testability analysis in clause 2.1, influence of noise for FR2c is calculated as below.

Table 4 Influence of noise for FR2c

| Test case | Influence of noise |
| --- | --- |
| MOP (Spherical coverage) | 1.0 dB |
| MPR | 0.55 dB |
| Minimum output power | 1.0 dB (with relaxation) |
| SEM | 1.0 dB |
| ACLR | 1.0 dB (at testable test points) |
| Tx Spurious (80 GHz – 87 GHz) | 0.41 dB (Same as 80 GHz) |
| Rx Spurious (80 GHz – 87 GHz) | 0.64 dB (Same as 80 GHz) |

According to the above analysis, we propose the following uncertainty values for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 11: For FR2c, remove [ ] from the values of Uncertainty of the RF power measurement equipment, gNB emulator uncertainty.

Proposal 12: For Quality of Quiet Zone, Mismatch, Uncertainty of the RF power measurement equipment, Amplifier Uncertainties, Influence of XPD, Uncertainty of the Network Analyzer, Uncertainty of the absolute gain of the calibration antenna, Quality of the Quiet Zone for Calibration Process, Influence of the calibration antenna feed cable, adopt the same uncertainty values as f = 80 GHz to the frequency range 80 GHz – 87 GHz in spurious test cases.

Proposal 13: Adopt Table 4 for uncertainty value of Influence of noise for FR2c.

## 2.3.	MTSU and TT

Based on the above analysis in clause 2.1 and 2.2, total MU for each test case is calculated as below.

Table 5 Summary of total MU in each test case

| Test case | Total MU |
| --- | --- |
| MOP (Spherical coverage) | 6.12 dB |
| MPR | 6.16 dB |
| Minimum output power | 6.76 dB |
| OBW |  |
| BW = 50 MHz | 0.4 % |
| BW = 100 MHz | 0.4 % |
| BW = 200 MHz | 1.3 % |
| SEM | 6.14 dB |
| ACLR | 6.98 dB |
| ACS (case 1) | 8.91 dB |
| In-band blocking | 8.91 dB |
| Tx Spurious | 7.72 dB |
| Rx Spurious | 7.95 dB |

Proposal 14: Adopt Table 5 for MTSU of FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Using the same approach as FR2a and FR2b, test tolerance for FR2c is calculated as below.

Table 6 Summary of TT in each test case

| Test case | TT | Formula |
| --- | --- | --- |
| MOP (Spherical coverage) | 3.07 dB | TT = 0.60 x (MTSUIFF – 1.0) |
| MPR | 3.65 dB | TT = 0.65 x (MTSUIFF - 0.55) |
| Minimum output power |  | TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R |
| BW = 50 MHz | 1.01 dB | ΔSNRmr = 2.77 dB |
| BW = 100 MHz | 0 dB | ΔSNRmr = 4.44 dB |
| BW = 200 MHz | 0 dB | ΔSNRmr = 6.58 dB |
| BW = 400 MHz | 0 dB | ΔSNRmr = 9.07 dB |
| OBW | 0 kHz | - |
| SEM | 3.99 dB | TT = 0.65 x MTSUIFF |
| ACLR |  | TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change - R = 0 dB - TT due to metric change = 1.0 dB |
| BW = 50 MHz (Test ID 1-2, 4-5) | 5.20 dB | ΔSNRmr = 0.32 dB |
| BW = 50 MHz (Test ID 3, 6) | 5.14 dB | ΔSNRmr = 0.25 dB |
| BW = 50 MHz (Test ID 7-9) | 5.38 dB | ΔSNRmr = 0.49 dB |
| BW = 50 MHz (Test ID 13-15) | 5.43 dB | ΔSNRmr = 0.55 dB |
| BW = 100 MHz (Test ID 1-2, 4-5) | 5.50 dB | ΔSNRmr = 0.61 dB |
| BW = 100 MHz (Test ID 3, 6) | 5.38 dB | ΔSNRmr = 0.49 dB |
| BW = 100 MHz (Test ID 7-9) | 5.82 dB | ΔSNRmr = 0.93 dB |
| BW = 200 MHz (Test ID 3, 6) | 5.82 dB | ΔSNRmr = 0.93 dB |
| ACS (case 1) | 0 dB | - |
| In-band blocking | 0 dB | - |
| Tx Spurious | 0 dB | - |
| Rx Spurious | 0 dB | - |

Proposal 15: Adopt Table 6 for TT of FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

In addition, [ ] can be removed from MTSU and TT of MOP (EIRP, TRP), REFSENS, EIS spherical coverage for FR2c, if Proposal 11 is agreed.

Proposal 16: For FR2c, remove [ ] from MTSU and TT of MOP (EIRP, TRP), REFSENS, EIS spherical coverage.

# 3.	Conclusion

RAN5 is asked to endorse following proposals for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 1: Do not relax the test requirement of MOP (Spherical coverage) for FR2c.

Proposal 2: Do not relax the test requirement of MPR for FR2c.

Proposal 3: Relax the test requirement of Minimum output power for FR2c by 5.5 dB for CBW = 50 MHz, 8.5 dB for CBW = 100 MHz, 11.5 dB for CBW = 200 MHz, and 14.5 dB for CBW = 400 MHz.

Proposal 4: For FR2c OBW, do not relax the test requirement for CBW  200 MHz. For CBW = 400 MHz, further study is needed to solve testability issue.

Proposal 5: Do not relax the test requirement of SEM for FR2c.

Proposal 6: For FR2c ACLR, do not relax the test requirement of the green marked test points in Table 3 and do not test others.

Proposal 7: For FR2c ACS, relax the test requirement of case 1 by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz, and do not test case 1 for CBW > 100 MHz and case 2.

Proposal 8: For FR2c In-band blocking, relax the test requirement by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz, and do not test CBW > 100 MHz.

Proposal 9: Do not relax the test requirement of General Tx Spurious for FR2c.

Proposal 10: For Rx Spurious, apply the same relaxation as f = 80 GHz to the frequency range 80 GHz – 87 GHz.

Proposal 11: For FR2c, remove [ ] from the values of Uncertainty of the RF power measurement equipment, gNB emulator uncertainty.

Proposal 12: For Quality of Quiet Zone, Mismatch, Uncertainty of the RF power measurement equipment, Amplifier Uncertainties, Influence of XPD, Uncertainty of the Network Analyzer, Uncertainty of the absolute gain of the calibration antenna, Quality of the Quiet Zone for Calibration Process, Influence of the calibration antenna feed cable, adopt the same uncertainty values as f = 80 GHz to the frequency range 80 GHz – 87 GHz in spurious test cases.

Proposal 13: Adopt Table 4 for uncertainty value of Influence of noise for FR2c.

Proposal 14: Adopt Table 5 for MTSU of FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 15: Adopt Table 6 for TT of FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 16: For FR2c, remove [ ] from MTSU and TT of MOP (EIRP, TRP), REFSENS, EIS spherical coverage.

# 4.	References

[1] R5-233971, “Testability and MU analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#100

[2] R5-235140, “On the MU for n259”, ROHDE & SCHWARZ, RAN5#100

[3] R5-235753, “MU discussion on FR2c”, Anritsu, RAN5#100

[4] R5-232983, “MU discussion on FR2c”, Anritsu, RAN5#99