3GPP TSG-RAN5 Meeting #101 	R5-236818r1

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2772 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 7.6.14.1 for SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NR SA FR2 test case 7.6.14.1 for SA event triggered reporting tests with Pre-MG is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NR SA FR2 test case 7.6.14.1 for SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 7.6.14.1 would remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.14 (new), 7.6.14.1 (new), H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Addition of 7.6.14 (new) was removed and note for implementer was added to solve overlap with R5-236455. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS

<<Unchanged sections skipped>>

Table 7.6.13.2.5-2: SRS Configuration for FR2 Propagation Delay Compensation

|  | PDC-SRS.3 |  |
| --- | --- | --- |
| Field |  | Comment |
| c-SRS | Same as NRB,c in the test case |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 4 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n4 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | Sl1280, 160 | SRS transmission periodicity |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation

Editor's Note: This test case is incomplete in the following aspects:

- TT analysis is missing.

7.6.14.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event with autonomous activation/deactivation of Pre-MG. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 [6] clause 9.2.5.1 and 9.2.5.2.

7.6.14.1.2	Test applicability

This test applies to all types of NR UEs supporting 5GS NR SA FR2, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 capability from release 17 onwards.

7.6.14.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.14.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.14 .1.

7.6.14.1.4	Test description

7.6.14.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.14.1.4.1-1

Table 7.6.14.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 7.6.14.1-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Test environment parameters are given in Table 7.6.14.1.4.1-2.

Table 7.6.14.1.4.1-2: Initial conditions for SA event triggered reporting tests with Pre-MG

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.14.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	The general test parameter settings are set up according to Table 7.6.14.1.4.1-3.

2.	Message contents are defined in clause 7.6.14.1.4.3.

3.	The AoA setup for this test is Setup 3 as defined in clause A.3.15.3. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

Table 7.6.14.1.4.1-3: General test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | PCell (Cell 1) |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | SMTC.1 |  |
| A4-Offset | dB | -120 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Gap Pattern Id |  | 13 |  |
| Measurement gap offset | ms | 39 |  |
| Time offset between Cell 1 and Cell 2 |  | 3 s | Synchronous cells |
| T1 | s | 0.1 |  |
| T2 | s | 0.2 |  |
| T3 | s | 5 |  |

7.6.14.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

There are two different UE-specific BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A4 is used.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Tables 7.6.14.1.4.1-3, 7.6.14.1.5-1, and 7.6.14.1.5-2. Propagation conditions are set according to Annex C clauses C.2.2.

3. SS shall transmit an RRCReconfiguration message which include with Measurement configuration and 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

- Pre-MG status is assumed to be ‘deactivated’ when the BWP is active.

- BWP-1 includes bandwidth of the initial DL BWP and SSB.

- BWP-2 does not include bandwidth of the initial DL BWP and SSB.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions in T1.

6. When T1 expires, at the start of time duration T2, the serving gNB trigger Pre-MG activation starts via the DCI format 1_1 command for PCell DL BWP switch, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. And UE is expected to complete the Pre-MG activation within the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms in T2 as defined in TS 38.133 [6] clause 8.6.2 and clause 8.19.2.

7. At the start of time duration T3, the SS shall switch the power setting from T2 to T3 as specified in Table 7.6.14.1.5-2.

8. The UE shall NOT be able to receive PDSCH and report corresponding ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

9. The UE shall transmit a MeasurementReport message triggered by Event A4. With the overall delays measured from the beginning of time period T3 is less than 3.20 ms for UE supporting power class 1,5, 1.92 ms for UE supporting power class 2, 3, and 4.

10. After the SS receive the MeasurementReport message in step 9 or when T3 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11. Set NR Cell 2 physical cell identity = ((current NR cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

12. Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switcheson the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13. Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.14.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.14.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions GAP NEEDED Table H.3.1-4AA with A4-offset = -120dB Table H.3.1-5 Table H.3.1-6 with conditions Pattern #13, MGe and Pre-CFG Table H.3.1-8 with Condition CSI-RS RLM |

Table 7.6.14.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | entry 1 BWP-1 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP-Id2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 |  |
| defaultDownlinkBWP-Id | 0 | Initial BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | entry 1 BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink with condition BWP-Id2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.6.14.1.4.3-3: BWP-Downlink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.6 |  | BWP-Id1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.5 |  | BWP-Id2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.6.14.1.4.3-4: BWP-Uplink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.6 |  | BWP-Id1 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.5 |  | BWP-Id2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.6.14.1.4.3-5: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 3 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | TBWPswitchDelay | Defined in TS 38.133 [6], Table 8.6.2-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| } |  |  |  |

7.6.14.1.5	Test requirements

Table 7.6.14.1.4.1-3, 7.6.14.1.5-1, and Table 7.6.14.1.5-2 defines the primary level settings for all tests.

Table 7.6.14.1.5-1: NR Cell specific test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 24 |  |  | 24 |  |  |
| Initial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| BWP-1 Configuration |  | DLBWP.1.6 ULBWP.1.6 |  |  | N/A |  |  |
| BWP-2 Configuration |  | DLBWP.1.5 ULBWP.1.5 |  |  | N/A |  |  |
| RLM-RS |  | CSI-RS |  |  | N/A |  |  |
| PDSCH RMC configuration |  | SR.3.2 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | CCR.3.1 TDD |  |  | N/A |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI states |  | TCI.State.2 |  |  | N/A |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |  | 120 |  |  |
| OCNG Patterns |  | OP.5 |  |  | N/A |  |  |
| cellIndividualOffset | dB | N/A |  |  | 16 |  |  |
| SSB |  | SSB.1 FR2 |  |  | SSB.7 FR2 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |

Table 7.6.14.1.5-2: NR OTA Cell specific test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | Setup 3 defined in A.3.15.3 |  |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |  |
| Beam assumptionNote 3 |  | Rough |  |  | Rough |  |  |
| Es | dBm/SCS | -89+TT | -89+TT | -89+TT | -infinity | -89+TT | -89+TT |
| BB Note 4 | dB | -0.12+TT | -0.12+TT | -0.12+TT | -Infinity | -0.12+TT | -0.12+TT |
| SSB_RP | dBm/SCS | -89+TT | -89+TT | -89+TT | -infinity | -89+TT | -89+TT |
|  | dBm/95.04MHz | -64.41+TT | -64.41+TT | -64.41+TT | -Infinity | -64.41+TT | -64.41+TT |
| Time multiplexing of the downlink transmissions from each AoA |  | Defined in Figure 7.6.1.1.1-1 |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

During T1, the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

During T2 and T3, the UE shall not report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms as defined in TS 38.133 [6] clause 8.19.2.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Y ms from the beginning of time period T3, where Y is

-	3.2s for a UE supporting power class 1 and 5,

-	1.92s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

7.7	Measurement performance requirements

<<Unchanged sections skipped>>

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

<<Unchanged tables skipped>>

MeasGapConfig-DEFAULT

Measurement gap configuration.

Table H.3.1-6: MeasGapConfig: measurement gap configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasGapConfig ::= SEQUENCE { |  |  |  |  |  |  |  |
| gapFR2 CHOICE { |  |  |  |  |  | gapFR2 |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #13 |  |
| mgl |  | ms5dot5 |  |  |  | Pattern #13 |  |
| mgrp |  | ms40 |  |  |  | Pattern #13 |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapFR1 CHOICE { |  |  |  |  |  | gapFR1 |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #2 |  |
|  |  | 19 |  |  |  | Pattern #4 |  |
| mgl |  | ms3 |  |  |  | Pattern #2 |  |
|  |  | ms6 |  |  |  | Pattern #4 |  |
| mgrp |  | ms40 |  |  |  | Pattern #2 |  |
|  |  | ms20 |  |  |  | Pattern #4 |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapUE CHOICE { |  |  |  |  |  | gapUE |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #0 OR Pattern #2 OR Pattern #13 |  |
|  |  | 0 |  |  |  | RLM OR BFD |  |
| mgl |  | ms6 |  |  |  | Pattern #0 OR RLM OR BFD |  |
|  |  | ms3 |  |  |  | Pattern #2 |  |
|  |  | ms5dot5 |  |  |  | Pattern #13 |  |
| mgrp |  | ms40 |  |  |  | Pattern #0 OR Pattern #2 OR Pattern #13 OR RLM OR BFD |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ |  | 1 entry |  |  |  | MGe |  |
| GapConfig-r17[1] SEQUENCE{ |  |  |  |  |  |  |  |
| measGapId-r17 |  | 0 |  |  |  | Pattern #0 |  |
|  |  | 1 |  |  |  | Pattern #1 OR Pattern #13 |  |
|  |  | 2 |  |  |  | Pattern #24 |  |
| gapType-r17 |  | perUE |  |  |  |  |  |
| gapOffset-r17 |  | 39 |  |  |  | Pattern #0 OR Pattern #13 |  |
|  |  | 19 |  |  |  | Pattern #1 |  |
|  |  | 11 |  |  |  | Pattern #24 |  |
| mgl-r17 |  | ms6 |  |  |  | Pattern #0 OR Pattern #1 |  |
|  |  | ms5dot5 |  |  |  | Pattern #13 |  |
|  |  | ms10 |  |  |  | Pattern #24 |  |
| mgrp-r17 |  | ms40 |  |  |  | Pattern #0 OR Pattern #13 |  |
|  |  | ms80 |  |  |  | Pattern #1 OR Pattern #24 |  |
| mgta-r17 |  | ms0 |  |  |  |  |  |
| refServCellIndicator-r17 |  | Not present |  |  |  |  |  |
| refFR2-ServCellAsyncCA-r17 |  | Not present |  |  |  |  |  |
| preConfigInd-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | Pre-CFG |  |
| ncsgInd-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | NCSG |  |
| gapAssociationPRS-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | PRS |  |
| gapSharing-r17 |  | Not present |  |  |  |  |  |
| gapPriority-r17 |  | 2 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| Pattern #0 |  | Measurement gap pattern #0 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #1 |  | Measurement gap pattern #1 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #2 |  | Measurement gap pattern #2 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #4 |  | Measurement gap pattern #4 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #13 |  | Measurement gap pattern #13 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #24 |  | Measurement gap pattern #24 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| RLM |  | Measurement gap pattern for RLM tests |  |
| BFD |  | Measurement gap pattern for Beam Failure Dectection tests |  |
| gapFR2 |  | Indicates measurement gap configuration that applies to FR2 only. gapFR2 cannot be configured together with gapUE. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC |  |
| gapFR1 |  | Indicates measurement gap configuration that applies to FR1 only. gapFR1 cannot be configured together with gapUE. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC |  |
| gapUE |  | Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. In (NG)EN-DC, gapUE cannot be set up by NR RRC. |  |
| MGe |  | Indicates measurement gap enhancements configuration |  |
| Pre-CFG |  | Pre-configured measurement gap |  |
| NCSG |  | Measurement gap pattern for NCSG (network controlled small gap) tests |  |
| PRS |  | Measurement gap associated with PRS measurement |  |

<<End of change>>

<<Informal note for the implementer: 7.6.14.1 (new) should be added after 7.6.14.0 (new) added in R5-236455 (CR 2704).>>