3GPP TSG-RAN5 Meeting #101	R5-236964r1

# Chicago, US, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1706 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to spurious emissions co-existence for 2 UL Inter-band CA (38.521-3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Spurious emissions for UE co-existence for two-bands UL Inter-band CA requirements are not aligned with core specification TS 38.101-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct spurious emissions for UE co-existence for 2UL Inter-band CA: - Minimum conformance requirements in clause 6.5B.3.3.2.3 - Test requirements in clause 6.5B.3.3.2.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions for UE co-existence for two-bands UL Inter-band CA requirements will not be aligned with core specification TS 38.101-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2318523 Accompanying discussion document in R5-236443 This CR is for WIC “NR_CADC_NR_LTE_DC_R16-UEConTest“, and additional impact WIC “NR_CADC_NR_LTE_DC_R17-UEConTest” and “NR_CADC_NR_LTE_DC_R18-UEConTest”. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update table formatting resolving empty lines. Clarify WICs in the cover sheet. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

##### 6.5B.3.3.2	Spurious emission band UE co-existence for Inter-band within FR1

Editor's note:	The default and additional test configuration is analysed based on the assumption that only intermodulation products need to be tested. The band combinations without TP analysis in TR 38.905 are incomplete.

6.5B.3.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions for band UE co-existence for inter-band EN-DC.

6.5B.3.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.2.3	Minimum conformance requirements

This clause specifies the additional requirements for the specified uplink EN-DC, for coexistence with protected bands with the single CC uplink assigned to E-UTRA and NR bands for the specified uplink carrier aggregation configurations in Table 6.5B.3.3.2-1. The requirements in Tables 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the requirements in Tables 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5B.3.3.2.3-1 to 6.5B.3.3.2.3-4 and in 6.5.3.2 of [2] and clause 6.6.3.2 of [4] would be considered to be verified by the measurements verifying the uplink single carrier UE to UE co-existence requirements.

-------- Skipped unchanged clauses --------

Table 6.5B.3.3.2.3-2: Spurious emission band UE co-existence limits Rel-16

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |  |
| DC_1_n3 |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |  |  |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
| DC_1_n7 |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |  |  |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_1_n8 |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |  |  |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
| DC_1_n28 |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
| DC_1_n77 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |  |  |
| DC_1_n78 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |  |  |
| DC_1_n79 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |  |  |
| DC_3_n1 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5,16 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
| DC_3_n5 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
| DC_3_n7 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_3_n28 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 13 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |  |
| DC_3_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_7_n1 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5,16 |  |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7,16 |  |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7,16 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_7_n3 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_7_n5 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 7, 6 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 7, 6 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 14 |  |  |
| DC_7_n8 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_7_n28 |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_7_n66 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_7_n78 |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_8_n1 |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |  |  |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |  |  |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |  |  |
| DC_8_n3 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
| DC_8_n20 | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_8_n28 |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9, 12 |  |  |
| DC_8_n41 |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
| DC_8_n77 |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |  |
| DC_8_n78 |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |  |
| DC_11_n77 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_11_n78 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_11_n79 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_12_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_13_n2 |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |  |  |
| DC_13_n66 |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -35 |  | 0.00625 |  | 5 |  |  |
| DC_14_n2 |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |  |  |
| DC_14_n66 |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |  |  |
| DC_19_n77 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_19_n78 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_19_n79 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_20_n1 | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n3 | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_21_n77 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_21_n78 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_21_n79 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_26_n41 |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
| DC_26_n77 |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  | 2 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_26_n78 |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  | 2 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_26_n79 |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |  |
| DC_28_n3 |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |  |
| DC_28_n5 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
| DC_28_n7 |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |  |  |
| DC_28_n77 |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |  |
| DC_28_n78 |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |  |
| DC_28_n79 |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |  |
| DC_39_n41 | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |  |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |  |
| DC_39_n79 |  | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 18 |  |  |
|  |  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 18 |  |  |
| DC_40_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_40_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n77 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n78 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_48_n5 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-3: Spurious emission band UE co-existence limits Rel-17

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |
| DC_1_n3 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 67 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n7 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_1_n8 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n28 | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n41 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8, 20 |
| DC_1_n77 | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n78 | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n79 | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_2_n7 | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_3_n1 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_3_n5 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n7 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_3_n8 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n28 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 13 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_3_n41 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n77 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n79 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_5_n7 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 7, 6 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 7, 6 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 14 |
| DC_5_n30 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_5_n77 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_7_n1 | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7,16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7,16 |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n2 | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n3 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n5 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 7, 6 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 7, 6 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 14 |
| DC_7_n8 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n28 | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n66 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n71 | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n77 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n78 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n80 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_8_n1 | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_8_n3 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3.12 |
|  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5. 12 |
| DC_8_n20 | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_8_n28 | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9, 12 |
| DC_8_n41 | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_8_n77 | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_8_n78 | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_11_n77 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n78 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n79 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_12_n7 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_12_n78 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_13_n2 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n5 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n7 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_13_n48 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n66 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 803 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n71 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n77 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_14_n2 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n30 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n77 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n66 | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_18_n3 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_18_n79 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n1 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n77 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n78 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n79 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_20_n1 | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_20_n3 | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_21_n1 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n28 | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
| DC_21_n77 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n78 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n79 | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n41 | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
| DC_26_n77 | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n78 | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n79 | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_28_n3 | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n5 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
|  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
| DC_28_n7 | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n77 | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n78 | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n79 | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_39_n40 | Frequency range |  | 1805 |  |  |  | 1855 |  | -40 |  | 1 |  | 18 |
|  | Frequency range |  | 1855 |  |  |  | 1880 |  | -15.5 |  | 5 |  | 5, 7, 18 |
| DC_39_n41 | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 5 |
|  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 5, 7, 19 |
| DC_39_n79 | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 18 |
|  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 18 |
| DC_40_n1 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n41 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n78 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n79 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n28 | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_41_n77 | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n78 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n79 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_42_n1 | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_42_n3 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_48_n5 | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_66_n7 | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_66_n38 | Frequency range |  | 2620 |  | - |  | 2645 |  | -15.5 |  | 5 |  | 5, 7, 22 |
|  | Frequency range |  | 2645 |  | - |  | 2690 |  | -40 |  | 1 |  | 5, 22 |

Table 6.5B.3.3.2.3-4: Spurious emission band UE co-existence limits Rel-18

| EN-DC Configuration |  |  |  |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Protected band |  |  |  | Frequency range (MHz) |  |  |  |  |  |  |  |  |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  |  |  |  | NOTE |  |  |  |
| DC_1_n3 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 67 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_1_n7 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_1_n8 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_1_n28 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_1_n41 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 8, 20 |  |  |  |
| DC_1_n77 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
| DC_1_n78 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
| DC_1_n79 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 8 |  |  |  |
| DC_2_n7 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | 1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_3_n1 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_3_n5 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_3_n7 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_3_n8 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_3_n28 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 13 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_3_n41 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_3_n77 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_3_n79 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_5_n7 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 6 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 6 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 14 |  |  |  |
| DC_5_n30 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_5_n77 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_7_n1 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7,16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n2 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | 1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n3 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n5 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 6 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 6 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 14 |  |  |  |
| DC_7_n8 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n28 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n66 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n71 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | 1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n77 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n78 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_7_n80 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_8_n1 |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  |  |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  |  |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  |  |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_8_n3 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3.12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5. 12 |  |  |  |
| DC_8_n20 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 788 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_8_n28 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9, 12 |  |  |  |
| DC_8_n41 |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  |  |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_8_n77 |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 12 |  |  |  |
| DC_8_n78 |  |  |  |  | Frequency range |  |  |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 12 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 12 |  |  |  |
| DC_11_n77 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_11_n78 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_11_n79 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_12_n7 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_12_n78 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_13_n2 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_13_n5 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_13_n7 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_13_n48 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_13_n66 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_13_n71 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_13_n77 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_14_n2 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_14_n30 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_14_n77 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_14_n66 |  |  |  |  | Frequency range |  |  |  | 769 |  |  |  | - |  |  |  | 775 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 805 |  |  |  | -35 |  |  | 0.00625 |  |  |  |  | 5 |  |  |  |
| DC_18_n79 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_18_n3 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_18_n79 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_19_n1 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_19_n77 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_19_n78 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_19_n79 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_20_n1 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 788 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_20_n3 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 788 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 788 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_21_n1 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_21_n28 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_21_n77 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_21_n78 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_21_n79 |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_26_n41 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  |  |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 703 |  |  |  | - |  |  |  | 799 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_26_n77 |  |  |  |  | Frequency range |  |  |  | 703 |  |  |  | - |  |  |  | 799 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_26_n78 |  |  |  |  | Frequency range |  |  |  | 703 |  |  |  | - |  |  |  | 799 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_26_n79 |  |  |  |  | Frequency range |  |  |  | 703 |  |  |  | - |  |  |  | 799 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_28_n3 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_28_n5 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
| DC_28_n7 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_28_n77 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_28_n78 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_28_n79 |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_39_n40 |  |  |  |  | Frequency range |  |  |  | 1805 |  |  |  |  |  |  |  | 1855 |  |  |  | -40 |  |  | 1 |  |  |  |  | 18 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1855 |  |  |  |  |  |  |  | 1880 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 18 |  |  |  |
| DC_39_n41 |  |  |  |  | Frequency range |  |  |  | 1805 |  |  |  | - |  |  |  | 1855 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1855 |  |  |  | - |  |  |  | 1880 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 19 |  |  |  |
| DC_39_n79 |  |  |  |  | Frequency range |  |  |  | 1805 |  |  |  | - |  |  |  | 1855 |  |  |  | -40 |  |  | 1 |  |  |  |  | 18 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1855 |  |  |  | - |  |  |  | 1880 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 18 |  |  |  |
| DC_40_n1 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_40_n41 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_40_n78 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_40_n79 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_41_n28 |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  | 8 |  |  |  |  | 5, 17 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 14 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 662 |  |  |  | - |  |  |  | 694 |  |  |  | -26.2 |  |  | 6 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 758 |  |  |  | - |  |  |  | 773 |  |  |  | -32 |  |  | 1 |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 773 |  |  |  | - |  |  |  | 803 |  |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3, 9 |  |  |  |
| DC_41_n77 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  |  |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_41_n78 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_41_n79 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_42_n1 |  |  |  |  | Frequency range |  |  |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 7, 16 |  |  |  |
| DC_42_n3 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_48_n5 |  |  |  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  | 0.3 |  |  |  |  | 3 |  |  |  |
| DC_66_n7 |  |  |  |  | Frequency range |  |  |  | 2570 |  |  |  | - |  |  |  | 2575 |  |  |  | +1.6 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2575 |  |  |  | - |  |  |  | 2595 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 6, 7 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2620 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 6 |  |  |  |
| DC_66_n38 |  |  |  |  | Frequency range |  |  |  | 2620 |  |  |  | - |  |  |  | 2645 |  |  |  | -15.5 |  |  | 5 |  |  |  |  | 5, 7, 22 |  |  |  |
|  |  |  |  |  | Frequency range |  |  |  | 2645 |  |  |  | - |  |  |  | 2690 |  |  |  | -40 |  |  | 1 |  |  |  |  | 5, 22 |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	To simplify the above Tables, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

-------- Skipped unchanged clauses --------

6.5B.3.3.2.5	Test Requirement

Test requirements for Spurious Emissions UE Co-existence when both LTE and NR carriers are required to be active are release specific and can be found in Table 6.5B.3.3.2.5-1 If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined.

Test requirements for EN-DC only capable devices when using LTE anchor agnostic approach can be found in clause 6.5.3.2.5 in TS 38.521-1 [8].

For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-1 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.5-1: Requirements for inter-band within FR1 according to UE NR release

| EN-DC Configuration | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n28 | Rel-15 | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-15 Rel-16 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Rel-17 Rel-18 | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
| DC_1_n77 | Rel-15 | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  |  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  |  |  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | 1.6 |  | 5 |  | 5, 7, 8 |  |
| DC_1_n78 |  | Rel-15 |  | E-UTRA Band 3, 11, 21, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |
| DC_1_n79 | Rel-15 | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2_n5 | Rel-15 | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n66 | Rel-15 | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | Rel-15 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n78 | Rel-15 | E-UTRA Band 24, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | Rel-15 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n8 | Rel-15 | E-UTRA Band 20, 28, 38, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 7, 22, 41, 42, 43 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n28 | Rel-15 | NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 7, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_3_n77 | Rel-15 | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 | Rel-15 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Rel-15 | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_5_n66 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n77 |  | Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_5_n78 | Rel-15 | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 7 |
| DC_7_n28 | Rel-15 | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7_n78 | Rel-15 | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_8_n3 | Rel-16 Rel-17 Rel-18 | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n20 | Rel-16 Rel-17 Rel-18 | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 |  | Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 491 |  | - |  | 616 |  | -42 |  | 8 |  | 5, 17 |  |
| DC_8_n41 |  | Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |  |
| DC_8_n77 |  | Rel-15 |  | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_8_n78 | Rel-15 | E-UTRA band 34, 39, 40, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  |  | E-UTRA Band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_11_n77 |  | Rel-15 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_11_n79 | Rel-15 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_12_n66 | Rel-15 | E-UTRA Band 41,53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_12_n78 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_19_n77 | Rel-15 | E-UTRA Band 1, 3, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Rel-15 | E-UTRA Band 1, 3, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Rel-15 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_21_n77 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Rel-15 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_21_n79 | Rel-15 | E-UTRA Band 3, 21, 34, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_25_n41 | Rel-15 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_26_n41 |  | Rel-15 |  | E-UTRA Band 3, 5, 18, 19, 26, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |
|  |  |  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
| DC_26_n77 |  | Rel-15 |  | E-UTRA Band 1, 3, 34, 39, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  |  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  |  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_28_n5 |  | Rel-16 |  | Frequency range |  | 557 |  | - |  | 672 |  | -42 |  | 8 |  | 5, 17 |  |
|  | Rel-17 Rel-18 | Frequency range | 557 | - | 672 | -26.2 | 6 | 14 |
|  |  | Frequency range | 662 | - | 672 | -26.2 | 6 | 5 |
| DC_26_n78 | Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 Rel-18 | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n77 | Rel-15 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | Rel-15 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  |  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_28_n79 |  | Rel-15 |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_30_n5 |  | Rel-15 |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_39_n41 | Rel-15 | E-UTRA Band 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_39_n79 | Rel-15 | E-UTRA Band 8, 28, 41, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_40_n41 | Rel-15 | Bands 1, 34, 39, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 19 |  |
| DC_41_n28 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 1 | 3, 9 |
| DC_41_n77 |  | Rel-15 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Rel-15 Rel-16 Rel-17 Rel-18 |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_41_n78 | Rel-15 | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Rel-15 | E-UTRA Band 1, 3, 5, 8, 18, 19, 26, 28, 34, 40, 44, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-15 Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Rel-16 Rel-17 Rel-18 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66_n5 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 26, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n78 | Rel-15 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 – 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink. NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB: B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

-------- End of changes --------