Source:        Huawei, Samsung

Title:           TP analysis on FR1 6.4A.2.2.1 Carrier leakage for CA (2UL CA)

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC carrier leakage, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for carrier leakage of 2UL CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band and intra-band contiguous CA is defined as table below.

Table 6.5.2A.2.2.4.1-1: Test Configuration Table (inter-band CA)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | NA | QPSK | 5 | P_5@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 3 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 4 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 6 | 15 | 50 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 7 | 15 | 50 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 8 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 10 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 12 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 13 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 14 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 15 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 16 | 50 | 15 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 17 | 50 | 15 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 18 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 20 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 22 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 28 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 30 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 31 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| X+1 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| X+2 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.2.1.4.1-1: Test Configuration Table (intra-band contiguous CA)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause [4.3.1] for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 4 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 |  |  |
| 5 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 6 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 |  |  |
| 7 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 8 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 9 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 10 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 11 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 12 | 75 | 25 |  | QPSK | 8 | P_8@67 | S_0@0 |  |  |
| 13 | 75 | 75 | NA | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 14 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 15 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 16 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 17 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 18 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 19 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 20 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 21 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 22 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 23 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 24 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 25 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 26 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 27 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 28 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for carrier leakage for single carrier mode is defined as table below.

Table 6.4.2.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1, 3) |
| 1 |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: When the signalled DC carrier position is at Inner_1RB_Left, use Inner_1RB_Right for UL RB allocation. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, Carrier Leakage for CA is tested under Normal condition. Hence it is proposed to apply normal condition as test environment to align with LTE case.

Proposal 1: Define test environment as Normal for Carrier Leakage in FR1 for UL CA.

Test Subcarrier Spacing

As the Carrier Leakage is a sinusoidal unmodulated signal it is not expected that its power level depends on the choice of subcarrier spacing. Smallest SCS is selected as it is the most common one.

Proposal 2: Select smallest SCS for Carrier Leakage in FR1 for UL CA.

Test Frequencies

In LTE, Carrier Leakage for Inter-band CA and intra-band contiguous CA are tested for Low range and High range. The same test coverage as for LTE would be enough to ensure that the Carrier Leakage obeys the test requirement at low range and high range.

Proposal 3: Test Carrier Leakage for Low range and High range frequencies in FR1 for UL CA.

Test Channel Bandwidth

In LTE, Carrier Leakage is tested for Lowest and Highest NRB. However, as pointed in [3] it is not expected that the carrier leakage power level depends on the channel bandwidth. Hence it proposed to follow NR single carrier to use the Mid NRB for carrier leakage test for inter-band UL CA. For intra-band contiguous UL CA, testing highest NRB_agg will be enough.

Proposal 4: Test Carrier Leakage for Mid channel bandwidth in FR1 for UL inter-band CA. Test highest NRB_agg for intra-band contiguous UL CA.

Test Modulations and waveforms

In LTE Carrier Leakage is tested for test points without MPR. Therefore, DFT-s-OFDM QPSK should be selected as modulation and waveform as it does not have any MPR as configuration suggested for single carrier in[3].

Proposal 5: Test Carrier Leakage with DFT-s-OFDM QPSK.

RB allocation

In LTE, Carrier Leakage for CA is tested with highest allocation without MPR where the partial allocation is located on the lower and upper channel edge for PCC and without any PRB allocation for SCC. For 5G the DC carrier is no longer fixed at the center of the channel bandwidth but can be located anywhere inside the channel. However, Carrier Leakage can only be measured when the RB at the position of the DC carrier is not allocated. Taking this into account the natural choices are Inner_1RB_Left or Inner_1RB_Right as they keep the allocation to a minimum while having no MPR. For intra-band contiguous CA, the MPR for Inner contiguous RB allocation is applied when testing Inner_1RB_Left on PCC and 0RB on SCC, which is the minimum MPR requirements. Thus this RB allocation also applies for intra-band contiguous UL CA.

Proposal 6: Test Carrier Leakage with Inner_1RB_Left for PCC and 0PRB allocation for SCC. In case the signaled carrier position is at Inner_1RB_Left, use Inner_1RB_Right.

Number of test points for CA Carrier Leakage

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 1 | 1 | 2 |

Conclusion

For the inter-band UL CA carrier leakage requirement the test condition is proposed as below:

Proposal 1: Define test environment as Normal for Carrier Leakage in FR1 for UL CA.

Proposal 2: Select smallest SCS for Carrier Leakage in FR1 for UL CA.

Proposal 3: Test Carrier Leakage for Low range and High range frequencies in FR1 for UL CA.

Proposal 4: Test Carrier Leakage for Mid channel bandwidth in FR1 for UL inter-band CA. Test highest NRB_agg for intra-band contiguous UL CA

Proposal 5: Test Carrier Leakage with DFT-s-OFDM QPSK.

Proposal 6: Test Carrier Leakage with Inner_1RB_Left for PCC and 0PRB allocation for SCC. In case the signaled carrier position is at Inner_1RB_Left, use Inner_1RB_Right.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185311, “TP for updating TR 38.905 with FR1 Carrier Leakage test point analysis

[4] 3GPP TS 36.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.

[6] 3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.