3GPP TSG-RAN5 Meeting # 101	R5-237112r1

# Chicago, USA,  13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1450 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common test environment for IoT NTN RF/RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Thales, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common test environment for IoT NTN RF/RRM testing is currently missing in the spec compared to NR NTN TS 38.508-1 test specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added common test environment for IoT NTN RF/RRM testing as per discussion papers R5-2371109 and R5-237111. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 (New), 7.6 (New), 8.2.6 (New), 8.4.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-237109 and R5-237111. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - Updated UE location and ephemeris file for GSO as per R5-237111r1 - Updated LEO600 ephemeris file for editorial correction - Zip file names updated to reflect applicability to RF and RRM only - Bullet points removed in section 5.6.2 |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## 5.5	Reference radio bearer configurations

### 5.5.1	SRB and DRB parameters

#### 5.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 5.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config |  | Not present |  | SCell_AddMod |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n1 | Only one transmission per UL HARQ |  |
| } |  |  |  |  |
| drx-Config |  | Not present |  | SCell_AddMod |
| drx-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |

#### 5.5.1.2	Physical Layer configurations

Table 5.5.1.2-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: Clause 4.8.2.1.6 Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| pdsch-ConfigDedicated |  | PDSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| pucch-ConfigDedicated |  | PUCCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| pusch-ConfigDedicated |  | PUSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| uplinkPowerControlDedicated |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| tpc-PDCCH-ConfigPUCCH |  | Not present |  | SRB1 |
|  |  | TPC-PDCCH-Config-DEFAULT using condition PUCCH | See subclause 4.6.3 | RBC |
| tpc-PDCCH-ConfigPUSCH |  | Not present |  | SRB1 |
|  |  | TPC-PDCCH-Config-DEFAULT using condition PUSCH | See subclause 4.6.3 | RBC |
| cqi-ReportConfig |  | Not present |  | SRB1 |
|  |  | Not present |  | RBC |
| soundingRS-UL-ConfigDedicated |  | Not present |  | SRB1 |
|  |  | Not present |  | RBC |
| antennaInfo CHOICE { |  |  |  |  |
| defaultValue |  | NULL |  |  |
| } |  |  |  |  |
| schedulingRequestConfig |  | Not present |  | SRB1 |
|  |  | SchedulingRequest-Config-DEFAULT | See subclause 4.6.3 | SRB1 and HalfDuplex and (CEmodeA or CEmodeB) |
|  |  | Not present | See subclause 4.6.3 | RBC |
| } |  |  |  |  |
| cqi-ReportConfig-r10 |  | Not present |  | SCell_AddMod |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| SCell_AddMod | Addition or modification of SCell |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HalfDuplex | Used during Type B HalfDuplex Operation |

#### 5.5.1.3	SRB and DRB combinations

##### 5.5.1.3.1	Combinations on DL-SCH and UL-SCH

5.5.1.3.1.1	SRB1 and SRB2 for DCCH + n x AM DRB + m x UM DRB, where n=1 and m=0

This SRB and DRB combination is setup in UE Registration procedure and the Generic Radio Bearer Establishment with UE Test Mode Activated using specific message content - the default RRCConnectionReconfiguration message with condition SRB2-DRB(n, m).

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for IoT NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows:

Longitude: 121.56076999

Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites)

Altitude: 0

### 5.6.2	Ephemeris Information

#### 5.6.2.0	Ephemeris files

The ephemeris information to be used in IoT NTN tests defined in TS 36.521-4 [73] are contained in archives listed in Table 5.6.2-1 which accompany the present document:

Table 5.6.2-1. IoT NTN Ephemeris files

| Type of satellite | Archive |
| --- | --- |
| GSO (7º inclination) | Ephemeris_GSO_RF_RRM.zip |
| NGSO 1 (LEO-600 km) | Ephemeris_LEO600_RF_RRM.zip |
| NGSO 2 (LEO-1200 km) | Ephemeris_LEO1200_RF_RRM.zip |

All IoT NTN test cases defined in sections 6, 7 and 8 in TS 36.521-4 [73], unless otherwise stated, shall use the input in the ephemeris file corresponding to zero Doppler and constant delay for the type of stellite under test, keeping such information constant throughout the duration of each measurement in the test. In the case of NGSO testing, Ephemeris_LEO1200.zip shall be used for the ephemeris values.

#### 5.6.2.1	Assumptions for Ephemeris generation

This section provides the assumptions and steps used to generate Ephemeris files for GSO and NGSO orbits.

Assumptions:

1.	Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200.

2.	Ephemeris files include the full orbit in case of GSO satellite and only satellite visibility window for NGSO satellites.

3.	Each input in the ephemeris files include:

- Information for both ephemeris formats according to 36.331 [17] (stateVectors and OrbitalParameters).

- Resulting elevation angle, one way delay and Doppler effect (in [ppm]) so each test case can select the appropriate Doppler and delay conditions to be used.

4.	UE location is as defined in section 5.6.1.

5.	All possible elevation angles of the SAN as seen by the UE are included in the ephemeris files. Minimum elevation angle restriction will be defined per test case.

6.	Ephemeris granularity used shall comply with the following conditions:

- Enable leveraging of the same ephemeris information values in each period comprising the whole range of System Frame Number within epochTime in servingSatelliteInfo-r17 information element [17].

- For GSO, minimize ephemeris file size.

- For GSO, include an input corresponding to zero Doppler conditions with an absolute Doppler error lower than 50 Hz for a carrier frequency below 2GHz (0.025ppm).

#### 5.6.2.2	Ephemeris Generation

This section provides the steps to generate Ephemeris files for both GSO and NGSO orbits.

The software tool used to generate GSO ephemeris file has been GMAT (from NASA), which offers free licenses.

The steps to generate ephemeris files are as follows:

1. Configure satellite initial vector, time and keplerian representation.

2. Configure the ground station simulating the UE with relevant UE position.

3. Configure the propagators and the force models.

4. Configure UE topocentric coordinate systems.

5. Configure subscribers to represent plots, ground-track view or orbit view and setting the GMAT report file with variables for storage of results.

6. Prepare generation of parameters such as satellite position, speed, range, Delay and Doppler with respect to given elevation angles.

7. Set the mission preparation using the GUI or the script.

8. Generate data.

### 5.6.3	IoT NTN message contents for RF Tx/Rx, Demodulation tests

#### 5.6.3.1	ServingSatelliteInfo-r17 values in SIB31/SIB31-NB

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31/SIB31-NB shall be periodically signalled to the UE during test based on the Ephemeris files generated in clause 5.6.2

# 6	Test environment for Signalling test

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## 7.5	Reference radio bearer configurations

### 7.5.1	SRB and DRB parameters

#### 7.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 7.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5 Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| mac-MainConfig-v1020SEQUENCE { |  |  |  | SCell_AddMod |
| sCellDeactivationTimer-r10 |  | Not present |  |  |
| extendedBSR-Sizes-r10 |  | Not Present |  |  |
| extendedPHR-r10 |  | Not Present |  |  |
|  |  | Setup |  | UL CA |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |
| UL CA | This condition is used for UL CA. |

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for IoT NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Generation

This section provides the means to generate Ephemeris files for GSO and NGSO orbits.

#### 7.6.2.0	Ephemeris files

The ephemeris information to be used in IoT NTN tests defined in TS 36.521-3 [34], are contained in archives listed in Table 5.6.2-1 which accompany the present document.

All test cases defined in sections 13 and 14 in TS 36.521-3 [34], unless otherwise stated, shall use the input in the ephemeris file corresponding to constant non-zero Doppler and constant delay for the type of stellite under test for an elevation angle of 30º, keeping such information constant throughout the duration of each measurement in the test.

#### 7.6.2.1	Assumptions for Ephemeris generation

Same as in section 5.6.2.1.

#### 7.6.2.2	Ephemeris Generation

Same as in section 5.6.2.2.

### 7.6.3	IoT NTN message contents for RRM tests

#### 7.6.3.1	ServingSatelliteInfo-r17 values in SIB31

Same as in section 5.6.3.

# 8	NB-IoT test environment

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

### 8.2.5	NB-IoT Reference radio bearer configurations

#### 8.2.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

## 8.2.6	Test environment for NB-IoT NTN RF/Demod testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for NB-IoT NTN UEs, unless otherwise specified.

### 8.2.6.1	UE location

Same as in section 5.6.1.

### 8.2.6.2	Ephemeris Information

Same as in section 5.6.2.

### 8.2.6.3	NB-IoT NTN message contents for RF Tx/Rx, Demodulation tests

Same as in section 5.6.3.

## 8.3	NB-IoT Test environment for Signalling test

## <<< END OF CHANGES 3>>>

## <<< START OF CHANGES 4>>>

### 8.4.5	NB-IoT Reference radio bearer configurations

#### 8.4.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

### 8.4.6	Test environment for NB-IoT NTN RRM testing

This section defines the test environment which applies to all RRM test cases executed for NB-IoT NTN UEs, unless otherwise specified.

#### 8.4.6.1	UE location

Same as in section 7.6.1.

#### 8.4.6.2	Ephemeris Information

Same as in section 7.6.2.

#### 8.4.6.3	NB-IoT NTN message contents for RRM tests

Same as in section 7.6.3.

## <<< END OF CHANGES 4>>>

Add attached .zip files to TS 36.508 “Ephemeris_GSO_RF_RRM.zip”, “Ephemeris_LEO600_RF_RRM.zip” and “Ephemeris_LEO1200_RF_RRM.zip”