3GPP TSG-RAN5 Meeting # 101	R5-237112r3

# Chicago, USA,  13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1450 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common test environment for IoT NTN RF/RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Thales, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common test environment for IoT NTN RF/RRM testing is currently missing in the spec compared to NR NTN TS 38.508-1 test specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added common test environment for IoT NTN RF/RRM testing as per discussion papers R5-2371109 and R5-237111. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 (New), 7.6 (New), 8.2.6 (New), 8.4.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-237109 and R5-237111. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - Updated UE location and ephemeris file for GSO as per R5-237111r1 - Updated LEO600 ephemeris file for editorial correction - Zip file names updated to reflect applicability to RF and RRM only - Bullet points removed in section 5.6.2 Revision 2: - Ephemeris files removed and only default use ephemeris snapshots added. - Criteria used for ephemeris definition and applicability to different type of satellites added. - Assumptions for ephemeris generation updated. - Steps to generate ephemeris information removed (to be added to 36.905 in next meeting) Revision 3: - Title of clause 5.6.3.1 corrected - Clauses title added for clauses 8.2.6.3.1 and 8.4.6.3.1 |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## 5.5	Reference radio bearer configurations

### 5.5.1	SRB and DRB parameters

#### 5.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 5.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config |  | Not present |  | SCell_AddMod |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n1 | Only one transmission per UL HARQ |  |
| } |  |  |  |  |
| drx-Config |  | Not present |  | SCell_AddMod |
| drx-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |

#### 5.5.1.2	Physical Layer configurations

Table 5.5.1.2-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: Clause 4.8.2.1.6 Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| pdsch-ConfigDedicated |  | PDSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| pucch-ConfigDedicated |  | PUCCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| pusch-ConfigDedicated |  | PUSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| uplinkPowerControlDedicated |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | Not present |  | RBC |
| tpc-PDCCH-ConfigPUCCH |  | Not present |  | SRB1 |
|  |  | TPC-PDCCH-Config-DEFAULT using condition PUCCH | See subclause 4.6.3 | RBC |
| tpc-PDCCH-ConfigPUSCH |  | Not present |  | SRB1 |
|  |  | TPC-PDCCH-Config-DEFAULT using condition PUSCH | See subclause 4.6.3 | RBC |
| cqi-ReportConfig |  | Not present |  | SRB1 |
|  |  | Not present |  | RBC |
| soundingRS-UL-ConfigDedicated |  | Not present |  | SRB1 |
|  |  | Not present |  | RBC |
| antennaInfo CHOICE { |  |  |  |  |
| defaultValue |  | NULL |  |  |
| } |  |  |  |  |
| schedulingRequestConfig |  | Not present |  | SRB1 |
|  |  | SchedulingRequest-Config-DEFAULT | See subclause 4.6.3 | SRB1 and HalfDuplex and (CEmodeA or CEmodeB) |
|  |  | Not present | See subclause 4.6.3 | RBC |
| } |  |  |  |  |
| cqi-ReportConfig-r10 |  | Not present |  | SCell_AddMod |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| SCell_AddMod | Addition or modification of SCell |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HalfDuplex | Used during Type B HalfDuplex Operation |

#### 5.5.1.3	SRB and DRB combinations

##### 5.5.1.3.1	Combinations on DL-SCH and UL-SCH

5.5.1.3.1.1	SRB1 and SRB2 for DCCH + n x AM DRB + m x UM DRB, where n=1 and m=0

This SRB and DRB combination is setup in UE Registration procedure and the Generic Radio Bearer Establishment with UE Test Mode Activated using specific message content - the default RRCConnectionReconfiguration message with condition SRB2-DRB(n, m).

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for IoT NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows:

Longitude: 121.56076999

Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites)

Altitude: 0

### 5.6.2	Ephemeris Information

### Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 5.6.2.0	Assumptions for Ephemeris generation

This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits:

1.	Ephemeris information is generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200.

2.	UE location is as defined in section 5.6.1.

3.	PositionVelocity state vectors are used for EphemerisInfo signaling.

4.	For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km.

5.	One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

6.  Doppler (frequency) shift up to [0.93] ppm for GSO and up to 24ppm for NGSO (LEO-600).

#### 5.6.2.1	Ephemeris for zero Doppler conditions

All eMTC NTN test cases defined in sections 6, 7 and 8 in TS 36.521-4 [73], unless otherwise stated, shall use the ephemeris configuration in tables 5.6.2.1-1 to 5.6.2.1-3, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 5.6.2.1-3 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

- In case UE supports both GSO and NGSO satellites, ephemeris values in Table 5.6.1.2.1-1 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

Table 5.6.2.1-1: SystemInformationBlockType31 – eMTC NTN Ephemeris Information for GSO satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1--2: SystemInformationBlockType31 – eMTC NTN Ephemeris Information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm. |  |  |  |

Table 5.6.2.1-3: SystemInformationBlockType31 –eMTC IoT NTN Ephemeris Information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r1 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

### 5.6.3	eMTC NTN message contents for RF Tx/Rx, Demodulation tests

#### 5.6.3.1	ServingSatelliteInfo-r17 values in SIB31

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

# 6	Test environment for Signalling test

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## 7.5	Reference radio bearer configurations

### 7.5.1	SRB and DRB parameters

#### 7.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 7.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5 Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| mac-MainConfig-v1020SEQUENCE { |  |  |  | SCell_AddMod |
| sCellDeactivationTimer-r10 |  | Not present |  |  |
| extendedBSR-Sizes-r10 |  | Not Present |  |  |
| extendedPHR-r10 |  | Not Present |  |  |
|  |  | Setup |  | UL CA |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |
| UL CA | This condition is used for UL CA. |

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for IoT NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 7.6.2.0	Assumptions for Ephemeris generation

Same as in section 5.6.2.0.

#### 7.6.2.1 Ephemeris for elevation angle 30º

All eMTC NTN RRM test cases defined in in sections 14 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 8.4.6.2.1-1a to 8.4.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 8.4.6.2.1-3a or 8.4.6.2.1-3b shall be used.

Table 7.6.2.1-1a: SystemInformationBlockType31 – eMTC NTN Ephemeris Information (stateVectors format) for GSO satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-1b: SystemInformationBlockType31 – eMTC NTN Ephemeris Information (orbitalParameters format) for GSO satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 266532360 |  |  |
| longitude-r17 | 224998499 |  |  |
| inclination-r17 | 69234262 |  |  |
| meanAnomaly-r17 | 118496066 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-2a: SystemInformationBlockType31- eMTC NTN Ephemeris Information (stateVectors format) for NGSO (LEO-600) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2654549 |  |  |
| positionY-r17 | 4386991 |  |  |
| positionZ-r17 | 1594205 |  |  |
| velocityVX-r17 | 14581 |  |  |
| velocityVY-r17 | -34487 |  |  |
| velocityVZ-r17 | 120182 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-2b: SystemInformationBlockType31- eMTC NTN Ephemeris Information (orbitalParameters format) for NGSO (LEO-600) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 127431687 |  |  |
| eccentricity-r17 | 625504 |  |  |
| periapsis-r17 | 197025193 |  |  |
| longitude-r17 | 89801679 |  |  |
| inclination-r17 | 65355907 |  |  |
| meanAnomaly-r17 | 11209255 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-3a: SystemInformationBlockType31- eMTC NTN Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-3b: SystemInformationBlockType31- eMTC NTN Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 273626880 |  |  |
| eccentricity-r17 | 769300 |  |  |
| periapsis-r17 | 198830479 |  |  |
| longitude-r17 | 89834718 |  |  |
| inclination-r17 | 65473992 |  |  |
| meanAnomaly-r17 | 7822790 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

#### 7.6.2.2 Ephemeris for eMTC NTN RRM UL timing accuracy test cases

Table 7.6.2.2-1: SystemInformationBlockType31- eMTC NTN Ephemeris Information for GSO satellites (minimum Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 7.6.2.2-2: SystemInformationBlockType31- eMTC NTN Ephemeris Information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |

Table 7.6.2.2-3: SystemInformationBlockType31- eMTC NTN Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16989970 |  |  |
| positionY-r17 | 27591785 |  |  |
| positionZ-r17 | 1413211 |  |  |
| velocityVX-r17 | 390 |  |  |
| velocityVY-r17 | -65 |  |  |
| velocityVZ-r17 | 5945 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |

Table 7.6.2.2-4: SystemInformationBlockType31-eMTC NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2654549 |  |  |
| positionY-r17 | 4386991 |  |  |
| positionZ-r17 | 1594205 |  |  |
| velocityVX-r17 | 14581 |  |  |
| velocityVY-r17 | -34487 |  |  |
| velocityVZ-r17 | 120182 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |

Table 7.6.2.2-5: SystemInformationBlockType31- eMTC NTN Ephemeris Information for GSO  satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |

Table 7.6.2.2-6: SystemInformationBlockType31- eMTC NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2401021 |  |  |
| positionY-r17 | 3830217 |  |  |
| positionZ-r17 | 2907041 |  |  |
| velocityVX-r17 | 28879 |  |  |
| velocityVY-r17 | -61410 |  |  |
| velocityVZ-r17 | 105774 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -0-19.95 ppm |  |  |  |

### 7.6.3	eMTC NTN message contents for RRM tests

#### 7.6.3.1	ServingSatelliteInfo-r17 values in SIB31

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 7.6.2 (kept constant throughout the duration of the test).

# 8	NB-IoT test environment

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

### 8.2.5	NB-IoT Reference radio bearer configurations

#### 8.2.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

## 8.2.6	Test environment for NB-IoT NTN RF/Demod testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for NB-IoT NTN UEs, unless otherwise specified.

### 8.2.6.1	UE location

Same as in section 5.6.1.

### 8.2.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 8.2.6.2.0	Assumptions for Ephemeris generation

Same as 5.6.2.0.

#### 8.2.6.2.1	Ephemeris for zero Doppler conditions

All NB-IoT NTN test cases defined in sections 6, 7 and 8 in TS 36.521-4 [73], unless otherwise stated, shall use the ephemeris configuration in tables 8.2.6.2.1-1 to 8.2.6.2.1-3, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 8.2.6.2.1-3 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 8.2.6.2.1-2 shall be used.

- In case UE supports both GSO and NGSO satellites, ephemeris values in Table 8.2.6.2.1-1 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 8.2.6.2.1-2 shall be used.

Table 8.2.6.2.1-1: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 8.2.6.2.1-2: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm. |  |  |  |

Table 8.2.6.2.1-3: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

### 8.2.6.3	NB-IoT NTN message contents for RF Tx/Rx, Demodulation tests

#### 8.2.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31-NB shall be periodically signalled to the UE during test based on the Ephemeris information in clause 8.2.6.1 (kept constant throughout the duration of each measurement in the test).

## 8.3	NB-IoT Test environment for Signalling test

## <<< END OF CHANGES 3>>>

## <<< START OF CHANGES 4>>>

### 8.4.5	NB-IoT Reference radio bearer configurations

#### 8.4.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

### 8.4.6	Test environment for NB-IoT NTN RRM testing

This section defines the test environment which applies to all RRM test cases executed for NB-IoT NTN UEs, unless otherwise specified.

#### 8.4.6.1	UE location

Same as in section 7.6.1.

#### 8.4.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 8.4.6.2.0	Assumptions for Ephemeris generation

Same as in section 7.6.2.0.

#### 8.4.6.2.1 Ephemeris for elevation angle 30º

All NB-IoT NTN RRM test cases defined in in sections 13 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 8.4.6.2.1-1a to 8.4.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 8.4.6.2.1-3a or 8.4.6.2.1-3b shall be used.

Table 8.4.6.2.1-1a: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (stateVectors format) for GSO satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-1b: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (orbitalParameters format) for GSO satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 266532360 |  |  |
| longitude-r17 | 224998499 |  |  |
| inclination-r17 | 69234262 |  |  |
| meanAnomaly-r17 | 118496066 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-2a: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (stateVectors format) for NGSO (LEO-600) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2654549 |  |  |
| positionY-r17 | 4386991 |  |  |
| positionZ-r17 | 1594205 |  |  |
| velocityVX-r17 | 14581 |  |  |
| velocityVY-r17 | -34487 |  |  |
| velocityVZ-r17 | 120182 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-2b: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (orbitalParameters format) for NGSO (LEO-600) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 127431687 |  |  |
| eccentricity-r17 | 625504 |  |  |
| periapsis-r17 | 197025193 |  |  |
| longitude-r17 | 89801679 |  |  |
| inclination-r17 | 65355907 |  |  |
| meanAnomaly-r17 | 11209255 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-3a: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-3b: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (30º elevation angle conditions)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 273626880 |  |  |
| eccentricity-r17 | 769300 |  |  |
| periapsis-r17 | 198830479 |  |  |
| longitude-r17 | 89834718 |  |  |
| inclination-r17 | 65473992 |  |  |
| meanAnomaly-r17 | 7822790 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

#### 8.4.6.2.2 Ephemeris for NB-IoT NTN RRM UL timing accuracy test cases

Table 8.4.6.2.2-1: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (minimum Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 8.4.6.2.2-2: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |

Table 8.4.6.2.2-3: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16989970 |  |  |
| positionY-r17 | 27591785 |  |  |
| positionZ-r17 | 1413211 |  |  |
| velocityVX-r17 | 390 |  |  |
| velocityVY-r17 | -65 |  |  |
| velocityVZ-r17 | 5945 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |

Table 8.4.6.2.2-4: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2654549 |  |  |
| positionY-r17 | 4386991 |  |  |
| positionZ-r17 | 1594205 |  |  |
| velocityVX-r17 | 14581 |  |  |
| velocityVY-r17 | -34487 |  |  |
| velocityVZ-r17 | 120182 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |

Table 8.4.6.2.2-5: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO  satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17129898 |  |  |
| positionY-r17 | 27505094 |  |  |
| positionZ-r17 | 1413939 |  |  |
| velocityVX-r17 | 116 |  |  |
| velocityVY-r17 | 378 |  |  |
| velocityVZ-r17 | -5944 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |

Table 8.4.6.2.2-6: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO-600)  satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 clause 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2401021 |  |  |
| positionY-r17 | 3830217 |  |  |
| positionZ-r17 | 2907041 |  |  |
| velocityVX-r17 | 28879 |  |  |
| velocityVY-r17 | -61410 |  |  |
| velocityVZ-r17 | 105774 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -0-19.95 ppm |  |  |  |

#### 8.4.6.3	NB-IoT NTN message contents for RRM tests

#### 8.4.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31-NB shall be periodically signalled to the UE during test based on the Ephemeris information in clause 8.4.6.2 (kept constant throughout the duration of the test).

## <<< END OF CHANGES 4>>>