3GPP TSG-RAN5 Meeting #101	R5-237124r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0848 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Test point analysis for NR NTN frequency error test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test point analysis for NR NTN Frequency error test case is still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction Test point analysis for NR NTN Frequency error test case 6.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case incomplete without Test point analysis. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Negative maximum Doppler added |  |  |  |  |  |  |  |  |

<< START OF CHANGES >>

## 4.4	Test point analysis for satellite access test cases in TS 38.521-5

This section contains information on test point selection for NR NTN test cases in [10]. The general rules in this section apply to all the NR NTN test cases. Separate analysis is not provided for each single test case unless specific test requirement deviates from the general rules.

General rules of test point selection:

Considering that NR NTN operating bands are defined like FR1 FDD bands, test environment, frequencies, SCSs, channel bandwidths, waveforms, and modulations of a Tx or Rx test for an NR NTN band should be selected based on the same principles of the corresponding single carrier test for an FR1 FDD band.

Table 4.4-1: NR UE transmitter test point selection for NTN

| Subclause | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| 6.2.1 UE maximum output power | 540 | General rules of test point selection apply. Test points of TC 6.2.1 from TS 38.521-1 can be leveraged. | RAN5#98 |
| 6.2.2 Maximum Output Power Reduction (MPR) | contiguous allocation: 1000 | General rules of test point selection apply. Test points of TC 6.2.2 from TS 38.521-1 can be leveraged except for the test point for almost contiguous allocation. Note: There is no test point for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. | RAN5#98 |
| 6.2.3 UE additional maximum output power reduction | 216 for NS_100; 264 for NS_24 | General rules of test point selection apply. AMPR test points for NS_100 and NS_24 in TC 6.2.3 of TS 38.521-1 can be leveraged. | RAN5#100 |
| 6.2.4 Configured transmitted power | 30 | General rules of test point selection apply. Test points of TC 6.2.4 from TS 38.521-1 can be levereage except for Test ID 3, which applies to TDD bands only. | RAN5#99 |
| 6.3.1 Minimum output power | 45 | General rules of test point selection apply. Test points of TC 6.3.1 from TS 38.521-1 can be leveraged. | RAN5#98 |
| 6.3.3 Transmit On/OFF Time Mask | 40 | General rules of test point selection apply. Test points of TC 6.3.3 from TS 38.521-1 can be leveraged. | RAN5#100 |
| 6.4.1 Frequency error | 15 | 38.521-5_TPanalysis_6.4.1_FreqErr_v1.zip | RAN5#101 |
| 6.5.2.2 Spectrum emission mask | contiguous allocation: 144 | General rules of test point selection apply. Test points applicable to PC3 from TC 6.5.2.2 of TS 38.521-1 can be leveraged except for: -: Test IDs for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. - Test IDs that are specified with NOTE 3 in Table 6.5.2.2.4.1-1 of TS 38.521-1, which applies to TDD bands only - [Test IDs listed as applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16] | RAN5#100 |
| 6.5.2.4 Adjacent channel leakage ratio | contiguous allocation: 1000 | General rules of test point selection apply. Test points of TC 6.5.2.4 which are derived from Table 6.2.2.4.1-1 of TS 38.521-1, applicable to PC3, can be leveraged except for: - Test IDs for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. - Test IDs that are specified with NOTE 3 in Table 6.2.2.4.1-1 of TS 38.521-1, which applies to TDD bands only | RAN5#100 |
| 6.5.3.1 General spurious emission | 27 | General rules of test point selection apply. Test points of TC 6.5.3.1 from TS 38.521-1 can be leveraged | RAN5#100 |
| 6.5.3.2 Spurious emissions for UE co-existence | 27 | General rules of test point selection apply. Test points of TC 6.5.3.2 from TS 38.521-1 can be leveraged | RAN5#100 |
| 6.5.4 Transmit intermodulation | 8 | General rules of test point selection apply. Test points of TC 6.5.4 from TS 38.521-1 can be leveraged | RAN5#100 |

Table 4.4-2: NR UE receiver test point selection for NTN

| Subclause | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level | 45 | General rules of test point selection apply. Test points of TC 7.3.2 from TS 38.521-1 can be leveraged | RAN5#100 |
| 7.4 Maximum input level | 3 | General rules of test point selection apply. Test points of TC 7.4 from TS 38.521-1 can be leveraged limiting the modulation to 64QAM. | RAN5#99 |

<< END OF CHANGE >>

Add attached .zip files to TS 36.508 “ 38.521-5_TPanalysis_6.4.1_FreqErr_v1.zip”