Title:	Discussion on test point selection for CA_NS_203 A-MPR in FR2

Source:	China Unicom, Huawei, HiSilicon

Introduction

The purpose of this paper is to provide test point selection proposal for 38.101-2 A-MPR requirements when CA_NS_203 is signalled (band n258). Additional RF emission requirements apply in this scenario in the form of Spurious emissions, meaning this proposal applies for test cases 6.2A.3 (A-MPR for CA) and 6.5A.3.3 (Additional Spurious emission for CA).

Test point selection is considered for PC1, PC2, PC3 and PC4.

Discussion

Since A-MPR for CA_NS_203 in RAN4 is defined together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviors need to be checked:

- UE apply too much A-MPR (more than RAN4 allow)

- UE apply to little A-MPR (causing too much RF emissions)

Case A can be verified in A-MPR test case.

Case B can be verified in A-Spurious test case and does not require check of UE output power if it is ensured that the same test point is tested inside A-MPR test.

A-MPR for CA_NS_203 is defined in 38.101-2 [1] for intra-band contiguous CA as:

For power class 1, A-MPR for CA_NS_203 shall be 6.5dB if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB, otherwise.

For power class 2, 3 and 4, A-MPR for CA_NS_203 shall be 2.5 dB, if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB otherwise.

The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the lowest CC among the configured UL CA.

A-Spurious for CA_NS_203 is defined in 38.101-2 [2] as:

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.4-1: Additional requirements (NS_203)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

Test Environment

RF emission requirements for CA_NS_203 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Normal conditions, and the same selection is appropriate also for Additional Spurious emission.

Test Frequencies

Protected range for CA_NS_203 is below band n258. It is judged that the worst case is when the carrier frequency is as close as possible to the protected range, which implies Low frequency range.

Test Channel Bandwidth

For PC1, PC2, PC3 and PC4, for most UL modulations and cumulative aggregated bandwidths, A-MPR is smaller than MPRC_CA and hence does not need to be considered in the test point selection. The only exceptions are:

- Contiguous allocation on UL CA with DFT-s-BPSK or DFT-s-QPSK and cumulative aggregated BW <=400MHz.

- Contiguous allocation in a single CC with DFT-s-BPSK or DFT-s-QPSK and cumulative aggregated BW<=400 MHz

The second configuration could apply to all power classes, and could be considered as the general test configuration.

The A-MPR requirements are the same for a particular power class for any channel bandwidth. The test channel bandwidth could be selected as Highest aggregated bandwidth with cumulative aggregated BW <=400MHz.

  - Test Subcarrier Spacing

RF emission requirements for CA_NS_203 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with 120 kHz. Selection can therefore be reused from normal spurious test case which means 120 kHz.

  - UL Modulation

As mentioned in section 2.3, the option 2 could be considered as the general test configuration for all power classes.

Regular spurious emissions for FR2 is tested with QPSK DFT-s-OFDM modulation and this could be adopted by A-SE test case.

  - Uplink RB allocation

As mentioned in section 2.3, the option 2 could be considered as the general test configuration for all power classes. The single carrier MPR will apply. The Uplink RB allocation needs to be selected so that MPR is less than A-MPR. For all power classes, Inner RB Region 1 will lead to a 0dB MPR and could be selected as test configuration.

  - Check of alignment of test parameters between power measurement test case (A-MPR) and RF emissions test case (A-Spur)

Now that test point proposal is completed, it is required to check that all test points selected in RF emissions test case (Spurious in this case) is covered in A-MPR test case.

Result:

The test points in this paper matches the test points in A-Spurious and A-MPR test cases in TS38.521-2 for all power classes.

Proposal

Following test points are proposed for CA_NS_203 A-MPR and A-Spur test cases (PC1, PC2, PC3 and PC4)

- Test environment: Normal Conditions

- Test frequencies: Low range

- Test Channel Bandwidth: Highest aggregated bandwidth with cumulative aggregated BW <=400MHz

- Test SCS: 120 kHz

- RB allocation and modulation: Inner RB Region 1 (DFT-s-OFDM QPSK )

Number of Test Points

The number of test points for NS_203 A-MPR is 1

The number of test points for NS_203 A-Spurious is 1.

References

- TS 38.101-2 v17.8.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)