3GPP TSG-RAN WG5 Meeting #101                                                 	R5-237153r1

# Chicago, USA, 13 – 17 November 2023

Agenda Item:	5.3.5.11

Source: 	Ericsson

Title: 	Discussion regarding limiting number of test frequencies for CA combinations 38.508-1

Document for:	Discussion and Endorsement

# Introduction

The tables for test frequencies for CA combinations, FR1 and FR2, including UL CA, in 38.508-1 will be very long if all carrier bandwidth (CBW) combination should be added. There is a need to discuss how to limit the number of combinations to be added in the tables.

The examples below affect tables in 38.508-1 clause 4.3.1.2.4	“NR intra-band non-contiguous CA configurations” and 4.3.1.1.5 “NR intra-band non-contiguous CA configurations in FR1”

# Discussion

To include all CBW combinations of each sub-block (SB) and SB’s CC would create a huge number of combinations and is not realistic, see example below.

Example 1: FR2 CA_n260(3A-O-2Q)

The configuration include 6 SBs (“3A-O-2Q” => A+A+A+O+Q+Q) where A, O and Q are FR2 bandwidth classes (BWC)

BWC A has 1CC, BWC O has 2CC, BWC Q has 4 CC (ref 38.101-2, clause 5.3A.4)

The Maximum supported Aggregated CBW for CA_n260(3A-O-2Q) is 2200 MHz (Ref 38.101-2, Table 5.5A.2-2

The SB/CC structure of CA_n260(3A-O-2Q) is:

| Onfig BWC-groups | 3A |  |  | O | 2Q |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sub-block | SB1 | SB2 | SB3 | SB4 | SB5 | SB6 |
| BWC | A | A | A | O | Q | Q |
| #CCs | 1 | 1 | 1 | 2 | 4 | 4 |
| SB CBW | CBWSB1_CC1 | CBWSB2_CC1 | CBWSB3_CC1 | CBWSB4_CC1 + CBWSB4_CC2 | CBWSB5_CC1 + CBWSB5_CC2 + CBWSB5_CC3 + CBWSB5_CC4 | CBWSB6_CC1 + CBWSB6_CC2 + CBWSB6_CC3 + CBWSB6_CC4 |
| Aggregated CBW | Sum of all CBW of all SB CC components (13, CBWSB1_CC1 to CBWSB6_CC4) |  |  |  |  |  |

For n260 and SCS=120 kHz case the supported CBWs are: 50,100.200, 400 MHz, where 400 MHz is optional (Ref: 38.101-2 v18.2.0, Table 5.4.5-1)

The number of CBWSBx_CCx combinations for the four CBW values (50,100.200, 400 MHz) would end up in 4^(13-1)= 16777218 CBWSBx_CCx combinations. The limitation of Max Aggregated CBW=2200 MHz of the configurations as specified in 38.101-2 would reduce the number but still the number of combinations would be huge.

Example 2: FR1 CA_n77(2A)

The configuration includes only 2 SB, however the supported CBW for BCS 1 are: 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100, (12 CBW).

This gives 12*12 = 144 CBW combinations including “Low” and “High” range. Even this creates too long tables with test frequencies.

Observation 1: The number of test frequencies/CA CBW combinations will be too high if test frequencies for all CBW combinations are to be specified.

Proposal 1: Limit the combinations to only cover the following:

- Maximum supported Aggregated CBW (corresponding to Highest NRB_agg), according to ref [2].

- Minimum supported Aggregated CBW (corresponding to Lowest NRB_agg), according to ref [2].

- Default test frequencies for signaling tests as stated in ref [1].

Copy of ref [2] below:

NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested.

# Conclusion

Observation 1: The number of test frequencies/CA CBW combinations will be too high for CA CBW combinations.

Proposal 1: Limit the combinations to only cover the following for test frequencies for CA combinations, FR1 and FR2, including UL CA:

- Maximum supported Aggregated CBW (corresponding to Highest NRB_agg), according to [2].

- Minimum supported Aggregated CBW (corresponding to Lowest NRB_agg), according to [2].

- Default test frequencies for signaling tests as stated in ref [1].

# References

[1]		38.508-1 User Equipment (UE) conformance specification; Part 1: Common test environment

v18.0.0, clause 6.2.3

[2]		38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; v18.0.0, Table 6.2A.2.1.4.1-2b, Note 4