3GPP TSG-RAN5 Meeting #101	R5-237155r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2831 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NTN parallel measurement gap-based inter-frequency test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NTN parallel measurement gap inter-frequency test cases have been added in 38.133 and are missing in 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new test cases 14.5.2.4 and 14.5.2.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.5.2.4, 14.5.2.5, 14.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: fixed 3GU coversheet issues |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

14.5.2.4.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.4.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.4.

14.5.2.4.4	Test description

Three cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1, NR FR1 neighbour cell (Cell 2) on NR RF channel 2, and NR FR1 neighbour cell (Cell 3) also on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.4.4.1-1, 14.5.2.4.4.1-3 and 14.5.2.4.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Two parallel non-overlapping measurement gap pattern configurations, MeasGapId #0 and MeasGapId #1, are provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.4.4.1-1.

Table 14.5.2.4.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.4-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.4-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.4.4.1-2.

Table 14.5.2.4.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.4.4.1-3.

2.	Message contents are defined in clause 14.5.2.4.4.3.

3.	Cell 1, Cell 2 and Cell 3 are NR cells (PCell and two neighbor cells, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1, Cell 2 and Cell 3 are satellite access cells].

Table 14.5.2.4.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR Cell2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 for MeasGapId #0 0 for MeasGapId #1 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 for MeasGapId #0 19 for MeasGapId #1 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cell 2,3 |  | Config 1,2 | 3ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | [1] |  |

14.5.2.4.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.4.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, two parallel gap configurations, and two SMTC configurations, respectively, according to Tables 14.5.2.4.4.1-3 and 14.5.2.4.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.4.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [922] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.4.4.3	Message contents

FFS

14.5.2.4.5	Test requirements

Tables 14.5.2.4.4.1-3 and 14.5.2.4.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access.

Table 14.5.2.4.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  | 2 |  |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |  |  |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |  |  |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.5 FR1 |  | SSB.1 FR1 |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.Y |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity |  | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.26+TT | -70.05+TT |  | -62.26+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [920] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.2.5	Void

#### 14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Whether GNSS configuration is needed is FFS

- Test requirement is TBD in 38.133

14.5.2.6.1	Test purpose

The purpose of this test is to verify that the multiple gaps capable UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 14.5.2.0.1.

14.5.2.6.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and 2 parallel measurement gaps (parallelMeasurementGap-r17, TS 38.306).

14.5.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.2.6.

14.5.2.6.4	Test description

Three cells are deployed in the test, which are NR FR1 PCell (Cell 1) on NR RF channel 1, NR FR1 neighbour cell (Cell 2) on NR RF channel 2, and NR FR1 neighbour cell (Cell 3) also on NR RF channel 2. The test parameters for PCell and neighbour cell are given in Tables 14.5.2.6.4.1-1, 14.5.2.6.4.1-3 and 14.5.2.6.5-1 below. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. Two parallel partially overlapping measurement gap pattern configurations, MeasGapId #0 and MeasGapId #1, are provided to the UE.

[During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.]

14.5.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.2.6.4.1-1.

Table 14.5.2.6.4.1-1: Supported test configurations for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Configuration | Description |
| --- | --- |
| 14.5.2.6-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.2.6-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.2.6.4.1-2.

Table 14.5.2.6.4.1-2: Initial conditions for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.2.6.4.1-3.

2.	Message contents are defined in clause 14.5.2.6.4.3.

3.	Cell 1, Cell 2 and Cell 3 are NR cells (PCell and two neighbor cells, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1, Cell 2 and Cell 3 are satellite access cells].

Table 14.5.2.6.4.1-3: General test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR Cell2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 for MeasGapId #0 1 for MeasGapId #1 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 79 for MeasGapId #0 4 for MeasGapId #1 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |
| Hysteresis | dB | Config 1,2 | 0 |  |
| CP length |  | Config 1,2 | Normal |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |
| Filter coefficient |  | Config 1,2 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1,2 | 3s | Cell 1 and Cell 2 are synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1,2 | 5ms | Cell 1 and Cell 3 are asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2 | 5 |  |
| T2 | s | Config 1,2 | 1.5 |  |

14.5.2.6.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.2.6.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an inter-frequency measurement object, event A3, two parallel gap configurations, and two SMTC configurations, respectively, according to Tables 14.5.2.6.4.1-3 and 14.5.2.6.5-1.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.2.6.5-1. T2 starts.

6.	[UE shall transmit a MeasurementReport message triggered by event A3]. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.2.6.4.3	Message contents

FFS

14.5.2.6.5	Test requirements

Tables 14.5.2.6.4.1-3 and 14.5.2.6.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access.

Table 14.5.2.6.5-1: Cell specific test parameters for NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  | 2 |  |  |
| Satellite information |  |  | Config 1 | SSC.1 |  | NSC.1 |  |  |  |  |
|  |  |  | Config 2 | SSC.2 |  | NSC.2 |  |  |  |  |
| Duplex mode |  |  | Config 1,2 | FDD |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.1 FDD |  | NA |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 FDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 FDD |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.2 |  | SMTC.2 |  | SMTC.Y |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity |  | -91+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
|  |  | dB | Config 1,2 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity |  | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT | -70.05+TT | -62.26+TT | -70.05+TT |  | -62.26+TT |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<< End of Changes >>