3GPP TSG-RAN5 Meeting #101	R5-237181r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0561 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of scheduling restrictions PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RLM tests with scheduling restrictions require support of pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap optional capabilities. There are currently no PICS defined for any of these two. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new PICS for the above mentioned optional capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case applicability will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: fixed 3GU coversheet issues |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item |  |  |  |  |  |  | UE Physical Layer Baseline Implementation Capabilities |  |  |  |  |  |  | Ref. |  |  |  |  |  |  | Release |  |  |  |  |  |  | Mnemonic |  |  |  |  |  |  | M |  |  |  |  |  |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  |  |  |  |  |  | Comments |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  | Support PDSCH reception based on semi-persistent scheduling |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_downlinkSPS |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  | Support 256QAM for PDSCH for FR1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdsch_256QAM_FR1 |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  | Support 256QAM for PDSCH for at least one NR FR2 band |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdsch_256QAM_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | Support 256QAM for PUSCH for at least one NR FR1 band |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pusch_256QAM_FR1 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4a |  |  |  |  |  |  | Support 256QAM for PUSCH for at least one NR FR2 band |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pusch_256QAM_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdsch_MappingTypeA |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  | Support receiving PDSCH using PDSCH mapping type B |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdsch_MappingTypeB |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | Support resource allocation Type 0 for PUSCH |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_ra_Type0_PUSCH |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  | Support scaling factor 0.75 is applied to the band in the max data rate calculation |  |  |  |  |  |  | 38.306, 4.2.7 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_scalingFactor0dot75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_csi_RS_CFRA_ForHO |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  | Support Type 1 PUSCH transmissions with configured grant |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_configuredUL_GrantType1 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  | Support Type 2 PUSCH transmissions with configured grant |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_configuredUL_GrantType2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdsch_RepetitionMultiSlots |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) |  |  |  |  |  |  | 38.306, 4.2.7.7 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_dynamicSwitch_SUL |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset |  |  |  |  |  |  | 38.306, 4.2.7.8 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_nrMIMO_CB_PUSCH |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value different from "oneLayer" |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | Supports MIMO layers at the UE for PUSCH transmission using non-codebook precoding |  |  |  |  |  |  | 38.306, 4.2.7.8 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_nrMIMO_NonCB_PUSCH |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Set to true if maxNumberMIMO-LayersNonCB-PUSCH has value different from "oneLayer" |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  | Support receiving PDSCH with interleaved VRB-to-PRB mapping |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_interleavingVRB_ToPRB_PDSCH |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | Support dynamic EN-DC power sharing for at least one EN-DC band combination_FR1 only |  |  |  |  |  |  | 38.306, 4.2.7.9 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_dynamicPowerSharingENDC |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  | Supports up to 10 search spaces in a SCell per BWP |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberSearchSpaces |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_spatialBundlingHARQ_ACK |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  | Support alternative additional DMRS position for co-existence with LTE CRS |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_additionalDMRS_DL_Alt |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pusch_RepetitionMultiSlots |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  | Support beam correspondence without UL beam sweeping |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_beamCorrespondenceWithoutUL_BeamSweeping |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |  |  |  |  |  |  |
| 22A |  |  |  |  |  |  | Support beam correspondence based on SSB |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_beamCorrespondence_SSBbased |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22B |  |  |  |  |  |  | Support beam correspondence based on CSI-RS |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_beamCorrespondence_CSI_RSbased |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |  |  |  |  |  | 38.306, 4.2.7.6 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberMIMO_LayersPDSCH_eightLayers |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23A or 23B set to true. |  |  |  |  |  |  |
| 23A |  |  |  |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 4 Layers. |  |  |  |  |  |  | 38.306, 4.2.7.6 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberMIMO_LayersPDSCH_fourLayers |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23 or 23B set to true. |  |  |  |  |  |  |
| 23B |  |  |  |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 2 Layers. |  |  |  |  |  |  | 38.306, 4.2.7.6 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberMIMO_LayersPDSCH_twoLayers |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23 or 23A set to true. |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  | Supports DCI and timer based active BWP switching delay type1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SwitchingDelay_Type1 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |  |  |  |  |
| 24A |  |  |  |  |  |  | Supports DCI and timer based active BWP switching delay type2 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SwitchingDelay_Type2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |  |  |  |  |
| 24B |  |  |  |  |  |  | Supports incremental delay for DCI and timer based active BWP switching type 1 on multiple CCs simultaneously |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_bwp_SwitchingMultiCCs_type1_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |  |  |  |  |  |  |
| 24C |  |  |  |  |  |  | Supports incremental delay for DCI and timer based active BWP switching type 2 on multiple CCs simultaneously |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_bwp_SwitchingMultiCCs_type2_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |  |  |  |  |  |  |
| 25A |  |  |  |  |  |  | Support modified MPR behaviour bit 0 |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_modifiedMPR_behaviour_bit0 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Applicable to FR2 bands n257, n258, n260 and n261 |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  | Support modified MPR behaviour |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_modifiedMPR_behaviour |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  | Support almost contiguous UL CP-OFDM transmissions in FR1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_almostContiguousCP_OFDM_UL_FR1 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  | Support almost contiguous UL CP-OFDM transmissions in FR2 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_almostContiguousCP_OFDM_UL_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_crossSlotScheduling |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pusch_halfpiBPSK_FR1 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31a |  |  |  |  |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pusch_halfpiBPSK_FR2 |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency |  |  |  |  |  |  | 38.306, 4.2.7.6 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_multiDCI_MultiTRP_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_rateMatchingLTE_CRS |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  | Support of BWP operation without bandwidth restriction |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_WithoutRestriction |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  | Support of receiving SCell dormancy indication on SPCell using DCI format 2_6 outside the active time |  |  |  |  |  |  | 38.306, 4.2.7.4 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_scellDormancyOutsideActiveTime_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  | Supports pi/2-BPSK modulation scheme for power boosting in FR1 |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_powerBoosting_pi2BPSK |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  | Support of dynamic 1Tx-2Tx UL Tx switching |  |  |  |  |  |  | 38.306, 4.2.7.1 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_ULTxSwitchingBandPair |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_mpr_PowerBoost_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  | Supports the alternative 64QAM MCS table for PDSCH |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_dl_64qam_mcs_tableAlt |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  | Supports the CQI table with target BLER of 10^-5 |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_cqi_tableAlt |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  | Supports of single DCI based spatial division multiplexing scheme |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_singledci_sdm |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 FDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto2_FR1_FDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 FDD bands |  |  |  |  |  |  |
| 42a |  |  |  |  |  |  | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 TDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto2_FR1_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 TDD bands |  |  |  |  |  |  |
| 42b |  |  |  |  |  |  | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR2 bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto2_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR2 bands |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 FDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto4_FR1_FDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 FDD bands |  |  |  |  |  |  |
| 43a |  |  |  |  |  |  | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 TDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto4_FR1_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 TDD bands |  |  |  |  |  |  |
| 43b |  |  |  |  |  |  | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR2 bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_SameNumerology_upto4_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR2 bands |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 FDD bands |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_DiffNumerology_FR1_FDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 FDD bands |  |  |  |  |  |  |
| 44a |  |  |  |  |  |  | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 TDD bands |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_DiffNumerology_FR1_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 TDD bands |  |  |  |  |  |  |
| 44b |  |  |  |  |  |  | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR2 bands |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_bwp_DiffNumerology_FR2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR2 bands |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  | Support PUSCH repetition type B |  |  |  |  |  |  | 38.306, 4.2.7.7 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_pusch_RepetitionTypeB_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  | Support of 2-Step RACH |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_twoStepRACH_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  | Support monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_dci_Format1_2And0_2_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  | Support of multi-DCI based multi-TRP |  |  |  |  |  |  | 38.306, 4.2.7.6 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_multi_dci_multi_trp |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  | Support of single DCI based FDMSchemeA |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_single_dci_fdmschemeA |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55 |  |  |  |  |  |  | Support of single-DCI based inter-slot TDM |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_single_dci_interslot_tdm |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  | Support of maximum number of TRS resource sets per CC which the UE can track simultaneously is at least 2 |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_simultaneous_TRS |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 57 |  |  |  |  |  |  | Support of low PAPR DMRS |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_lowPAPR_DMRS_pusch_precoding |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 58 |  |  |  |  |  |  | Support of UL full power transmission mode of full power |  |  |  |  |  |  | 38.306, 4.2.7.7 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_ul_FullPwrMode_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 59 |  |  |  |  |  |  | Support of UL full power transmission mode of fullpowerMode1 |  |  |  |  |  |  | 38.306, 4.2.7.7 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_ul_FullPwrMode1_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  | Support of UL full power transmission mode of fullpowerMode2 |  |  |  |  |  |  | 38.306, 4.2.7.7 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_ul_FullPwrMode2_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 61 |  |  |  |  |  |  | Support of PDSCH processing capability 2 |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_pdsch_processing_cap2 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62 |  |  |  |  |  |  | Support Pre-Emption Indication |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_preEmptIndication_DL |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 63 |  |  |  |  |  |  | Support of SSB based BFD |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberSSB_BFD |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 64 |  |  |  |  |  |  | Support of CSI-RS based BFD |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberCSI_RS_BFD |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  |  |  | Support of SSB and/or CSI-RS based Link Recovery |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberCSI_RS_SSB_CBD |  |  |  |  |  |  | CY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  |  |  |  |  |  | Support of type II codebook |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_typeIICodebook |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 67 |  |  |  |  |  |  | Support of Enhanced Type II codebook with at least 16 ports per CSI-RS resource |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_enhanced_typeII_codebook |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  |  |  | Support of TDD NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  |  |  |  |  | 38.101-1, 5.4.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_frequencyShift7p5khz_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory since Rel-16 |  |  |  |  |  |  |  |
| 69 |  |  |  |  |  |  | Support of FDD NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  |  |  |  |  | 38.101-1, 5.4.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_frequencyShift7p5khz_FDD |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 71 |  |  |  |  |  |  | Support of density of CSI-RS for Channel Measurement Report |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_supportedCSI_RS_Density_CMR |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 72 |  |  |  |  |  |  | Support of SSB/CSI-RS for L1-SINR measurement |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_ssb_csirs_SINR_measurement |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  |  |  |  |  |  | Support of SSB as CMR with dedicated CSI-IM for L1-SINR measurement |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_supportedSINR_meas_ssbWithCSI_IM |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74 |  |  |  |  |  |  | Support of SSB as CMR with dedicated NZP IMR for L1-SINR measurement |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_supportedSINR_meas_ssbWithNZP_IMR |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 75 |  |  |  |  |  |  | Support of CSI-RS as CMR with dedicated NZP IMR configured for L1-SINR measurement |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_supportedSINR_meas_csirsWithNZP_IMR |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 76 |  |  |  |  |  |  | Support of CSI-RS as CMR without dedicated IMR configured for L1-SINR measurement |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_supportedSINR_meas_csi_RSWithoutIMR |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 77 |  |  |  |  |  |  | Support of SCell beam failure recovery |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | Pc_scellBFR |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 78 |  |  |  |  |  |  | Support of the maximum number of activated TCI states per BWP per CC is other than n1, including control and data |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_maxNumberActiveTCI_PerBWP |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  | Support enhanced UL performance for the transient period |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_enhancedUL_TransientPeriod_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  | Supports the priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_dci_DL_PriorityIndicator_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 81 |  |  |  |  |  |  | Supports the priority indicator field configured in DCI formats 0_1 and 0_2 in a BWP when configured to monitor both DCI formats 0_1 and 0_2 in the BWP |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_dci_UL_PriorityIndicator_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | A UE supporting this feature shall also support ul-IntraUE-Mux-r16 and dci-Format1-2And0-2-r16 |  |  |  |  |  |  |  |
| 82 |  |  |  |  |  |  | Supports restricting data transmission from a given LCH to a configured (sub-) set of dynamic grant priority levels |  |  |  |  |  |  | 38.306, 4.2.6 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_lch_ToGrantPriorityRestriction_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 83 |  |  |  |  |  |  | Supports two PUCCH group in CA with a same numerology across CCs for data and control channel. |  |  |  |  |  |  | 38.306 4.2.7.7 |  |  |  |  |  |  | Rel15 |  |  |  |  |  |  | pc_twoPUCCH_group |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 84 |  |  |  |  |  |  | Support of transparent Tx diversity requirements for at least one NR FR1 band |  |  |  |  |  |  | 38.306, 4.2.7.2 38.331, Annex C |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_txDiversity_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 only This capability has been introduced in Rel-16 and is early implementable from Rel-15 onwards. |  |  |  |  |  |  |  |
| 85 |  |  |  |  |  |  | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pusch_RepetitionCRC_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86 |  |  |  |  |  |  | Support of DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  |  |  |  |  |  | Support of CQI reporting with 4 bits per subband for NTN and shared spectrum channel access |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_cqi_4_BitsSubbandNTN_SharedSpectrumChAccess_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports CQI reporting with 4 bits per subband for NTN and shared spectrum channel access |  |  |  |  |  |  |  |
| 88 |  |  |  |  |  |  | Support of propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_ta_BasedPDC_NTN_SharedSpectrumChAccess_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access |  |  |  |  |  |  |  |
| 89 |  |  |  |  |  |  | Support of 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n8 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |  |  |  |  |  |  |  |
| 90 |  |  |  |  |  |  | Support of 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |  |  |  |  |  |  |  |
| 91 |  |  |  |  |  |  | Support of NTN features in GSO scenario |  |  |  |  |  |  | 38.306, 4.2.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_ntn_ScenarioSupport_r17_GSO |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports NTN features in GSO scenario |  |  |  |  |  |  |  |
| 92 |  |  |  |  |  |  | Support of NTN features in NGSO scenario |  |  |  |  |  |  | 38.306, 4.2.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_ ntn_ScenarioSupport_r17_NGSO |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | UE supports NTN features in NGSO scenario |  |  |  |  |  |  |  |
| 93 |  |  |  |  |  |  | Support of RSSI measurements and channel occupancy reporting |  |  |  |  |  |  | 38.306, 4.2.7.2a |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_rssi_ChannelOccupancyReporting_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 94 |  |  |  |  |  |  | Supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 |  |  |  |  |  |  | 38.306, 4.2.7.11 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_extendedBand_n77_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Applicable for UE support band n77 and in the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. |  |  |  |  |  |  |  |
| 95 |  |  |  |  |  |  | Supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 |  |  |  |  |  |  | 38.306, 4.2.7.11 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_extendedBand_n77_2_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | Applicable for UE support band n77 and in Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. |  |  |  |  |  |  |  |
| 96 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 97 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 99 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 101 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 102 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 103 |  |  |  |  |  |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104 |  |  |  |  |  |  | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 FDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pdsch_1024QAM_2MIMO_FR1_r17_FDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 FDD bands |  |  |  |  |  |  |  |
| 105 |  |  |  |  |  |  | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 TDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pdsch_1024QAM_2MIMO_FR1_r17_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 TDD bands |  |  |  |  |  |  |  |
| 106 |  |  |  |  |  |  | Support of 1024QAM modulation scheme for PDSCH for FR1_FDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pdsch_1024QAM_FR1_r17_FDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 FDD bands |  |  |  |  |  |  |  |
| 107 |  |  |  |  |  |  | Support of 1024QAM modulation scheme for PDSCH for FR1_TDD bands |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pdsch_1024QAM_FR1_r17_TDD |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | FR1 TDD bands |  |  |  |  |  |  |  |
| 108 |  |  |  |  |  |  | Support RTT-based propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS. |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_rtt_BasedPDC_CSI_RS_ForTracking_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | A UE supporting this feature shall also indicate support of csi-RS-ForTracking and supportedSRS-Resources as specified in TS 38.331. |  |  |  |  |  |  |  |
| 109 |  |  |  |  |  |  | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS. |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_rtt_BasedPDC_PRS_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | If UE provides parameter maxNumberPRS-Resource-r17 and optionally parameter maxNumberPRS-ResourceProcessedPerSlot-r17 as described in TS 38.331, consider this as supported, otherwise not supported. A UE supporting this feature shall also indicate support of supportedSRS-Resources as specified in TS 38.331. |  |  |  |  |  |  |  |
| 110 |  |  |  |  |  |  | Support propagation delay compensation based on legacy TA procedure for TN and non-shared spectrum channel access. |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_ta_BasedPDC_TN_NSSChAccess_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 |  |  |  |  |  |  | Support Type 1 PUSCH transmissions with configured grant |  |  |  |  |  |  | 38.306 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_type1_PUSCH_RepetitionMultiSlots |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 112 |  |  |  |  |  |  | Support Type 2 PUSCH transmissions with configured grant |  |  |  |  |  |  | 38.306 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_type2_PUSCH_RepetitionMultiSlots |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 113 |  |  |  |  |  |  | Support the dynamic indication of the number of repetitions for PUSCH transmission |  |  |  |  |  |  | 38.306 4.2.7.10 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_pusch_RepetitionTypeA_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 114 |  |  |  |  |  |  | Support repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication |  |  |  |  |  |  | 38.306 4.2.7.7 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_pucch_Repetition_F0_1_2_3_4_DynamicIndication_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 115 |  |  |  |  |  |  | Support the increased maximum number of PUSCH Type A repetitions up to 32 |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_maxNumberPUSCH_TypeA_Repetition_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 116 |  |  |  |  |  |  | Support PUSCH repetitions based on available slots |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_puschTypeA_RepetitionsAvailSlot_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 117 |  |  |  |  |  |  | Supports TB processing over multi-slot PUSCH without repetition |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_tb_ProcessingMultiSlotPUSCH_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 118 |  |  |  |  |  |  | Supports repetition of TB processing over multi-slot PUSCH |  |  |  |  |  |  | 38.306 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_tb_ProcessingRepMultiSlotPUSCH_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 119 |  |  |  |  |  |  | Support SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_sfn_schemeA_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 120 |  |  |  |  |  |  | Support SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_sfn_schemeB_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 121 |  |  |  |  |  |  | Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-16 |  |  |  |  |  |  | pc_multi_sps_r16 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | The UE can include this feature only if the UE indicates support of downlinkSPS. |  |  |  |  |  |  |  |
| 122 |  |  |  |  |  |  | Support transmission of two PUCCH formats in TDM in the same slot |  |  |  |  |  |  | 38.306, 4.2.7.10 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_twoPUCCH_AnyOthersInSlot |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 123 |  |  |  |  |  |  | Support of unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_unifiedJointTCI_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 124 |  |  |  |  |  |  | Support of unified TCI state operation with separate DL/UL TCI update for intra-cell beam management |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_unifiedSeparateTCI_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 125 |  |  |  |  |  |  | Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_mTRP_InterCell_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 126 |  |  |  |  |  |  | Support of unified TCI with separate DL/UL TCI update for inter-cell beam management |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_unifiedSeparateTCI_InterCell_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 127 |  |  |  |  |  |  | Support of dynamic 2Tx-2Tx UL Tx switching |  |  |  |  |  |  | 38.306, 4.2.7.1 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_2Tx_ULTxSwitchingBandPair_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |  |  |  |  |  |  |  |
| 128 |  |  |  |  |  |  | Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. |  |  |  |  |  |  | 38.306, 4.2.7.2 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_aperiodicCSIRS_AdditionalBandwidth_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 129 |  |  |  |  |  |  | Indicates whether the UE supports conditional PSCell addition in EN-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in EN-DC. |  |  |  |  |  |  | 38.306, 4.2.7.9 |  |  |  |  |  |  | Rel-17 |  |  |  |  |  |  | pc_condPSCellAdditionENDC_r17 |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 130 |  |  |  |  |  |  | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot (pdcch-MonitoringAnyOccasions) |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdcchMonitoringAnyOccasions |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 131 |  |  |  |  |  |  | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot with minimum time separation between two consecutive transmissions of PDCCH with span (pdcch-MonitoringAnyOccasionsWithSpanGap) |  |  |  |  |  |  | 38.306, 4.2.7.5 |  |  |  |  |  |  | Rel-15 |  |  |  |  |  |  | pc_pdcchMonitoringAnyOccasions_SpanGap |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of Changes >>