3GPP TSG RAN WG5 Meeting #101 	R5-237209r1

Chicago, USA, 13 – 17 November 2023

Source:		Bluetest

Title:	Updates to RC MU Analysis for NR FR1 TRP-TRS Enhancement

Agenda Item:		5.3.22.2 Discussion Papers (Measurement Uncertainty (MU) assessment proposals for TR 38.870)

Document for:	Discussion and Endorsement

# Introduction

## Continuing the work on the RAN5 MU Work Plan for NR FR1 TRP-TRS Enhancement (Rel-18) [1] and the earlier document R5-235795 from 3GPP TSG RAN WG5 Meeting #100 [2], this document updates the MU table to reflect recent changes elsewhere in appendix B TR 38.870 [4].

## Approach

## Changes in this text reflect the latest work on the MU for the reference method of both TRP and TRS measurement uncertainty.

## In this draft, we align the definitions of MU from the AC approach from [4] with those from the RC approach.

Proposal 1: Endorse text updates to RC MU in 38.870

The MU definition in B.2.6 Sensitivity measurement: output level step resolution was changed in [4] to be specific to the AC implementation. Here we add an RC version of the definition as B.3.4 using the earlier pre 0.6.0 text which covers the alternate method.

Both TRP and TRS update the term for input mismatch based on B.2.1. We propose to follow the values for the AC method since they provide broad coverage over both instrument and system implementation. Additional RC specific analysis is recommended to add insight relative to this MU value and would require less work to apply as B.2.1 does now.

Proposal 2: Consider an optimized analysis of B.2.1 for the RC method to simplify calculation of MU term at next meeting.

The MU description in B.3.1 “Additional Power Loss in EUT Chassis” was written to align with previous 3GPP studies and CTIA which considered a broad set of device types. The recommended fixed value is high to accommodate even large IoT appliances. An alternative fixed value for handset devices should be considered which would reduce this default term to less than 0.1 dB.

Proposal 3: Consider a second fixed value term for handset devices in B.3.1

## Summary of RC Specific MU Changes

## Recommended changes to TRP (B.4.2)

UID 1: Tracks mismatch similar to AC analysis (see note above)

UID 2: While not required, a measured value is more typically used with the RC method for the cable loss to the receiver.

UID 4: Align MU values with AC to use comms tester or spectrum analyzer instead of power meter.

UID 5: Use updated term for distribution description. No change in value.

UID 6: Fix typo in value to match description in B.3.1

UID 8: Phantom uncertainty added. Same values and analysis as AC.

UID 9: Fix labeling and use TRP value from B.2.13

UID 10: Update distribution term to Normal. Value updated with divisor term.

UID 14: Since we use the efficiency method, this term goes to 0 per B.2.3

UID 16: Update distribution term to Normal. Value updated with divisor term.

UID 17: Fix typos in table, final value stays the same.

## Recommended changes to TRS (B.5.2)

Table B.5.2-1 required several edits to fix UID numbering, order, and remove obsolete (0 valued) MU terms “Statistical uncertainty of BER terms” and “BER Data rate normalizations”

UID 1: Same change procedure as described above for TRP on mismatch uncertainty

UID 4: Update with latest term for communications tester [5]

UID 5: Update output step resolution term per B.3.4

UID 6: Update description of distribution, no value change

UID 7: Match value specified in B.3.1

UID 8: Remove square brackets

UID 9: Add phantom uncertainty consistent with AC analysis

UID 10: Update distribution and remove square brackets

UID 11: Update distribution and remove square brackets

UID 14: Update distribution description, no value change

UID 15: Remove square brackets

UID 17: Update distribution and align with similar AC value.

UID 18: Update description and distribution naming, no value change, remove square brackets.

# References

- R5-225924, “MU workplan for NR FR1 TRP-TRS Enhancement (Rel-18)”, Rhode & Schwarz, vivo, 3GPP TSG RAN5 Meeting #97-e, November 2022

- R5-235795, “RC MU Analysis for NR FR1 TRP-TRS Enhancement (Rel-18)”, Bluetest AB, Electronic meeting, 22-26 August 2023.

- 3GPP TR 38.810 V16.6.0 (2020-10)

- 3GPP TR 38.870 V0.6.0 (2023-10)

- R5-237152, “AC MU Analysis for NR FR1 TRP-TRS (Rel.17)”

# Conclusion

The text submission should be considered for the development of the MU sections in TR 37.870

Proposal 1: Endorse the text proposal below

Proposal 2: Consider an optimized analysis of B.2.1 for the RC method to simplify calculation of MU term at next meeting.

Proposal 3: Consider a second fixed value term for handset devices in B.3.1

# Appendix - Text Proposal to TR 38.870

< Unchanged Text Deleted >

< Beginning of Changes >

## B.3.4	Sensitivity measurement: output level step resolution

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, used power step resolution creates this uncertainty.  Output power step used in the sensitivity measurement is divided by factor 2 and then a rectangular distribution applied to obtain the uncertainty.

< Unchanged Text Deleted >

< Beginning of Changes >

### B.4.2	MU Assessment for TRP in Reverberation Chamber

Table B.4.2-1 Uncertainty contributions in TRP measurement for reverberation chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Mismatch of receiver chain | B.2.1 |
| 2 | Insertion loss of receiver chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | Measurement Receiver: uncertainty of the absolute level | B.2.4 |
| 5 | Quality of Spatial Uniformity | B.3.3 |
| 6 | Additional power loss in EUT chassis | B.3.1 |
| 7 | DUT Tx-power drift | B.2.9 |
| 8 | Uncertainty related to the use of phantoms | B.2.11 |
| 9 | Random uncertainty | B.2.13 |
| Stage 1: Calibration measurement |  |  |
| 10 | Uncertainty of network analyzer | B.2.15 |
| 11 | Mismatch of receiver chain | B.2.1 |
| 12 | Insertion loss of receiver chain | B.2.2 |
| 13 | Mismatch in the connection of calibration antenna | B.2.1 |
| 14 | Influence of the calibration antenna feed cable | B.2.3 |
| 15 | Influence of the measurement antenna cable | B.2.3 |
| 16 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | B.2.16 |
| 17 | Uniformity of transfer function | B.3.3 |

Table B.4.2-2 Preliminary example of uncertainty budget for TRP measurement hand only (browsing mode) for reverberation chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] | Prob Distr | Div | ci | Standard Uncertainty [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гreceiver <0.33 Г measurement antenna connection <0.5 Cable loss > 3dB | 0.26 | U-shaped | 1.41 | 1 | 0.18 |
| 2 | Insertion loss of receiver chain | Typically Measured | 0.1 | Rectangular | 1.73 | 1 | 0.06 |
| 3 | Influence of the fixed measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 4 | Measurement Receiver: uncertainty of the absolute level | From datasheet of communication tester or Spectrum Analyzer | 0.54 | Normal | 2 | 1 | 0.27 |
| 5 | Quality of Spatial Uniformity | Statistics of chamber | 0.5 | Actual | 1 | 1 | 0.5 |
| 6 | Additional power loss in EUT chassis | The EUT not present in the chamber during calibration measurement | 2 | Rectangular | 1.73 | 1 | [0.12] |
| 7 | DUT Tx-power drift | Drift | 0.2 | Rectangular | 1.73 | 1 | 0.12 |
| 8 | Uncertainty related to the use of phantoms | Material and Geometry | 0.64 | Rectangular | 1.73 | 1 | 0.37 |
| 9 | Random uncertainty | Using the same setup and stirring sequence | 25 | Rectangular | 1.73 | 1 | [0.14] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| 10 | Uncertainty of network analyzer | Manufacturer’s uncertainty calculator, covers whole NA setup | 0.5 | Normal | 2 | 1 | [0.25] |
| 11 | Mismatch of receiver chain | Taken in to account in NA setup uncertainty | 0 | U-shaped | 1.41 | 1 | 0.00 |
| 12 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 13 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | U-shaped | 1.41 | 1 | 0.00 |
| 14 | Influence of the feed cable of the calibration antenna | Efficiency calibration used | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 15 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 16 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration certificate | 0.58 | Normal | 2 | 1 | [0.29] |
| 17 | Uniformity of transfer function | Statistics of chamber | 0.5 | Actual | 1 | 1 | [0.5] |
| Combined standard uncertainty |  |  |  |  |  |  | [0.97] |
| Expanded uncertainty (Confidence interval of 95 %) |  |  |  |  |  |  | [1.90] |

Table B.4.2-2 Preliminary example of uncertainty budget for TRP Beside Head and Hand (Talk mode) measurement for reverberation chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] | Prob Distr | Div | ci | Standard Uncertainty [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гreceiver <0.33 Г measurement antenna<0.5 | 0.26 | U-shaped | 1.41 | 1 | 0.18 |
| 2 | Insertion loss of receiver chain | Typically measured | 0.1 | Rectangular | 1.73 | 1 | 0.06 |
| 3 | Influence of the fixed measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 4 | Measurement Receiver: uncertainty of the absolute level | Spectrum Analyzer | 0.54 | Normal | 2 | 1 | 0.27 |
| 5 | Quality of Spatial Uniformity | Statistics of chamber | 0.5 | Actual | 1 | 1 | [0.5] |
| 6 | Additional power loss in EUT chassis | The EUT not present in the chamber during calibration measurement | 0.2 | Rectangular | 1.73 | 1 | [0.12] |
| 7 | DUT Tx-power drift | Drift | 0.2 | Rectangular | 1.73 | 1 | 0.12 |
| 8 | Uncertainty related to the use of phantoms | Material and Geometry, Beside Head/Hand | 0.99 | Rectangular | 1.73 | 1 | 0.57 |
| 9 | Random Uncertainty | Using the same setup and stirring sequence | 0.25 | Rectangular | 1.73 | 1 | [0.14] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| 10 | Uncertainty of network analyzer | Manufacturer’s uncertainty calculator, covers whole NA setup | 0.5 | Normal | 2 | 1 | 0.25 |
| 11 | Mismatch of receiver chain | Taken in to account in NA setup uncertainty | 0 | U-shaped | 1.41 | 1 | 0.00 |
| 12 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 13 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | U-shaped | 1.41 | 1 | 0.00 |
| 14 | Influence of the feed cable of the calibration antenna | Efficiency calibration used | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 15 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | Rectangular | 1.73 | 1 | 0.00 |
| 16 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration certificate | 0.58 | Normal | 2 | 1 | [0.29] |
| 17 | Uniformity of transfer function | Statistics of chamber | 0.5 | Actual | 1 | 1 | [0.5] |
| Combined standard uncertainty |  |  |  |  |  |  | [1.06] |
| Expanded uncertainty (Confidence interval of 95 %) |  |  |  |  |  |  | [2.08] |

# B.5	MU Assessment for TRS

## B.5.1	MU Assessment for TRS in Anechoic Chamber

< Unchanged Text Deleted >

< Beginning of Changes >

## B.5.2	MU Assessment for TRS in Reverberation Chamber

Table B.5.2-1 Uncertainty contributions in TRS measurement for reverberation chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Mismatch of transmitter chain | B.2.1 |
| 2 | Insertion loss of transmitter chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | Communication Tester: uncertainty of the absolute output level | B.2.5 |
| 5 | Sensitivity measurement: output level step resolution | B.3.4 |
| 6 | Quality of Spatial Uniformity | B.3.3 |
| 7 | Additional power loss in EUT chassis | B.3.1 |
| 8 | DUT sensitivity drift | B.2.10 |
| 9 | Uncertainty related to the use of phantoms | B.2.11 |
| 10 | Random uncertainty | B.2.13 |
| Stage 1: Calibration measurement |  |  |
| 11 | Uncertainty of network analyzer | B.2.15 |
| 12 | Mismatch of transmitter chain | B.2.1 |
| 13 | Insertion loss of transmitter chain | B.2.2 |
| 14 | Mismatch in the connection of calibration antenna | B.2.1 |
| 15 | Influence of the calibration antenna feed cable | B.2.3 |
| 16 | Influence of the measurement antenna cable | B.2.3 |
| 17 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | B.2.16 |
| 18 | Uniformity of transfer function | B.3.3 |

Table B.5.2-2: Preliminary example of uncertainty budget for TRS hand only (browsing mode) measurement for reverberation chamber method for NR FR1 bands

| UID | Uncertainty Source |  | Comment |  | Uncertainty Value [dB] | Prob Distr | Div | ci | Standard Uncertainty [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Mismatch of transmitter chain |  | Гcomm tester <0.29 Г meas antenna<0.5 Cable Loss > 3db |  | 0.22 | U-shaped | 1.41 | 1 | 0.16 |
| 2 | Insertion loss of transmitter chain |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 3 | Influence of the fixed measurement antenna cable |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 4 | Communication tester: uncertainty of the absolute level |  | From datasheet of communication tester. |  | 1.3 | Normal | 2 | 1 | 0.65 |
| 5 | Sensitivity measurement: output level step resolution |  | Step 0.5dB |  | 0.25 | Rectangular | 1.73 | 1 | 0.14 |
| 6 | Quality of Spatial Uniformity | Statistics of chamber | 0.5 | Actual | 1 | 1 | 0.5 |
| 7 | Additional power loss in EUT chassis |  | The EUT not present in the chamber during calibration measurement |  | 0.2 | Rectangular | 1.73 | 1 | [0.12] |
| 8 | DUT sensitivity drift |  | Drift measurement |  | 0.2 | Rectangular | 1.73 | 1 | 0.12 |
| 9 | Uncertainty related to the use of phantoms |  | Material and Geometry for Data Mode Fixture |  | 0.64 | Rectangular | 1.73 | 1 | 0.37 |
| 10 | Random Uncertainty |  | Using the same setup and stirring sequence |  | 0.4 | Normal | 2 | 1 | 0.2 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| 11 | Uncertainty of network analyzer |  | Manufacturer’s uncertainty calculator, covers NA setup |  | 0.5 | Normal | 2 | 1 | 0.25 |
| 12 | Mismatch of transmitter chain |  | Taken in to account in NA setup uncertainty |  | 0 | U-shaped | 1.41 | 1 | 0 |
| 13 | Insertion loss of transmitter chain |  | Systematic with Stage 2 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 14 | Mismatch in the connection of calibration antenna |  | Taken in to account in NA setup uncertainty |  | 0 | U-shaped | 1.73 | 1 | 0 |
| 15 | Influence of the feed cable of the calibration antenna |  | Gain calibration with dipole |  | 0.3 | Rectangular | 1.73 | 1 | 0.17 |
| 16 | Influence of the fixed measurement antenna cable |  | Systematic with Stage 2 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 17 | Uncertainty of the absolute gain of the calibration antenna |  | Calibration certificate |  | 0.58 | Normal | 2 | 1 | [0.29] |
| 18 | Uniformity of transfer function |  | Statistics of chamber |  | 0.5 | Actual | 1 | 1 | 0.5 |
| Combined standard uncertainty |  |  |  |  |  |  |  |  | [1.16] |
| Expanded uncertainty (Confidence interval of 95 %) |  |  |  |  |  |  |  |  | [2.28] |

Table B.5.2-2: Preliminary example of uncertainty budget for TRS Beside Head and Hand (Talk mode) measurement for reverberation chamber method for NR FR1 bands

| UID | Uncertainty Source |  | Comment |  | Uncertainty Value [dB] | Prob Distr | Div | ci | Standard Uncertainty [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Mismatch of transmitter chain |  | Гcomm tester <0.29 Г meas antenna <0.5 Cable loss > 3dB |  | 0.22 | U-shaped | 1.41 | 1 | 0.16 |
| 2 | Insertion loss of transmitter chain |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 3 | Influence of the fixed measurement antenna cable |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 4 | Communication tester: uncertainty of the absolute level |  | Manufacturer’s data sheet |  | 1.3 | Normal | 2 | 1 | 0.65 |
| 5 | Sens. measurement: output level step resolution |  | Step 0.5dB |  | 0.25 | Rectangular | 1.73 | 1 | 0.14 |
| 6 | Quality of Spatial Uniformity | Statistics of chamber | 0.5 | Actual | 1 | 1 | 0.5 |
| 7 | Additional power loss in EUT chassis |  | The EUT not present in the chamber during calibration measurement |  | 0.2 | Rectangular | 1.73 | 1 | [0.12] |
| 8 | DUT sensitivity drift |  | Drift measurement |  | 0.2 | Rectangular | 1.73 | 1 | 0.12 |
| 9 | Uncertainty related to the use of phantoms |  | Material and Geometry for Data Mode Fixture |  | 0.99 | Rectangular | 1.73 | 1 | 0.57 |
| 10 | Random Uncertainty |  | Using the same setup and stirring sequence |  | 0.4 | Normal | 2 | 1 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| 11 | Uncertainty of network analyzer |  | Manufacturer’s uncertainty calculator, covers NA setup |  | 0.5 | Normal | 2 | 1 | 0.25 |
| 12 | Mismatch of transmitter chain |  | Taken in to account in NA setup uncertainty |  | 0 | U-shaped | 1.41 | 1 | 0 |
| 13 | Insertion loss of transmitter chain |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 14 | Mismatch in the connection of calibration antenna |  | Taken in to account in NA setup uncertainty |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 15 | Influence of the feed cable of the calibration antenna |  | Gain calibration with dipole |  | 0.3 | Rectangular | 1.73 | 1 | 0.17 |
| 16 | Influence of the fixed measurement antenna cable |  | Systematic with Stage 1 (=> cancels) |  | 0 | Rectangular | 1.73 | 1 | 0 |
| 17 | Uncertainty of the absolute gain of the calibration antenna |  | Calibration certificate |  | [0.58] | Normal | 2 | 1 | [0.29] |
| 18 | Quality of Spatial Uniformity |  | Statistics of chamber |  | 0.5 | Actual | 1 | 1 | 0.5 |
| Combined standard uncertainty |  |  |  |  |  |  |  |  | [1.1.24] |
| Expanded uncertainty (Confidence interval of 95 %) |  |  |  |  |  |  |  |  | [2.43] |

< End of Changes >