3GPP TSG-RAN5 Meeting #101	R5-237214r1

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0778 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new PDSCH Reference Channel TDD to cover FR2 PDCCH Test 2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 7.3.2.2.2 test 2-2 configuration requires 2 symbols duration and specific PDSCH TDRA configuration. However, current additional PDSCH Reference Channel Table A.3.3.2.5-3 does not consider 7.3.2.2.2 test 2-2 configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new DCI_1_0 for test 2-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.3.2.2.2 test 2-2 will not be aligned with Table A.3.3.2.5-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 updates Binary Channel Bits Per Slot and Max. Throughput averaged over 2 frames in new DCI_1_0 Test 2-2 column |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### A.3.3.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.3.2.5-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-1.1 TDD | R.PDCCH.5-1.2 TDD | R.PDCCH.5-1.3 TDD | R.PDCCH.5-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |  |
| CORESET frequency domain allocation |  | 60 | 60 | 60 | 60 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 40 | 56 | 56 | 12 |  |  |

Table A.3.3.2.5-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

Table A.3.3.2.5-3: Additional PDSCH Reference Channel TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| DCI format |  | DCI 1_0 | DCI 1_1 | DCI 1_0 Test 2-2 |
| TDD UL/DL pattern |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |
| Number of consecutive PDSCH symbols |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 8 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 13 | 13 | 12 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 13 | 13 | 12 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 | 1 |
| Number of DMRS REs |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 12 | 12 | 12 |
| Overhead for TBS determination |  | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3104 | 3624 | 2152 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 4480 | 5504 | 4032 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 4480 | 5504 | 4032 |
| Transport block CRC per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 | 16 | 16 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 1 | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 10296 | 11880 | 7128 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 15048 | 18216 | 13464 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 15048 | 18216 | 13464 |
| Max. Throughput averaged over 2 frames | Mbps | 26,022 | 31.667 | 22,796 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames Note 3: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. |  |  |  |  |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>