3GPP TSG RAN5 Meeting #101	 	R5-237254r1

Chicago, USA, 13th – 17th November 2023

Source:		Rohde & Schwarz

Title:	On the MU for n259

Agenda Item:		5.4.17

Document for:	Discussion and Endorsement

# Introduction

In previous meetings, the MU for FR2c has been discussed [1, 2, 3, 4]. However, values for MU contributors uncertainty of the RF power measurement equipment and gNB emulator uncertainty have not been finalized. We continue the discussion with this contribution.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## Uncertainty of the RF power measurement equipment

Based on our assessment, we propose a value of 3.6 dB as default for FR2c.

Proposal 1: For TCs other than the spurious emissions TCs, apply a value of 3.6 dB for the uncertainty of the RF power measurement equipment in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3.

## gNB emulator uncertainty

Based on our assessment, we propose a value of 3.6 dB as default for FR2c..

Proposal 2: For TCs other than the spurious emissions TCs, apply a value of 3.6 dB for the gNB emulator uncertainty in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2c |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the uncertainty of the RF power measurement equipment in the frequency range of FR2c and make the following proposals:

Proposal 1: For TCs other than the spurious emissions TCs, apply a value of 3.6 dB for the uncertainty of the RF power measurement equipment in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3.

Proposal 2: For TCs other than the spurious emissions TCs, apply a value of 3.6 dB for the gNB emulator uncertainty in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3.

# References

- R5-233174, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-232983, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-235140, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #100, August 2023

- R5-235753, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #100, August 2023