3GPP TSG RAN5 Meeting #101	 	R5-237255r1

Chicago, USA, 13th – 17th November 2023

Source:		Rohde & Schwarz

Title:	On the MU and TT for FR2 UL MIMO

Agenda Item:		5.4.17

Document for:	Discussion and Endorsement

# Introduction

At RAN5 #92-e, the MU for FR2 UL MIMO MPR, SEM and ACLR TCs has been discussed [2]. However, the MU for several other FR2 UL MIMO TCs has not yet been specified. In order to progress these TCs, we share our assessment of the MU.

# Discussion

First we clarify the OTA test procedure for UL MIMO TCs and afterwards present the assessment of the MU.

## OTA test procedure

Already in the SISO TCs, UEs are allowed to transmit on both polarizations simultaneously as specified in clause 6.1 of TS 38.521-2 [2]:

The UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Observation 1: The UE is allowed to transmit simultaneously on both polarizations in the SISO TCs.

The test procedures specified in Annex K of TS 38.521-2 [2] for EIRP and TRP are based on the measurement of the rms power on the two polarizations of the electromagnetic field.

Observation 2: The measurement procedures for EIRP and TRP in Annex K of TS 38.521-2 [2] are based on both polarizations.

Due to considering the sum power of both polarizations, these procedures can be reused for the UL MIMO TCs which use power measurements.

Proposal 1: For TCs using power measurements reuse the measurement procedures for EIRP and TRP already specified in Annex K of TS 38.521-2 [2].

## Measurement uncertainty

### General

From a power measurement perspective, there is no principal difference between an UE transmitting a single layer on two polarizations or two layers. Since simultaneous transmission of a single layer on two polarizations is allowed, the MU assessment of the single layer TCs has already considered dual polarizations and the associated impacts, e.g., on the influence of noise. Therefore, we propose to reuse most of the MU contributors and their values from the SISO TCs except for influence of noise.

Proposal 2: For UL MIMO TCs based on power measurements reuse the MU contributors and their values from the SISO TCs except for the influence of noise.

The influence of noise and relaxation depend on the power levels applied in the TC and have to be assessed on a TC per TC level.

Proposal 3: Assess the influence of noise and the relaxation on a TC per TC level.

As the MU is the same for the UL MIMO and SISO TCs except possibly the influence of noise and since the latter is considered separately in TT calculation, we propose to apply the same TT calculation for an UL MIMO TC as the one of the corresponding SISO TC.

Proposal 4: Reuse the TT formula from the corresponding SISO TCs for the UL MIMO TCs.

In the following, we analyze the influence of noise for MOP EIRP/TRP, MOP spherical, MinOP and Tx OFF power TCs for PC3.

### MOP EIRP/TRP

The minimum conformance requirement is specified as follows for UL MIMO MOP EIRP/TRP in TS 38.521-1 [2]:

6.2D.1.1.3.1	UE maximum output power for UL MIMO for power class 1

The following requirements define the maximum output power radiated by the PC1 UE. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.1-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Power class 1 UE is used for fixed wireless access (FWA).

Table 6.2D.1.1.3.1-1: UE minimum peak EIRP for UL MIMO for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| n262 | 34.2 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. |  |

Table 6.2D.1.1.3.1-2: (void)

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.1-3 below for UE with UL MIMO. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.1-3: UE maximum output power limits for UL MIMO for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |
| n262 | 35 | 55 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE with UL MIMO is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.1-4 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.1-4: UE spherical coverage for UL MIMO for power class 1

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| n262 | 26.0 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance. |  |

6.2D.1.1.3.3	UE maximum output power for UL MIMO for power class 3

The following requirements define the maximum output power radiated by the PC3 UE.. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.3-1 below. The period of measurement shall be at least one sub frame (1 ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.3-1: UE minimum peak EIRP for UL MIMO for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| n262 | 16.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE 2: Min Peak EIRP refers to the total EIRP for the UL beams peaks. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.3-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.3-2: UE maximum output power limits for UL MIMO for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |

Following excerpt is from TS 38.101-2 V18.3.0 (2023-09) [8]:

#### 6.2D.1.5	UE maximum output power for UL MIMO for power class 5

The following requirements define the maximum output power radiated by the PC4 UE. Requirements apply to UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.5-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Power class 5 UE is used for fixed wireless access (FWA).

Table 6.2D.1.5-1: UE minimum peak EIRP for UL MIMO for power class 5

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| n259 | 27.7 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.5-3 below for UE with UL MIMO. The maximum allowed EIRP is derived from regulatory requirements. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.5-2: UE maximum output power limits for UL MIMO for power class 5

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |

In comparison to SISO requirements the max TRP and max EIRP are identical for PC1, PC3, and PC5. For the Min peak EIRP, UE multi-band relaxation (MBR) factors for PC3 do not apply in the UL MIMO TCs. However, the impact of the MBR on the influence of noise is negligible. Therefore, we propose to reuse the MU and TT of the SISO MOP EIRP/TRP TC for the UL MIMO EIRP/TRP TC.

Proposal 5: Reuse the MU and TT of the SISO MOP EIRP/TRP TC for the UL MIMO EIRP/TRP TC for PC1, PC3 and PC5.

### MOP spherical

The minimum requirement is stated in TS 38.521-2 [2] as follows:

6.2D.1.1.3.1	UE maximum output power for UL MIMO for power class 1

...

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE with UL MIMO is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.1-4 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.1-4: UE spherical coverage for UL MIMO for power class 1

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| n262 | 26.0 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance. |  |

6.2D.1.1.3.3	UE maximum output power for UL MIMO for power class 3

…

Table 6.2D.1.1.3.3-3: (void)

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.3-4 below. The requirement is verified with the test metric of EIRP (Link=spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.3-4: UE spherical coverage for UL MIMO for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

Following excerpt is from TS 38.101-2 [8]:

#### 6.2D.1.5	UE maximum output power for UL MIMO for power class 5

…

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE with UL MIMO is defined as the spherical coverage requirement and is found in Table 6.2D.1.5-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.5-3: UE spherical coverage for UL MIMO for power class 5

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 22 |
| n258 | 22.4 |
| n259 | 19.7 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance |  |

This is the identical requirement as for the SISO case except that there is no MBR for UL MIMO in PC3. Since the impact of the latter is negligible, we propose to reuse the MU and TT of the SISO MOP spherical TC for the UL MIMO spherical TC.

Proposal 6: Reuse the MU and TT of the SISO MOP spherical TC for the UL MIMO MOP spherical TC for PC1, PC3 and PC5.

### Minimum output power

The minimum requirement for UL MIMO reads (TS 38.101-2 [8]):

### 6.3D.1	Minimum output power for UL MIMO

#### 6.3D.1.0	General

The minimum output power is defined as the mean power in at least one sub frame (1ms). The minimum controlled output power is defined as the EIRP, i.e. the sum of the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the UE power is set to a minimum value.

#### 6.3D.1.1	Minimum output power for UL MIMO for power class 1

For UE supporting UL MIMO, the minimum output power shall not exceed the sum of the values specified in Table 6.3.1.1-1 and the quantity 10*log10(Number of Layers).

#### 6.3D.1.2	Minimum output power for UL MIMO for power class 2, 3 and 4

minimum output power shall not exceed the sum of the values specified in Table 6.3.1.2-1 and the quantity 10*log10(Number of Layers).

#### 6.3D.1.3	Minimum output power for UL MIMO for power class 5 and 6

For UE supporting UL MIMO, the minimum controlled output power is defined as the EIRP, i.e. the sum of the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the UE power is set to a minimum value. The minimum output power shall not exceed the values specified in Table 6.3.1.3-1. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

For single layer transmission 10*log10(Number of Layers) equals to 0 dB and the test of the requirement is covered in the SISO TC.

The MIMO TC is executed in 2 layers resulting in a relaxation of 3 dB compared to the SISO TC. Due to the low power levels of PC3 it is important to consider this 3 dB in the derivation of the relaxation and test tolerance.

For PC3, SISO the common agreement on the influence of noise is captured in Table B.2.2.27-1 in TR 38.903 [3]:

Table B.2.2.27-1: Uncertainty value for influence of noise for PC3 for IFF

| Test case | Frequency range | Noise floor | Minimum requirement | Estimated SNRtotal [dB/400MHz] | Relaxation | Influence of noise |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum output power | FR2a | -10.6dBm/400MHz | -13dBm | -2.54 (NOTE 1) | 8.4 | 1.0 (with relaxation) |
|  | FR2b | -5.5dBm/400MHz | -13dBm | -7.64 (NOTE 1) | 13.5 | 1.0 (with relaxation) |
| NOTE 1: Estimated SNR is calculated based on agreed influence of noise. |  |  |  |  |  |  |

For PC3, MIMO with 2 layers the minimum requirement is -10 dBm. Thus, the relaxation can be reduced by 3 dB for a CHBW of 400 MHz. Since relaxation for 400 MHz is still required, the influence of noise is 1 dB, i.e., the same as for SISO TC. Therefore, the MU for the MIMO and SISO TCs are identical with a value of 6.15 dB for FR2a and FR2b, NTC.

Proposal 7: Apply an influence of noise of 1 dB for UL MIMO minimum output power TC for a channel bandwidth of 400 MHz, PC3, IFF and QoQZ up to 40cm for PC3.

For the SISO TC the TT has been specified as:

TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) - R

We propose to reuse this formula in the MIMO case since the MUs for MIMO and SISO are identical. The resulting TT for FR2a and FR2b for the UL MIMO Minimum Output Power TC are summarized in Tables 1 and 2, respectively.

Table 1: TT and relaxation for MIMO Minimum Output Power in FR2a, PC3, NTC

| CHBW (MHz) | SNRtotal (dB) | R (dB) | SNRmr (dB) | TT (dB) |
| --- | --- | --- | --- | --- |
| 50 | 9.6 | 0.0 | 0.45 | 3.80 |
| 100 | 6.6 | 0.0 | 0.86 | 4.21 |
| 200 | 3.6 | 2.4 | 1.57 | 2.52 |
| 400 | 0.6 | 5.4 | 2.72 | 0.67 |

Table 2: TT and relaxation for MIMO Minimum Output Power in FR2b, PC3, NTC

| CHBW (MHz) | SNRtotal (dB) | R (dB) | SNRmr (dB) | TT (dB) |
| --- | --- | --- | --- | --- |
| 50 | 4.5 | 1.5 | 1.32 | 3.17 |
| 100 | 1.5 | 4.5 | 2.32 | 1.17 |
| 200 | -1.5 | 7.5 | 3.82 | 0.00 |
| 400 | -4.5 | 10.5 | 5.82 | 0.00 |

For PC1 the power level is in a similar range as for spherical coverage PC3 and the impact of the 3dB relaxation is negligible. Therefore, we propose to reuse the MU and TT of the SISO minimum output power TC for the UL MIMO minimum output power TC.

For PC5, there is no 3dB relaxation in the minimum conformance requirement and the requirements are the same for SISO and MIMO. Thus, we propose to reuse of the SISO minimum output power TC for the UL MIMO minimum output power TC.

Proposal 8: For PC3 and NTC, apply the TT and relaxation of Table 1 and Table 2 for the UL MIMO Minimum Output Power TC. For PC1 and PC5, reuse the MU and TT of the SISO minimum output power TC for the UL MIMO minimum output power TC.

### Tx OFF Power

The minimum conformance for UL MIMO Tx OFF power minimum requirement is specified in TS 38.521-2 [2] as follows:

6.3D.2.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports. During DTX and measurements gaps, the transmitter is not considered OFF. The minimum output power shall not exceed the values specified in Table 6.3.2.3-1. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Thus, the UL MIMO minimum requirement is identical to the SISO requirement and the influence of noise, relaxation and TT can be reused.

Proposal 9: Reuse MU and TT of the SISO Tx OFF Power TC for the corresponding MIMO TC.

Accompanying CR in Ref. [4], updates the editor’s note with respect to the OTA test procedure and the MU & TT in TS 38.521-2. CR in Ref. [5] updates the editor’s notes of the corresponding EN-DC test cases in TS 38.521-3. The associated applicability update is contained in the CR in Ref. [6]. CR in Ref. [7] documents the assessment in TR 38.903.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2 |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the MU for UL MIMO TCs which apply power measurements and make the following observations and proposals:

Observation 1: The UE is allowed to transmit simultaneously on both polarizations in the SISO TCs.

Observation 2: The measurement procedures for EIRP and TRP in Annex K of TS 38.521-2 [2] are based on both polarizations.

Proposal 1: For TCs using power measurements reuse the measurement procedures for EIRP and TRP already specified in Annex K of TS 38.521-2 [2].

Proposal 2: For UL MIMO TCs based on power measurements reuse the MU contributors and their values from the SISO TCs except for the influence of noise.

Proposal 3: Assess the influence of noise and the relaxation on a TC per TC level.

Proposal 4: Reuse the TT formula from the corresponding SISO TCs for the UL MIMO TCs.

Proposal 5: Reuse the MU and TT of the SISO MOP EIRP/TRP TC for the UL MIMO EIRP/TRP TC for PC1, PC3 and PC5.

Proposal 6: Reuse the MU and TT of the SISO MOP spherical TC for the UL MIMO MOP spherical TC for PC1, PC3 and PC5.

Proposal 7: Apply an influence of noise of 1 dB for UL MIMO minimum output power TC for a channel bandwidth of 400 MHz, PC3, IFF and QoQZ up to 40cm.

Proposal 8: For PC3 and NTC, apply the TT and relaxation of Table 1 and Table 2 for the UL MIMO Minimum Output Power TC. For PC1 and PC5, reuse the MU and TT of the SISO minimum output power TC for the UL MIMO minimum output power TC.

Proposal 9: Reuse MU and TT of the SISO Tx OFF Power TC for the corresponding MIMO TC.

# References

- R5-215533, Out-of-band emissions for FR2 UL MIMO Measurement uncertainties and test tolerances, Keysight Technologies UK Ltd, Sporton, 3GPP TSG RAN WG 5 Meeting #92-e, May 2021

- TS 38.521-2, V18.0.0, 3GPP TSG RAN WG5, September 2023

- TR 38.903, V18.0.0, 3GPP TSG RAN WG5, September 2023

- R5-237257, Update of MU and TT in FR2 UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237258, Editor's notes updates for FR2 UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237259, Applicability update for FR2 UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237256, Update of MU for FR2 UL MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- TS 38.101-2, V18.3.0, 3GPP TSG RAN WG4, September 2023