# Title:	Test Tolerances for PDSCH with inter-cell interference

Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for PDSCH with inter-cell interference.

The test case in question, from 38.521-4, is:

- 5.2.2.1.15,		2Rx FDD FR1 PDSCH with inter-cell interference

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in TS 38.101-4

The test conditions are defined in the following extracts from TS 38.101-4 v17.10.0.

##### 5.2.2.1.15	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.2.1.15-3, with the addition of test parameters in Table 5.2.2.1.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.15-1.

Table 5.2.2.1.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance under 2 receive antenna conditions, when transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 1-2 |

Table 5.2.2.1.15-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for Test 1-1 Disabled for Test 1-2 |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 N/A for Test 1-2 |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-1 7.58 for Test 1-2 | 2.29 for Test 1-1 N/A for Test 1-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-1 2nd SSB in Slot #0 for Test 1-2 | 1st SSB in Slot#0 for Test 1-1 N/A for Test 1-2 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 15.4 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

# 3. Discussion

The test consists of two subtests:

- Subtest 1-1: Checks the PDSCH performance on the serving cell (Cell 1) in the presence of two interfering cells (Cells 2 and 3)

- Subtest 1-2: Checks the PDSCH performance on the serving cell (Cell 1) in the presence of one interfering cell (Cell 2)

From a Test Tolerance analysis perspective, each subtest is analyzed independently in the accompanying spreadsheet.

The Test system measures PDSCH performance, and the minimum performance requirement is expressed as the SNR required to achieve 70% of the maximum throughput obtainable from the specified Reference Measurement Channel. We note that the SNR is specified as  of Cell1 in the spreadsheet.

To analyse the test case, it is helpful to visualise these as pie-charts showing the distribution of power, as below (example shown for subtest 1-1). Separate charts are given for all power within the channel bandwidth, and for the power within Noc’ (noise and interference only):

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables 5.2.2.1.15-2 and 5.2.2.1.15-3 in TS 38.101-4, which are INR of the interfering cells Cell 2 and Cell 3, SNR of the target cell. Since we need to compute the power contributions of each cell in linear domain, we need to factor Noc for this case. For ease of understanding, we use the term Es/Noc for each cell’s SNR. All the other parameters such as fractions of power and Es/Noc’ (SINR) are derived.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Note that the original specified Es/Noc for each Cell is a parameter settable by the Test system, and therefore subject to uncertainty. By contrast, Cell 1 Es/Noc’ is not directly settable by the test system, because it depends on several cell powers. Although Es/Noc’ is not specified in the test case, this SINR is critical for the test case.

c) Uncertainties

The SS provides 3 cells on the same frequency, and AWGN. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.5 dB

- Ratio of cell 1 signal / Fading, Ês1 / Noc ±0.8 dB

- Ratio of cell 2 signal / Fading, Ês2 / Noc ±0.8 dB

- Ratio of cell 3 signal / Fading, Ês3 / Noc ±0.8 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

The absolute level of Noc is specified as ±1.5 dB as for other demodulation tests, and is not critical.

Cells 1, 2 and 3 are faded, so the uncertainty for Ês/Noc needs to be widened from the usual default value of ±0.3dB. The approach taken here aligns with the values used for demodulation test cases in Annex F.1 of TS 38.521-4:

Overall system uncertainty for fading conditions comprises two quantities:

1. Signal-to-noise ratio uncertainty ±0.3dB

2. Fading profile power uncertainty ±0.7dB for 2Tx

Items 1 and 2 are assumed to be uncorrelated so can be root sum squared:

Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty2 + Fading profile power uncertainty2) = SQRT (0.32 + 0.72) = 0.76dB, round up to 0.8dB.

The effect of AWGN flatness and signal flatness is not included, because for wanted signal demodulation one of the critical parameters is Es/Noc’, which is SINR. As the largest proportion of the overall interference is the interfering cells, the demodulation is mainly determined by SIR. The signal and the interference generated by the test equipment are subject to the same flatness effect, so in the ratio signal-to-interference the flatness variation will cancel.

d) Controlled parameters critical to verdict

In this type of multi-cell demodulation test case, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case the following parameters are critical:

- Cell 1 Es/Noc'

- Cell 2 Es/Noc (the ratio of Interferer to Noc)

- Cell 3 Es/Noc (the ratio of Interferer to Noc)

The stimulus set by the Test system should be within these constraints for the UE measurement. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the Cell 1 Es/Noc would cause 1dB error in the Cell 1 Es/Noc’, so the sensitivity factor is 1.000. However, an error of 1dB in the absolute AWGN level Noc would cause no change to Cell 1 Es/Noc’ because this ratio is independent of the absolute AWGN value, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Noc’ which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example:

- The effect of Cell 2 Es/Noc uncertainty on Cell 1 Es/Noc’  is x 0.69

Cell 2 forms (69%/(11.5%+69%+19.5%) of the total Noc’. A change in the power of Cell 2 Es/Noc alone is diluted in the overall interference. These factors can also be derived intuitively, by looking at the Power in Noc’ pie chart in section 3.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

We observe the test equipment uncertainties could take the UE outside the specified Cell 2 Es/Noc and Cell 3 Es/Noc values. A cell 2 Es/Noc or cell 3 Es/Noc value lower than that specified in the test makes the wanted signal harder to demodulate (lower Es/Noc means that the proportion of AWGN, which is not cancelled, becomes higher). Cell 2 Es/Noc and cell 3 Es/Noc are therefore increased by an amount sufficient to achieve a minimum Es/Noc of at least the requirement value specified in section g).

We observe that increasing the cell 2 Es/Noc and cell 3 Es/Noc values has decreased the SINR (Cell 1 Es/Noc’), and that the test equipment uncertainties could reduce it further. Cell 1 Es/Noc is therefore increased by an amount sufficient to ensure that the minimum Cell 1 Es/Noc’ value meets the original requirement. This change does not affect the cell 2 Es/Noc and cell 3 Es/Noc values, so if the steps are taken in the order given this part of the process is not iterative.

f) Parameters modified by Test Tolerances

Based on the process in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Cell 2 Es/Noc is increased by an amount sufficient to achieve the original Cell 2 Es/Noc value. This offset is identical to the Es/Noc test system uncertainty 0.8dB.

Cell 3 Es/Noc is increased by an amount sufficient to achieve the original Cell 3 Es/Noc value. This offset is identical to the Es/Noc test system uncertainty 0.8dB.

Cell 1 Es/Noc is increased by an amount sufficient to achieve the original Cell 1 Es/Noc’ (SINR) derived in b). The actual offset is found empirically by observing the minimum/maximum parameter values in step g).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Cell 1 Es/Noc’, Cell 2 Es/Noc and Cell 3 Es/Noc). The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit (in these test cases, all limits are one-sided) are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

# 5. Treatment of other similar test cases

Similar test tolerances have been computed for other PDSCH tests with inter-cell interference in the attached spreadsheet. These are 5.2.2.2.16, 5.2.3.1.15 and 5.2.3.2.16.