3GPP TSG-RAN5 Meeting #102	R5-240764r1

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3067 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration of frequencyInfoSL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During RAN5#99, there was a discussion (R5-232496) on the ambiguous definition of frequencyInfoSL-r16. Then a LS (R5-233768) is sent to RAN2 to seek for clarification. After two meeting cycles RAN1 and RAN2 have agreed to clarify frequencyInfoSL-r16 as follows in reply LS (R2-231618): R2-2313618: RAN2 agrees to add, in TS 38.331, the field description for frequencyInfoSL in IE SL-MeasObjectList as " It indicates the lowest usable subcarrier on the carrier where SL RSRP is measured, determined according to sl-AbsoluteFrequencyPointA in IE SL-FreqConfig/SL-FreqConfigCommon and offsetToCarrier in IE SCS-SpecificCarrier configured for sl-SCS-SpecificCarrierList in IE SL-FreqConfig/SL-FreqConfigCommon. See TS 38.211 [16], clause 8.2.5.". Based on RAN2’s reply LS, we suggest update the default configuration of frequencyInfoSL-r16 in 38.508-1 Table 4.6.6-16 accordingly. |
| Summary of change: |  | Default configuration of frequencyInfoSL-r16 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.8, 4.6.3, 4.6.6, 6.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Updated according to comments received: Add value of frequencyInfoSL into test frequency tables. Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.1.8	Test frequencies for NR Sidelink operating bands

4.3.1.8.1	Test frequencies for NR Sidelink operating bands in FR1

4.3.1.8.1.1	Reference test frequencies for NR Sidelink operating band n38

FFS

4.3.1.8.1.2	Reference test frequencies for NR Sidelink operating band n47

Table 4.3.1.8.1.2-1: Test frequencies for NR operating band n47 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] |  | Range |  | Carrier centre [MHz] |  | Carrier centre [ARFCN] |  | point A [MHz] |  | sl-absoluteFrequencyPointA [ARFCN] |  | offsetToCarrier [Carrier PRBs] |  | frequencyInfoSL [ARFCN] |  | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-SSB Low |  | S-SSB Mid |  | S-SSB High |
| 10 | 52 |  | Low |  | 5860.005 |  | 790667 |  | 5855.325 |  | 790355 |  | 0 |  | 790355 |  | 790421 |  | 790673 |  | 790913 |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5794.59 |  | 786306 |  | 504 |  | 792354 |  | 792420 |  | 792672 |  | 792912 |
|  |  |  | High |  | 5919.99 |  | 794666 |  | 5914.23 |  | 794282 |  | 6 |  | 794354 |  | 794420 |  | 794672 |  | 794912 |
| 20 | 106 |  | Low |  | 5865 |  | 791000 |  | 5855.46 |  | 790364 |  | 0 |  | 790364 |  | 790430 |  | 791006 |  | 791570 |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5789.73 |  | 785982 |  | 504 |  | 792030 |  | 792096 |  | 792672 |  | 793236 |
|  |  |  | High |  | 5914.995 |  | 794333 |  | 5904.375 |  | 793625 |  | 6 |  | 793697 |  | 793763 |  | 794339 |  | 794903 |
| 30 | 160 |  | Low |  | 5870.01 |  | 791334 |  | 5855.61 |  | 790374 |  | 0 |  | 790374 |  | 790440 |  | 791340 |  | 792228 |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5784.87 |  | 785658 |  | 504 |  | 791706 |  | 791772 |  | 792672 |  | 793560 |
|  |  |  | High |  | 5910 |  | 794000 |  | 5894.52 |  | 792968 |  | 6 |  | 793040 |  | 793106 |  | 794006 |  | 794894 |
| 40 | 216 |  | Low |  | 5875.005 |  | 791667 |  | 5855.565 |  | 790371 |  | 0 |  | 790371 |  | 790437 |  | 791673 |  | 792897 |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5779.83 |  | 785322 |  | 504 |  | 791370 |  | 791436 |  | 792672 |  | 793896 |
|  |  |  | High |  | 5904.99 |  | 793666 |  | 5884.47 |  | 792298 |  | 6 |  | 792370 |  | 792436 |  | 793672 |  | 794896 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-2: Test frequencies for NR operating band n47 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] |  |  | Range |  | Carrier centre [MHz] |  | Carrier centre [ARFCN] |  | point A [MHz] |  | sl-absoluteFrequencyPointA [ARFCN] |  | offsetToCarrier [Carrier PRBs] |  | frequencyInfoSL [ARFCN] |  | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-SSB Low |  |  | S-SSB Mid |  | S-SSB High |  |
| 10 | 24 |  | Low |  | 5860.005 |  | 790667 |  | 5855.685 |  | 790379 |  | 0 |  | 790379 |  | 790511 |  |  | 790679 |  | 790823 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5704.23 |  | 780282 |  | 504 |  | 792378 |  | 792510 |  |  | 792678 |  | 792822 |  |
|  |  |  | High |  | 5919.99 |  | 794666 |  | 5913.51 |  | 794234 |  | 6 |  | 794378 |  | 794510 |  |  | 794678 |  | 794822 |  |
| 20 | 51 |  | Low |  | 5865 |  | 791000 |  | 5855.82 |  | 790388 |  | 0 |  | 790388 |  | 790520 |  |  | 791000 |  | 791480 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5699.37 |  | 779958 |  | 504 |  | 792054 |  | 792186 |  |  | 792666 |  | 793146 |  |
|  |  |  | High |  | 5914.995 |  | 794333 |  | 5903.655 |  | 793577 |  | 6 |  | 793721 |  | 793853 |  |  | 794333 |  | 794813 |  |
| 30 | 78 |  | Low |  | 5870.01 |  | 791334 |  | 5855.97 |  | 790398 |  | 0 |  | 790398 |  | 790530 |  |  | 791346 |  | 792138 |  |
|  |  |  | Mid |  | 5889.9 |  | 792666 |  | 5694.51 |  | 779634 |  | 504 |  | 791730 |  | 791862 |  |  | 792678 |  | 793470 |  |
|  |  |  | High |  | 5910 |  | 794000 |  | 5893.8 |  | 792920 |  | 6 |  | 793064 |  | 793196 |  |  | 794012 |  | 794804 |  |
| 40 | 106 |  | Low |  | 5875.005 |  | 791667 |  | 5855.925 |  | 790395 |  | 0 |  | 790395 |  | 790527 |  |  | 791679 |  | 792807 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5689.47 |  | 779298 |  | 504 |  | 791394 |  | 791526 |  |  | 792678 |  | 793806 |  |
|  |  |  | High |  | 5904.99 |  | 793666 |  | 5883.75 |  | 792250 |  | 6 |  | 792394 |  | 792526 |  |  | 793678 |  | 794806 |  |
|  |  | NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-3: Test frequencies for NR operating band n47 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] |  | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 |  | Low |  | 5860.005 |  | 790667 |  | 5856.045 |  | 790403 |  | 0 |  | 790403 |  | 790667 |  | 790667 |  | 790667 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5523.15 |  | 768210 |  | 504 |  | 792402 |  | 792666 |  | 792666 |  | 792666 |  |
|  |  |  | High |  | 5919.99 |  | 794666 |  | 5911.71 |  | 794114 |  | 6 |  | 794402 |  | 794666 |  | 794666 |  | 794666 |  |
| 20 | 24 |  | Low |  | 5865 |  | 791000 |  | 5856.36 |  | 790424 |  | 0 |  | 790424 |  | 790688 |  | 791024 |  | 791312 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5518.47 |  | 767898 |  | 504 |  | 792090 |  | 792354 |  | 792690 |  | 792978 |  |
|  |  |  | High |  | 5914.995 |  | 794333 |  | 5902.035 |  | 793469 |  | 6 |  | 793757 |  | 794021 |  | 794357 |  | 794645 |  |
| 30 | 38 |  | Low |  | 5870.01 |  | 791334 |  | 5856.33 |  | 790422 |  | 0 |  | 790422 |  | 790686 |  | 791358 |  | 791982 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5513.43 |  | 767562 |  | 504 |  | 791754 |  | 792018 |  | 792690 |  | 793314 |  |
|  |  |  | High |  | 5910 |  | 794000 |  | 5892 |  | 792800 |  | 6 |  | 793088 |  | 793352 |  | 794024 |  | 794648 |  |
| 40 | 51 |  | Low |  | 5875.005 |  | 791667 |  | 5856.645 |  | 790443 |  | 0 |  | 790443 |  | 790707 |  | 791667 |  | 792627 |  |
|  |  |  | Mid |  | 5889.99 |  | 792666 |  | 5508.75 |  | 767250 |  | 504 |  | 791442 |  | 791706 |  | 792666 |  | 793626 |  |
|  |  |  | High |  | 5904.99 |  | 793666 |  | 5882.31 |  | 792154 |  | 6 |  | 792442 |  | 792706 |  | 793666 |  | 794626 |  |
|  |  | NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>

<Start of modified section 2>

–	ARFCN-ValueNR

Table 4.6.3-5: ARFCN-ValueNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueNR | absoluteFrequencySSB as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_SSB |
|  | absoluteFrequencyPointA as defined for the DL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_PointA |
|  | absoluteFrequencyPointA as defined for the UL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | UL_PointA |
|  | sl-absoluteFrequencySSB as defined for the SL NRf1 carrier | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_SSB |
|  | sl-AbsoluteFrequencyPointA as defined for the SL NRf1 carrier | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_PointA |
|  | FrequencyInfoSL as defined for the SL NRf1 carrier | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_MeasFrequencyInfo |

| Condition | Explanation |
| --- | --- |
| DL_SSB | IE absoluteFrequencySSB for downlink |
| SL_SSB | IE sl-absoluteFrequencySSB for sidelink |
| DL_PointA | IE absoluteFrequencyPointA for downlink |
| UL_PointA | IE absoluteFrequencyPointA for uplink |
| SL_PointA | IE sl-absoluteFrequencyPointA for sidelink |
| SL_MeasFrequencyInfo | IE frequencyInfoSL for sidelink |

<End of modified section 2>

<Start of modified section 3>

–	SL-MeasObjectList

Table 4.6.6-16: SL-MeasObjectList

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasObjectList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-ObjectId-r16)) OF SL-MeasObjectInfo-r16 { |  | 1 entry |  |  |
| SL-MeasObjectInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MeasObjectId-r16 |  | 1 |  |  |
| sl-MeasObject-r16 SEQUENCE { |  |  |  |  |
| frequencyInfoSL-r16 |  | ARFCN-ValueNR with condition SL_MeasFrequencyInfo | For signalling test cases, set as subclause 6.2.3.7. Otherwise, set as subclause 4.3.1.8. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 3>

<Start of modified section 4>

6.2.3.7	Test frequencies for NR sidelink configurations for signalling testing

The default channel bandwidths for NR sidelink signalling test are specified per NR sidelink operation band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR sidelink test frequencies are as follows:

-	for band with only one test frequency: Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

The test frequencies, subcarrier spacing, default channel bandwidth and S-SSBparameters for signalling is specified in Table 6.2.3.7-1 for PC5-only operations.

For concurrent operation case the operation configurations are specified in clause 4.3.1.8.2.1 for inter-band concurrent cases. The test frequencies for signalling are specified in Table 6.2.3.7-1 for PC5 carrier and in clause 6.2.3.1 for Uu carrier.

For tests which need NR PC5 carrier and NR Uu carrier but not concurrent operation case, the test frequencies for signalling are specified in Table 6.2.3.7-1 for PC5 carrier in clause 6.2.3.1 for Uu carrier.

Table 6.2.3.7-1: Test frequencies for NR Sidelink operating bands using 10 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n47 | 30 | 10 | 24 | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.8.1.2 for bandwidth=10 MHz and SCS=30 kHz. For sl-absoluteFrequencySSB, same value as for S-SSB Low in clause 4.3.1.8.1.2 for bandwidth=10 MHz and SCS=30 kHz is used. |  |  |  |  |  |  |

<End of modified section 4>