3GPP TSG-RAN5 Meeting #103	R5-242282r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5502 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Additional spurious emission test requirement for NS_45 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_TDD_2400_US-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The frequency range in Table 6.6.3.3.5.25-1/2 of additional requirements for 1.4/3/5/10M bandwidths does not align with definition in the Table 6.6.3.3.34-1/2 of 36.101-1 for NS-45 additional spurious emissions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the frequency band definition in the Table 6.6.3.3.5.25-1/2 to align with the correspond definition in TS 36.101-1 Correction of the frequency band definition in the Table 6.6.3.3.3.29-1/2 to align with the correspond definition in TS 36.101-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The frequency band definition keep incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.3.3, 6.6.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 version : 1: correction to WIC in cover sheet 2: correction of minimum requirement table 3: added clause 6.6.3.3.3 in cover sheet |  |  |  |  |  |  |  |  |

<Start of Change1 >

6.6.3.3.3.29	Minimum requirement (network signalled value “NS_45”)

When "NS_45" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.29-1 & 6.6.3.3.29-2. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.3.29-1: Additional requirements for 1.4,3 and 5 MHz channel bandwidths

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz |  |
| 0.009 < f ≤ 2477.5 | -25 | 1 MHz |
| 2477.5 < f ≤ 2478.5 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |
| 2496< f ≤ 2501 | -13 | 1 MHz |
| 2501 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |

Table 6.6.3.3.3.29-2: Additional requirements for 10 MHz channel bandwidths

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 0.009 < f ≤ 2473.5 | -25 | 1 MHz |
| 2473.5 < f ≤ 2478.5 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |
| 2496< f ≤ 2505 | -13 | 1 MHz |
| 2505 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |

The normative reference for this requirement is TS 36.101[2] subclause 6.6.3.3.34.

<End of Change1 >

<Skip Unchanged >

<Start of Change2 >

6.6.3.3.5.25	Test requirement (network signalled value “NS_45”)

When "NS 45" is indicated in the cell:

-	the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.5.25-1 & Table 6.6.3.3.5.25-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3.3.5.25-1: Additional requirements

Table 6.6.3.3.5.25-1: Additional requirements for 1.4,3 and 5 MHz channel bandwidths

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz |  |
| 0.009 < f ≤ 2477.5 | -25 | 1 MHz |
| 2477.5 < f ≤ 2478.5 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |
| 2496< f ≤ 2501 | -13 | 1 MHz |
| 2501 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |

Table 6.6.3.3.5.25-2: Additional requirements for 10 MHz channel bandwidths

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 0.009 < f ≤ 2473.5 | -25 | 1 MHz |
| 2473.5 < f ≤ 2478.5 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |
| 2496< f ≤ 2505 | -13 | 1 MHz |
| 2505 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |

<End of Change2 >