3GPP TSG-RAN5 Meeting #103	R5-242319r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3433 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR SDT: Addition of NR/5GC Test Model aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR/5GC test model needs to be enhanced to document the NR SDT testing methodology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new clause 7.3.2.3. Added new abbreviations in clause 3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RA and CG SDT procedures will not be documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.3.2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Text clarification made in clause 7.3.2.3.2, to make more generic to cater for the test cases needs. |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

BWP	Bandwidth Part

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CG-SDT	Configured Grant-based SDT

CORESET	Control Resource Set

CSS	Common Search Space

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FDD	Frequency Division Duplex

FD-FDD	Full-Duplex FDD

FR1	Frequency Range 1

FR2	Frequency Range 2

HD-FDD	Half-Duplex FDD

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LBT	Listen Before Talk

LSB	Least Significant Bit

MCG	Master Cell Group

MN	Master Node

MSB	Most Significant Bit

MUSIM	Multi-Universal Subscriber Identity Module

NCD-SSB	Non Cell Defining SSB

NE-DC	NR-E-UTRA Dual Connectivity

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PF	Paging Frame

PH	Paging Hyperframe

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PTC	Parallel Test Component

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RA-SDT	Random Access-based SDT

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

TDD	Time Division Duplex

UDC	Uplink Data Compression

UL	Uplink

UL-SCH	Uplink Shared Channel

USS	UE-specific Search Space

UT	Upper Tester

VNG	Virtual Noise Generator

<START OF NEXT MODIFIED SECTION>

##### 7.3.2.2.2	Half-Duplex FDD operation

A RedCap UE in Half-Duplex FDD mode (HD-FDD) has limitations in receiving an UL symbol following a DL symbol, hence a blank slot is needed in between, defined as gap T_Proc,2.

NOTE:	The gap T_Proc,2 is specified in TS 38.213 [21] clause 17.2.

If the UE under test is an HD-FDD UE:

-	TTCN configures each cell in the SS to operate in HD-FDD mode.

-	When explicit TimingInfo (i.e. not "Now") is provided in any DL transmission, the SS shall ignore the FDD mode and shall follow the explicit scheduling. It is the TTCN responsibility to schedule UL/DL operations based on the UE capability.

-	Paging is always explicitly scheduled and hence SS shall follow the explicit scheduling for paging.

-	When TimingInfo is "Now", the SS shall restrict UL/DL operations as per Table 7.3.2.2.2-1 below:

Table 7.3.2.2.2-1: Data scheduling for FR1: HD-FDD, SCS=15kHz (non-explicitly scheduled)

| Subframe | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | DCI P-RNTI / PDCCH UL grant#1 / SSBs | DCI P-RNTI / SIB1 | DCI P-RNTI / SIB1 / PDSCH#1 |  |  | PDCCH UL grant#2 / SI | SI | PDSCH#2 |  |  |
| UL |  |  |  | Gap T_Proc,2 | RA occasion / PUSCH#1 / HARQ of PDSCH#2 |  |  |  | Gap T_Proc,2 | PUSCH#2 / HARQ of PDSCH#1 |

#### 7.3.2.3	Small Data Transmission

##### 7.3.2.3.1	RA-SDT

For SDT procedure over RACH, SDT DTCH data is expected together with CCCH/RRCResumeRequest:

-	in Msg3 when 4-step RA type is configured,

-	or in MSGA when 2-step RA type is configured.

When configuring UE to RRC_INACTIVE state, in addition to the SRBs, all DRBs are configured in SS and security is activated.

##### 7.3.2.3.2	CG-SDT

For SDT procedure over CG resources, TTCN configures the initial UL and DL BWP and DCI in the SS according to parameters provided to UE in SDT-MAC-PHY-CG-Config-r17. When the CG-SDT procedure is completed, the initial UL and DL BWP are reconfigured in the SS as per default values provided to UE in RRCSetup/RRCResume.

SS is configured with the grant allocation type 2, as per clause 7.1.2.3.1.2.3.

When configuring UE to RRC_INACTIVE state, in addition to the SRBs, all DRBs are configured in SS and security is activated.