3GPP TSG-RAN5 Meeting #103	R5-242373r1

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2748 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding transmitter requirements for band n106 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing transmitter requirements for band n106 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added transmitter requirements for band n106 in clauses - 6.2.1 UE maximum output power - 6.2.2 UE maximum output power reduction - 6.2.3 UE additional maximum output power reduction - 6.5.3.2.5 Spurious emissions for UE co-existence test requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmitter requirements for band n106 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.3, 6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Proposed implementation ordef for the CRs => 2754, 2755, 2756, 2747, 2748, 2749 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add CR implementation order to the coversheet |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 |  |  |  |  | 23 | ± 23 |
| n8 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n41 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| n106 |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n91 |  |  |  |  | 23 | e |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1,4 Table 6.2.2.5-r.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n100 | NS_01 |  |  |  |  |  |  |  |
| n101 | NS_01 |  |  |  |  |  |  |  |
| n106 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.5.3.2.3-4: Requirements for spurious emissions for UE co-existence Rel-18

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15, 43 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n2 | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 42, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n5, n89 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 52, 53, 54 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 71, 72, 74, 75, 76, 85, 103 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| n8, n81, n93, n94 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 54, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n13 | E-UTRA Band 2, 4, 5,12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 54, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14, 103 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
| n14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 12, 15 |
| n18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| n20, n82, n91, n92 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| n24, n99 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n25 | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 42, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n26 | E-UTRA Band 1, 2, 3, 4, 5, 7, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53, 54 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76, NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| n30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| n34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 74, 75, 76, NR Band n78, n79, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 25, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 71, 72, 74, 75, 76, 85, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| n39, n98 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 15, 26, 33 |
| n40, n97 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 44 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8 |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n71, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| n50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 65, 66, 67, 68, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
| n51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
| n53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| n65 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76 NR Band n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 43 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n66, n86 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 47, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 30, 38, 48, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 15, 42 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 15, 45 |
|  | Frequency range | 1475.9 | - | 1510.9 | -35 | 1 | 15, 46 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 54, 65, 66, 70, 71, 74, 85, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n78 | E-UTRA Band 1, 2, 3, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 28, 32, 34, 38, 39, 40, 41, 50, 65, 66, 67, 70, 71, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n95 | E-UTRA Band 1, 3 , 5, 8, 28, 39, 40, 41 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n100 | E-UTRA Band 1, 3, 8, 20, 28, 31, 32, 33, 34, 38, 40, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7, 22, 42 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| n101 | E-UTRA Band 1, 3, 8, 20, 22, 28, 31, 32, 38, 40, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7, 42, 43 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| n106 | E-UTRA Band 2, 4, 12, 13, 14, 23, 24, 25, 30, 53, 54, 66, 70, 71, 85, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -30 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 [2] or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: 15 kHz SCS is assumed when RB is mentioned in the note when channel bandwidth is less than or equal to 50 MHz, lowest SCS is assumed when channel bandwidth is larger than 50 MHz. The transmission bandwidth in terms of RB position and range is not limited to 15 kHz SCS and shall scale with SCS accordingly. NOTE 4: Void NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Void NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 9: Void NOTE 10: Void NOTE 11: Void NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: Void NOTE 14: Void NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Void NOTE 21: This requirement is applicable for any channel bandwidths up to 20MHz within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: Void NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: Void NOTE 29: Void NOTE 30: Void NOTE 31: Void NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth up to 20MHz and confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. The above restriction is applicable to only power class 3 UEs. NOTE 34: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718-728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 35: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: Void NOTE 37: Void NOTE 38: Void NOTE 39: Void NOTE 40: Void NOTE 41: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power configured as high as possible but no higher than 15 dBm. NOTE 42: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is more than 1460 MHz and less than or equal to 1470 MHz for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is more than 1460 MHz and less than or equal to 1465 MHz for 10 MHz bandwidth. NOTE 43: TThis requirement is applicable for UE which is operating in power class 3 and NR channel bandwidths up to 20 MHz within frequency range 1920-1980 MHz. NOTE 44: As exceptions, for 90 and 100 MHz channel bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. NOTE 45: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is equal to or less than 1460 MHz. NOTE 46: Applicable for 5 MHz bandwidth and when the NR carrier is within 1447.9 – 1462.9 MHz. |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.5.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-1 to 6.5.3.2.3-4 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5.3.2.3-1 to 6.5.3.2.3-4 for difference releases.

The requirements for the UE are release specific and can be found in Tables 6.5.3.2.3-1 to 6.5.3.2.3-4. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.5.3.2.5-1.

Table 6.5.3.2.5-1: UE Requirements according to UE NR release and supported E-UTRA and NR band

| UE Requirements per release |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR Band | Rel-15 | Rel-16 | Rel-17 | Rel-18 |
| n1 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n2 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n3 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n5 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n7 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n8 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n12 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n13 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n14 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n18 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n20 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n24 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n25 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n26 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n28 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n30 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n34 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n38 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n39 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n40 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n41 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n48 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n50 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n51 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n53 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n65 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n66 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n70 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n71 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n74 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n77, n78 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n79 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n91 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n92 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n93 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n94 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n100 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n101 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n106 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| NOTE 1: The frequency range applicable with network signalling values of NS_04, NS_17, NS_18, NS_05, NS_43, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are covered in subclause 6.5.3.3 Additional Spurious Emissions NOTE 2: The restriction on the maximum uplink transmission to 54 RB in Notes 21 and 22 of Tables 6.5.3.2.3-1 to 6.5.3.2.3-3 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future. |  |  |  |  |

-------- End of changes --------