3GPP TSG-RAN5 Meeting #103	R5-242533r1

# Fukuoka City, Fukuoka, May 20 - 24 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3139 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Test frequency limitations for CA in Annex C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The number of CA combinations in test frequency tables will be reduced when not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Clause C.2.4.3.4 updated with clarification that only when the number of CA combination becomes to high, the number of combinations to be added in tables should be reduced. -Clause C.3.2 updated with rounding operations to avoid risk of erroneous calculation result. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The number of CA combinations in test frequency tables might be reduced when there is no need. - SSB, Coreset#0 and test frequencies might be erroneous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.4.3.4, C.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Wording in notes in clause C.3.2 changed. |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

#### C.2.4.3.4	Determination of CBW combinations to add in initial test frequency tables

There is a need to, for some bands, limit the number of initial CBW combinations in the tables for the test frequencies for UL CA, clause 4.3.1.1.5: NR intra-band non-contiguous CA configurations in FR1 and clause 4.3.1.2.4: NR intra-band non-contiguous CA configurations in FR2. If the total number of CBW combination exceeds a value of 35 combinations, the number of combinations should be reduced according to the steps described in this clause. The intention is to include the CBW combinations that is the most likely to be needed for testing. . .

The determination of which CBW combinations to add in the initial tables, is based on two statements stated in test cases:

1.	Initial conditions for test: Lowest NRB_agg,  Highest NRB_agg.

2.	If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested.

The determination of initial CBW combinations to add in new tables for the test frequencies shall follow below steps:

1.	One, and only one, combination for each possible value of aggregated CBW for a specific BCS. The maximum aggregated BW in table will be limited to Maximum supported Aggregated CBW as stated in 38.521-1 [14], Table 5.5A.2-1. The aggregated BW of the combinations will be restricted to even 10 MHZ values.

2.	Each combination according to step 1 above shall be configured to use the highest NRB_PCC (CBW) possible, to comply with statement 2 above.

<< Unchanged clauses omitted>>

## C.3.2	Determination of SSB, CORESET#0 and signalling parameters for a PCell

Calculation of SSB and CORESET#0 parameters is limited to FR1 carriers with SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a PCell.

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	Determine SSB and CORESET#0:

1a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

1b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

1c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

1d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor(Round((FSSBlow - FcarrierLow) / CRBsize),5) + FcarrierLow Note 1

Note 1: The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. The “Round” operation is to be done to 5 decimals. It is needed to avoid that a calculated value of for example 5.9999999999 is Floored to the value of 5.

1e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = Round((FoffsetToPointA - FcarrierLow) / CRBsize) Note 2

Note 2: The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. It is needed to make sure that the Max_OffsetRBs is an integer. For example a calculated value of Max_OffsetRBs = 5.9999999999 shall give a Max_OffsetRBs = 6.

1f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 1g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1f.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.3 and the procedure is completed.

1g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1). Cases requiring more than 3*ΔFRaster shifts shall be analyzed individually and, if eventually agreed, marked in section 4.3 to have been obtained applying certain exceptions over this section C.3.2. In case of an exception, for Mid, Mid-Low and Mid-High, shifting frequency up can also be considered to achieve the frequency closer to the ideal case.

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.3.2-1.

Table C.3.2-1: Downlink and uplink offsetToCarrier default values for different frequency ranges

| Frequency range | Downlink offsetToCarrier | Uplink offsetToCarrier |
| --- | --- | --- |
| Low range | 0 | 0 |
| Mid range | 102 | 504 |
| High range | 504 | 6 |
| Mid-Low range | 12 | 36 |
| Mid-High range | 24 | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |  |

In case low, mid and high range are exactly the same frequency, use offsetToCarrier associated to low range.

2b.	Determine FpointA:

FPointA = FcarrierLow - offsetToCarrier * PRBsize

3.	Calculate ΔFOffsetCORESET-0-Carrier:

The ΔFOffsetCORESET-0-Carrier value is used to calculate the Offset Carrier CORESET#0 parameter included in the test frequency tables in sub-clauses 4.3.1 and 6.2.3.

ΔFOffsetCORESET-0-Carrier = FoffsetToPointA - OffsetRB * CRBsize - FcarrierLow

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to the index associated with the selected OffsetRBs value in the applicable table, 13-1 to 13-10, in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | (FOffsetToPointA - FPointA) / (12*{15 kHz for FR1; 60 kHz for FR2}). The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | Set to value calculated in step 2a. The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

##### << End of changes>>