3GPP TSG-RAN5 Meeting #103	R5-242537r1

# Fukuoka City, Fukuoka, May 20 - 24 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0891 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of checklist for intra band CA in Annex B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing checklist in B.9 for Intra band CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of checklist for Intra band CA, clause B.9.0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Missing checklist for intra band CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

# B.9	Reference Sensitivity checklist for CA

The purpose of this annex is to facilitate the reference sensitivity test point analysis in TR 38.905 for NR CA configurations.

B.9.0	Checklist for intra band CA

1)	Add the new CA configuration to Table 4.1.3.1-1 with NE: (Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply).

2)	Add the 2 DL intra band CA configuration in TS 38.521-1 [2] Table 7.3A.1.5-1

## B.9.1	Checklist for two bands

1)	Check if the 2 DL band configuration has reference sensitivity exceptions in the tables listed below:

-	UL Harmonic Distortion (HD), as defined in TS 38.101-1 [5], Table 7.3A.4-1.

-	RX Harmonic Mixing (HM), as defined in TS 38.101-1 [5], Table 7.3A.4-4 to Table 7.3A.4-4a.

-	Intermodulation Distortion (IMD), as defined in TS 38.101-1 [5], Table 7.3A.5-1 to Table 7.3A.5-1a.

-	Cross Band Isolation (CBI), as defined in TS 38.101-1 [5], Table 7.3A.6-1 to Table 7.3A.6-1b.

2)	If reference sensitivity exceptions are found for two bands above:

-	Add test frequencies for HD, HM and CBI to Table 4.1.3.1-6.

-	Add two band test points to test case in TS 38.521-1 [2] Table 7.3A.1_1.4.1-1.

3)	Add the result from the check to Table 4.1.3.1-2 for 2CC if they do not already exist.

-	If there are no sensitivity exceptions then still add the band configuration to Table 4.1.3.1-2 with

NE: (Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply).

4)	Add the 2 DL band configuration in TS 38.521-1 [2] Table 7.3A.1.5-1

## B.9.2	Checklist for three bands

1)	Check all three band fallback configurations in accordance to B.9.1.

2)	Check for any 3DL/2UL IMD reference sensitivity exceptions in TS 38.101-1 [5], Table 7.3A.5-2 to Table 7.3A.5-2a.

3)	If exceptions are found for three bands above:

-	Add three band test points to test case in TS 38.521-1 [2] Table 7.3A.2_1.4.1-1.

4) Add the result from the check to Table 4.1.3.1.3-3 for the three bands if they not already exist.

-	If there are no sensitivity exceptions then still add the band configuration to Table 4.1.3.1-3 with

NE: (Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply).

5)	Add the 3 DL band configuration in TS 38.521-1 [2] Table 7.3A.2.5-1

##### << End of changes>>