3GPP TSG-RAN5 Meeting # 103	R5-242543r1

# Fukuoka City, Fukuoka, JP,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3142 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Added Common test environment for NR-NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024/05/01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Common test environment for NTN test cases is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Common test enviromnent for NTN test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN test cases will not be implementatble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Type corrected in 4.5B.3, rephrased same section for more clarity |  |  |  |  |  |  |  |  |

<Beginning of changes>

## 4.5B	Common test environment for Vertical UEs

### 4.5B.1	SNPN-only UEs

This clause defines the test environment which applies to all test cases listed in TS 38.523-2 [19], clause 4.3 and executed for SNPN-only UEs (TS 38.508-2 [10] A.4.1-5/3), unless otherwise specified.

-	Each configured NR Cell is an SNPN cell, i.e., condition ‘SNPN’ (Standalone NPN cell) applies in the default message contents.

-	The SIB1 of each NR Cell includes the NPN-Identity-r16 set to snpn-r16 with the PLMN of the cell and the nid-List-r16 IE configured with only one entry of NID with Assignment mode = 1 and NID value = 0.

-	UE’s "list of subscriber data" is pre-configured with the list of all PLMNs and NID value = 0.

NOTE: PLMNs are specified in Table 4.4.2-3 or clause 6.3.2 unless specified otherwise in the test case.

### 4.5B.2	RedCap UEs

This clause defines the test environment which applies to all test cases executed against RedCap UEs, unless otherwise specified.

-	The test channel bandwidth is reduced, to a maximum of 20MHz for FR1 and 100MHz for FR2.

-	The following ASN.1 types are configured with condition pc_supportOfRedCap_r17 = TRUE:

-	SIB1 in Table 4.6.1-28,

-	SIB4 in Table 4.6.2-3,

-	RLC-Config in Table 4.6.3-149, and

-	PDCP-Config in Table 4.6.3-99.

-	In case of HD-FDD mode, the following ASN.1 type is configured with condition pc_halfDuplexFDD_TypeA_RedCap_r17 = TRUE:

-	SIB1 in Table 4.6.1-28.

### 4.5B.3	NR-NTN Only UEs

This clause defines the test environment which applies to all test cases executed against NR-NTN UEs, unless otherwise specified.

-	Each configured NR Cell is an NTN cell

-	Test uses system information Combination NR-28 and NR-29

-    The following RRC message is configured with condition NTN.

-    SIB1 in Table 4.6.1-28

-    The following ASN.1 type is configured with condition GSO or

NGSO.

-    SIB19 in Table 4.6.2-18C

## 4.5C	UE Position Requirements for NTN testing

### 4.5C.1	General

This clause defines the UE positioning requirements which apply for all NR NTN cases.

-	UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position:

-	Latitude: 25.08439333

-	Longitude: 121.56076999

-	Altitude: 0

<End of Changes >