3GPP TSG-RAN5 Meeting #103	R5-242881r1

# Fukuoka, Japan, May 20th 2024 - May 24th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3141 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM test case 16.2.1.2 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To align with latest version of 38.133; - To update test parameters taking test tolerance into consideration. - To update Editor’s note |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3165 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2410405 RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To update test parameter based on R4-2410405 and updated TT analysis. Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

16.2.1.2	NR SA FR1 CG-SDT for 2Rx UE

16.2.1.2.1	Test purpose

To verify that the UE properly perform TA validation for CG-SDT transmission as specified in TS 38.133 [6] clause 5.2B.2.1.

16.2.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards and supporting CG-SDT.

16.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.2.1.2.

16.2.1.2.4	Test description

Same as the test description given in clause 6.2.1.4.

16.2.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.2.1.4.1 with following exceptions:

-	Table 6.2.1.4.1-1 is replaced by Table 16.2.1.2.4.1-1;

-	Table 6.2.1.4.1-2 is replaced by Table 16.2.1.2.4.1-2;

-	Table 6.2.1.4.1-3 is replaced by Table 16.2.1.2.4.1-3;

Table 16.2.1.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.2.1.2-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR FDD |
| 16.2.1.2-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR TDD |
| 16.2.1.2-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, NR TDD |
| 16.2.1.2-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR HD-FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.2.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.2.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD Configuration | Config 1,4 |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.3 FDD |  |
|  | Config 2 |  | CCR.1.3 TDD |  |
|  | Config 3 |  | CCR.2.2 TDD |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB. 1 RedCap FR1 |  |
| OCNG Patterns |  |  | OP.1 |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |
| DRX configuration |  |  | DRX.7 |  |
| T_delay_modeB |  | s | 4 |  |
| T1 |  | s | 0.4 |  |
| T2 |  | s | 1.28 | 2 x W1 |
| T3 |  | s | 2.72 | T_timer_modeB - W1 -W2 |
| T4 |  | s | 1.5 | W2+W3 |
| T5 |  | s | 1.68 | T1+T2 |
| T6 |  | s | 4.22 | T3+W2+W3 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 8 |  |
| cg-SDT-RSRP-ThresholdSSB | Config 1,2,4 | dBm | -111 | Including test tolerance |
|  | Config 3 |  | -110 |  |
| cg-SDT-TimeAlignmentTime |  |  | Infinity |  |
| CG-SDT resource period |  | ms | 320 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |
| Propagation condition |  |  | AWGN |  |

16.2.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.2.1.4.2.

16.2.1.2.4.3	Message contents

Same as the message contents given in 6.2.1.4.3 with following exceptions:

-	Table 6.2.1.4.3-2 is replaced by Table 16.2.1.2.4.3-1.

Table 16.2.1.2.4.3-1: SDT-MAC-PHY-CG-Config-r17

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 46 | -110dBm | Config 3 |
|  | 45 | -111dBm | Config 1,2,4 |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.1.2.5	Test requirements

Same as the test requirements given in clause 6.2.1.5 with following exceptions:

-	Table 6.2.1.5-1 is replaced by Table 16.2.1.2.5-1.

Table 16.2.1.2.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| Note1 | 1,2,3,4 | dBm/15kHz | -100 |  |  |  |  |  |
| Note1 | 1,2,4 | dBm/SSB SCS | -100 |  |  |  |  |  |
|  | 3 |  | -97 |  |  |  |  |  |
|  |  | dB | 0 | 12 | 0 | 14 | 14 | 0 |
| SS RSRP Note2 | 1,2,4 | dBm/SSB SCS | -100 | -88 | -100 | -86 | -86 | -100 |
|  | 3 |  | -97 | -85 | -97 | -83 | -83 | -97 |
| Io Note2 | 1,2,4 | dBm/ 9.36 MHz | -69.04 | -59.78 | -69.04 | -57.88 | -57.88 | -69.04 |
|  | 3 | dBm/ 18.36 MHz | -66.11 | -56.86 | -66.11 | -54.95 | -54.95 | -66.11 |
|  |  | dB | 0 | 12 | 0 | 14 | 14 | 0 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |