3GPP TSG-RAN5 Meeting #103	R5-242962r1

# Fukuoka City, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0517 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test Case 14.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Case 14.3.6 is missing from the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Case 14.3.6 has been added to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 14.3.6 will remain missing from the specification, |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Removed editor’s note for test requirements, as it is included inTT analysis note. Corrected typos in message contents |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 14.3.6	NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance is missing

- Message contents require update

#### 14.3.6.1	Test purpose

The purpose of the test is to verify that the RSTD measurements when the measurements are within the same Rx TEG meets the accuracy requirements specified in TS 38.133 [50] clause 10.1.23.2 in an environment with AWGN propagation conditions.

#### 14.3.6.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports UE assisted DL-TDOA positioning and nr-UE-RxTEG-ID-MaxSupport.

#### 14.3.6.3	Minimum conformance requirements

Same as defined in clause 14.3.5.3.

#### 14.3.6.4	Test description

##### 14.3.6.4.1	Initial conditions

The supported test configuration is listed in Table 14.3.6-1.

Table 14.3.6.4-1: Test Configuration

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid-Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.3.6.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.13.

2.	The general test parameter settings are set up according to Table 14.3.6.5-1.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.3.6.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR2. GP#24 is configured if UE supports GP#24. otherwise, GP#0 is configured for the test.

6.	The true RSTD (which is the receive time difference for frame 0 between cell 2 and cell 1 as seen at the UE antenna connector) is set to 5898 Tc (about 3 s)

##### 14.3.6.4.2	Test procedure

The test consists of two sub-tests; the difference between the sub-tests is the PRS configuration. The test consists of a set-up period and a measurement period. Cell 1 and Cell 2 are both active during the complete test. The beginning of the measurement period shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. The NR-DL-TDOA-RequestLocationInformation message and the DL-TDOA assistance data as defined in clause 14.3.6.4.3 shall be provided to the UE during the set-up period.

The UE is configured with measurement gap pattern ID # 24 if UE supports MG#24, otherwise #0 before T2.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 14.3.6.5-1 as appropriate. Propagation conditions are set according to clause 4.15.2.

4.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

5.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the UE supports nr-UE-RxTEG-ID-MaxSupport in the NR-UE-TEG-Capability IE and DL-TDOA capabilities supported by the UE in the NR-DL-TDOA-ProvideCapabilities IE.

6.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-TDOA-ProvideAssistanceData IE. If the UE message at step 5 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

7.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the nr-UE-RxTEG-Request-r17 in the NR-DL-TDOA-RequestLocationInformation IE such that the UE receives the message T ms before the start of the measurement period, where T = 50 ms.

8.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-TDOA-ProvideLocationInformation IE within the response time (see clause 4.15.3).

9.	If the UE message at step 8 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

10.	The SS shall check the nr-RSTD-r16 value for Cell 2 in the nr-DL-TDOA-SignalMeasurementInformation-r16 according to Table 14.3.6.5-3.

11.	Repeat steps 2-10 in Table 14.3.6.4-1 until the confidence level according to Annex D is achieved.

12.	Repeat step 1-11 for the other sub-test defined in Table 14.3.6.5-1 as appropriate.

If all (applicable) sub-tests pass, the whole test passes. If one (applicable) sub-test fails, the whole test fails.

##### 14.3.6.4.3	Message contents

Table 14.3.6.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 1 | DL-TDOA |  |

Table 14.3.6.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-TDOA-RequestCapabilities-r16 | TRUE |

Table 14.3.6.4.3-3: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 14.3.6.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 |  |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | Not present |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTEG-Request-r17 | requested |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.6.4.3-4: LPP ProvideAssistanceData

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 | Not present |  |  |
| tbs-ProvideAssistanceData-r14 | Not present |  |  |
| wlan-ProvideAssistanceData-r14 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 14.3.6.4.3-5 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 | Not present |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present |  |  |
| assistanceDataValidityArea-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.6.4.3-5: NR-DL-PRS-AssistanceData

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Sub-test 1-1, Sub-test 1-2, Sub test 2-1 and Sub-test 2-2 |
|  | kHz30 |  | Sub-test 3-1 and Sub-test 3-2 |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs | Sub-test 1-1, Sub-test 2-1 and Sub-test 3-1 |
|  | 21 | 104 PRBs | Sub-test 1-2 and Sub-test 2-2 |
|  | 28 | 132 PRBs | Sub-test 3-2 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  | Sub-test 1-1, Sub-test 2-1 and Sub-test 3-1 |
|  | n4 |  | Sub-test 1-2, Sub-test 2-2 and Sub-test 3-2 |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 ms |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 ms |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.1.4.3-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.6.4.3-6: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Sub-tests 1-1, Sub-test 1-2, Sub-test 2-1 and Sub-test 2-2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Sub-tests 3-1 and Sub–test 3-2 |
| n320-r16 | 20 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  | Sub-tests 1-2, Sub-test 2-2 and Sub-test 3-2 |
|  | n2 |  | Sub-tests 1-1, Sub-test 2-1 and Sub-test 3-1 |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | Sub-tests 1-1, Sub-test 2-1 and Sub-test 3-1 |
| n4-r16 | 0 |  | Sub-tests 1-2, Sub-test 2-2 and Sub-test 3-2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Sub-test 1-1 Cell 1, Sub-test 1-2 Cell 1, Sub-test 2-1 Cell 1, Sub-test 2-2 Cell 1, Sub-test 3-1 Cell 1,and Sub-test 3-2 Cell 1 |
|  | 4 |  | Sub-test 1-1 Cell 2, Sub-test 1-2 Cell 2, Sub-test 2-1 Cell 2, Sub-test 2-2 Cell 2, Sub-test 3-1 Cell 2, and Sub-test 3-2 Cell 2. |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.6.4.3-7: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { | 2 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 | Present |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 | Present |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |
| } |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 14.3.6.5	Test requirement

Table 14.3.6.5-1 define the primary level settings including test tolerances for the test.

Table 14.3.6.5-1: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |  | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1~3 | slot | 0 | 4 |  | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 |  | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |  | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |  | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |  | -61.72 | -61.72 |
| PRS | 1~3 | dB | -5.7 | -11.9 |  | -5.7 | -11.9 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 14.3.6.3.

The value of the LPP Time IE depends on the UE capabilities. The calculation is FFS

Table 14.3.6.5-2: RSTD accuracy requirements for the reported values

| Test Configuration | Lowest reported value | Highest reported value |
| --- | --- | --- |
| Sub-test 1 | FFS | FFS |
| Sub-test 2 | FFS | FFS |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.

< End of changes >