3GPP TSG-RAN5 Meeting #103	R5-242972r1

# Fukuoka City, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0527 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test Case 16.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Case 16.2.5 is missing from the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Case 16.2.5 has been added to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 16.2.5 will remain missing from the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Removed calculations in test requirements, pending TT analysis. Added Editor’s note for Message contents update”. Corrected typos in message contents |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 16.2.5	PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR1 SA

Editor’s Note: This test case has been completed for the following configurations:

- Test tolerance is missing

- Message contents require update

#### 16.2.5.1	Test purpose

The purpose of the test is to verify the PRS RSRP measurement requirements for reduced number of samples specified in TS 38.133 [50] Clause 9.9.3.5 for single positioning frequency layer under AWGN propagation conditions in FR1 in standalone scenario.

#### 16.2.5.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-AoD positioning and supportedDL-PRS-ProcessingSamples-RRC-CONNECTED.

#### 16.2.5.3	Minimum conformance requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and if positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, if not all PRS resources to be measured are available in the same measurement gap occasion during Tavailable_PRS,i, otherwise Tlast,i = Ti + MGL,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

#### 16.2.5.4	Test description

##### 16.2.5.4.1	Initial conditions

The supported test configurations in listed in Table 16.2.5.4-1.

Table 16.2.5.4-1: Test Configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 16.2.5.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.13.

2.	The general test parameter settings are set up according to Table 16.2.5.5-1 and Table 16.2.5.5-2.

3.	Propagation conditions are set according to clause 4.16.2.

4.	Message contents are defined in clause 16.2.5.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is the neighbour cell. All cells are on the same RF channel distributed in single positioning frequency layer.

##### 16.2.5.4.2	Test procedure

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2. All two cells transmit PRS during T2. The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The NR-DL-AoD-ProvideAssistanceData and NR-DL-AoD-RequestLocationInformation as defined in TS 37.355 [49] clause 6.5.11 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-AoD assistance data and location information request.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 16.2.5.5-1 and Table 16.2.5.5-2. Propagation conditions are set according to clause 4.16.2.

4.	T1 starts.

5.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

6.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the DL-AoD capabilities supported by the UE in the NR-DL-AoD-ProvideCapabilities IE.

7.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-AoD-ProvideAssistanceData IE. The position of neighbour Cell 2 is described in 3GPP TS 37.571-5 [20]. If the UE message at step 6 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

8.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the NR-DL-AoD-RequestLocationInformation IE such that the UE receives the message T ms before the start of T2, where T = 50 ms.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.2.5.5-2.

10.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-AoD-ProvideLocationInformation IE within the response time (see clause 4.16.3). The UE shall perform and report the PRS-RSRP measurements for Cell 2 with respect to the reference cell in the DL-TDOA assistance data, Cell 1. If the UE transmits an NR-DL-AoD-ProvideLocationInformation IE including the nr-DL-PRS-RSRP-Result-r16 field for Cell 2 within the response time then the number of successful tests is increased by one. If the UE fails to report the NR-DL-AoD-ProvideLocationInformation IE with both the nr-DL-PRS-RSRP-Result-r16 fields included within the response time then the number of failure tests is increased by one.

11.	If the UE message at step 10 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

12.	Repeat steps 2-11 in Table 16.2.5.4-1 until the confidence level according to Annex D is achieved.

##### 16.2.5.4.3	Message contents

Table 16.2.5.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 1 0 0 0 | DL-AoD |  |

Table 16.2.5.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-AoD-RequestCapabilities-r16 | TRUE |

Table 16.2.5.4.3-3: LPP ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r15 | Not present |  |  |
| tbs-ProvideAssistanceData-r15 | Not present |  |  |
| wlan-ProvideAssistanceData-r15 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 16.2.5.4.3-4 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 SEQUENCE { |  |  | Depending on UE capabilities, i.e. support for UE-based DL-AoD |
| nr-TRP-LocationInfo-r16 | As defined in TS 37.571-5 [20] |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| nr-TRP-BeamAntennaInfo-r17 | Not present |  |  |
| nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 | Not present |  |  |
| nr-DL-PRS-TRP-TEG-Info-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| nr-DL-PRS-BeamInfo-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.2.5.4.3-4: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Config 1 and Config 2 |
|  | kHz30 |  | Config 3 |
| dl-PRS-ResourceBandwidth-r16 | 21 | 104 PRBs | Config 1 and Config 2 |
|  | 28 | 132 PRBs | Config 3 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | N4 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 16.2.5.4.3-5 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | As specified in Table 16.2.5.5-1 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | As specified in Table 16.2.5.5-1 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 16.2.5.4.3-5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.2.5.4.3-5: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Config 1 &2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Config 3 |
| n320-r16 | 20 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n1 |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| N4-r16 | 0 |  | Cell 1 |
|  | 1 |  | Cell 2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.2.5.4.3-6: LPP RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsRequired |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 16.2.5.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s. |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-AoD-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r16 | Not present |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r17 | Not present |  |  |
| maxDL-PRS-RSRPP-MeasurementsPerTRP-r17 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | requested |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| multiMeasInSameReport-r17 |  |  |  |
| } |  |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.2.5.4.3-7: LPP ProvideLocation Information

| Derivation Path: 37.355 clause 6.5.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-AoD-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16 { |  |  |  |
| NR-DL-AoD-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 | Present. Any value acceptable. |  |  |
| nr-DL-PRS-RxBeamIndex-r16 |  |  |  |
| nr-DL-AoD-AdditionalMeasurements-r16 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-DL-AoD-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-AoD-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 | Present. Any value acceptable. |  |  |
| nr-DL-PRS-RxBeamIndex-r16 |  |  |  |
| nr-DL-AoD-AdditionalMeasurements-r16 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-DL-AoD-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-dl-AoD-LocationInformation-r16 |  |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| nr-DL-AoD-SignalMeasurementInstances-r17 | Not present |  |  |
| nr-DL-AoD-LocationInformationInstances-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 16.2.5.5	Test requirement

Table 16.2.5.5-1 and Table 16.2.5.5-2 define the primary level settings including test tolerances for the test.

Table 16.2.5.5-1: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table 16.2.5.5-2: Cell-specific test parameters for PRS RSRP measurement reporting delay

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| Io | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

< End of changes >