3GPP TSG-RAN5 Meeting #103 	R5-243007r2

# Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3194 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to K1 for RRM TDD SCS 15 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no ‘PDSCH-to-HARQ_feedback timing indicator’ for SCS=15kHz in Table 7.2.2.1-1: Physical layer parameters for DCI format 1_1, so K1=7 is used according to Table 4.3.6.1.2.2-1: Physical layer parameters for DCI format 1_1. As a result, some ACK/NACK do not fall in UL slots. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define K1 values as below, ‘PDSCH-to-HARQ_feedback timing indicator’ for TDDConf.1.1 (SCS=15kHz) was added to Table 7.2.2.1-1: Physical layer parameters for DCI format 1_1. - K1 = 2 if mod(i,5) = 0 - K1 = 2 if mod(i,5) = 1 (for SR.1.2) - K1 = 3 if mod(i,5) = 4 where i is slot index per frame; i = {0,…,9} |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH will not be maximized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: K1 definition for mod(i,5) = 1 was added considering SR.1.2 TDD. r2: The condition for added K1 was separated to SR.1.1 TDD and SR.1.2 TDD. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.2.2.1	Downlink physical layer parameters

As defined in clause 4.3.6 with the following exceptions:

Table 7.2.2.1-1: Physical layer parameters for DCI format 1_1

| Derivation Path: Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 0 in pucch-ResourceSetID[1] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “000” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For 120KHz SCS K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} | - | FR2 |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 6 if mod(i,10) = 8 K1 = 5 if mod(i,10) = 0 K1 = 5 if mod(i,10) = 1 K1 = 5 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 9 where i is slot index per frame; i = {0,…,19} |  | TDDConf.2.1 |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For FDD K1 = 2 |  | FDD |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 2 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 4 where i is slot index per frame; i = {0,…,9} |  | TDDConf.1.1 AND SR.1.1 TDD |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 2 if mod(i,5) = 0 K1 = 2 if mod(i,5) = 1 K1 = 3 if mod(i,5) = 4 where i is slot index per frame; i = {0,…,9} |  | TDDConf.1.1 AND SR.1.2 TDD |

| Condition | Explanation |
| --- | --- |
| TDDConf.2.1 | TDD UL/DL configuration for SCS=30kHz |
| TDDConf.1.1 | TDD UL/DL configuration for SCS=15kHz |
| SR.1.1 TDD | PDSCH RMC SR.1.1 TDD defined in TS 38.533 Annex A.1.1 |
| SR.1.2 TDD | PDSCH RMC SR.1.2 TDD defined in TS 38.533 Annex A.1.1 |

Table 7.2.2.1-2: Physical layer parameters for DCI format 1_0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For FDD K1 = 2 |  |

<<End of change>>