3GPP TSG-RAN5 Meeting #103	R5-243177r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0644 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Capability clauses for inter-SN CPC for EN-DC and NR-DC are missing. And inter-MN clauses are referring to wrong chapter in 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding capability clauses. Fixing incorrect references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability cannot be defined. Existing references will be confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-1; 38.523-2 CR 4449, 4450; 0480 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: a) Fixing pnemonics syntax |  |  |  |  |  |  |  |  |

-------- Start of change --------

### A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item |  | UE Mobility Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support inter-RAT Handover to EUTRA connected to EPC |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_interRAT_EUTRA_Handover |  | Yes |  |  |  |  |  |
| 2 |  | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_handoverInterF |  | Yes |  |  |  |  |  |
| 3 |  | Support Handover between FR1 and FR2 |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_FR1toFR2_Handover |  | Yes |  |  |  |  |  |
| 4 |  | Support Handover between FDD and TDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_FDDtoTDD_Handover |  | Yes |  |  |  |  |  |
| 5 |  | Support inter-RAT Handover to E-UTRA connected to 5GC |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_interRAT_eLTE_Handover |  | Yes |  |  |  |  |  |
| 6 |  | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC |  | 36.306, 4.3.34.9 |  | Rel-15 |  | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 |  | Yes |  |  |  |  |  |
| 7 |  | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC |  | 36.306, 4.3.34.8 |  | Rel-15 |  | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 |  | Yes |  |  |  |  |  |
| 8 |  | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC |  | 36.306, 4.3.34.11 |  | Rel-15 |  | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 |  | Yes |  |  |  |  |  |
| 9 |  | Support intra-frequency DAPS handover |  | 38.306, 4.2.7.5 |  | Rel-16 |  | pc_intraFreqDAPS_r16 |  | No |  |  |  | It is mandated if the UE supports asynchronous intra-frequency DAPS handover |  |
| 10 |  | Support inter-RAT Handover from NR to EN-DC |  | 38.306, 4.2.10 |  | Rel-16 |  | pc_interRAT_NR_ToENDC |  | CY |  |  |  | It is mandated if the UE supports EN-DC. |  |
| 11 |  | Support conditional handover |  | 38.306, 4.2.7.2 |  | Rel-16 |  | pc_condHandover_r16 |  | No |  |  |  |  |  |
| 12 |  | Support conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |  | 38.306, 4.2.7.2 |  | Rel-16 |  | pc_condHandoverFailure_r16 |  | No |  |  |  |  |  |
| 13 |  | Support 2 trigger events for same execution condition of conditional handover |  | 38.306, 4.2.7.2 |  | Rel-16 |  | pc_condHandoverTwoTriggerEvents_r16 |  | CY |  |  |  | It is mandated if the UE supports condHandover-r16. |  |
| 14 |  | Support inter-RAT Handover from NR to UTRA-FDD CELL_DCH CS |  | 38.306, 4.2.9 |  | Rel-16 |  | pc_handoverUTRA_FDD_r16 |  | No |  |  |  |  |  |
| 15 |  | Support inter-frequency DAPS handover |  | 38.306, 4.2.7.4 |  | Rel-16 |  | pc_interFreqDAPS_r16 |  | No |  |  |  | It is mandated if the UE supports asynchronous inter-frequency DAPS handover or supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  |
| 16 |  | UE supports asynchronous intra-frequency DAPS handover |  | 38.306, 4.2.7.5 |  | Rel-16 |  | pc_intraFreqAsyncDAPS_r16 |  | No |  |  |  |  |  |
| 17 |  | UE supports asynchronous inter-frequency DAPS handover |  | 38.306, 4.2.7.5 |  | Rel-16 |  | pc_interFreqAsyncDAPS_r16 |  | No |  |  |  |  |  |
| 18 |  | UE supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  | 38.306, 4.2.7.5 |  | Rel-16 |  | pc_inteFreqDiffSCS_DAPS_r16 |  | No |  |  |  |  |  |
| 19 |  | Support conditional PSCell change |  | 38.306, 4.2.7.2 |  | Rel-16 |  | pc_condPSCellChange_r16 |  | No |  |  |  |  |  |
| 20 |  | Support handover from 5GS to EPC/ePDG |  | 24.302, 7.2.2.1 |  | Rel-15 |  | pc_HO_from_5GS_to_EPC_ePDG |  | No |  |  |  |  |  |
| 21 |  | Support handover from EPC/ePDG to 5GS |  | 23.502, 4.11.4.1 |  | Rel-15 |  | pc_HO_from_EPC_ePDG_to_5GS |  | No |  |  |  |  |  |
| 22 |  | Support Handover from EPS to 5GC-N3IWF |  | 23.502, 4.11.3.1 |  | Rel-15 |  | pc_HO_from_EPS_to_5GC_N3IWF |  | No |  |  |  |  |  |
| 23 |  | Support Handover from 5GC-N3IWF to EPS |  | 23.502, 4.11.3.2 |  | Rel-15 |  | pc_HO_from_5GC_N3IWF_to_EPS |  | No |  |  |  |  |  |
| 24 |  | Support Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access |  | 23.502, 4.9.2.1 |  | Rel-15 |  | pc_HO_from_5GC_N3IWF_to_5GC |  | No |  |  |  |  |  |
| 25 |  | Support Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access |  | 23.502, 4.9.2.2 |  | Rel-15 |  | pc_HO_from_5GC_to_5GC_N3IWF |  | No |  |  |  |  |  |
| 26 |  | Supports location based conditional handover, i.e., CondEvent D1 |  | 38.306, 4.2.7.2 |  | Rel-17 |  | pc_locationBasedCondHandover_r17 |  | No |  |  |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |  |
| 27 |  | Support time based conditional handover, i.e., CondEvent T1 |  | 38.306, 4.2.7.2 |  | Rel-17 |  | pc_timeBasedCondHandover_r17 |  | No |  |  |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |  |
| 28 |  | Support of MN initiated conditional PSCell change in NR-DC |  | 38.306, 4.2.7.2 |  | Rel-17 |  | pc_mn_InitiatedCondPSCellChangeNRDC_r17 |  | No |  |  |  | A UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. |  |
| 29 |  | Support of MN initiated conditional PSCell change within all supported FR1-FDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17 |  | No |  |  |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |  |
| 30 |  | Support of MN initiated conditional PSCell change within all supported FR1-TDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17 |  | No |  |  |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |  |
| 31 |  | Support of MN initiated conditional PSCell change within all supported FR2-TDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17 |  | No |  |  |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |  |
| NOK1 |  | Support of SN initiated conditional PSCell change in NR-DC |  | 38.306, 4.2.7.2 |  | Rel-17 |  | pc_sn_InitiatedCondPSCellChangeNRDC_r17 |  | No |  |  |  | Indicates whether the UE supports SN initiated inter-SN conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. |  |
| NOK2 |  | Support of SN initiated conditional PSCell change within all supported FR1-FDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_sn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17 |  | No |  |  |  | Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR1-FDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. |  |
| NOK3 |  | Support of SN initiated conditional PSCell change within all supported FR1-TDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_sn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17 |  | No |  |  |  | Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR1-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. |  |
| NOK4 |  | Support of SN initiated conditional PSCell change within all supported FR2-TDD bands in EN-DC |  | 38.306, 4.2.9a |  | Rel-17 |  | pc_sn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17 |  | No |  |  |  | Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR2-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. |  |

----- End of change ------