Title: 	Test point analysis for Time mask for switching across three uplink bands for SUL with inter-band CA

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Time mask for switching for switching across three bands for SUL with inter-band CA.

- Discussion

The minimum requirement of Time mask for switching between two uplink carriers is extracted as below:

The switching time mask requirements specified in this sub-clause are applicable for an NR SUL band configuration with inter-band CA when the capability supportedBandPairListNR-r18 is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR SUL band configuration with inter-band CA, the number of uplink bands with different carrier frequencies is up to four.  NR UL carrier(s) in each of the up to four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability FeatureSetUplinkPerCC.

The switching time masks in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are applicable to each of the uplink band pairs in the SUL band configuration with inter-band CA, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter switchingOptionConfigForBandPair. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are located in either NR band X or band Y as indicated in RRC signalling uplinkTxSwitchingBandList [7]. For each band pair, the length of uplink switching period X is indicated by UE capability switchingPeriodFor1T when 1Tx-1Tx switching or 1Tx-2Tx switching between the two bands in the band pair is supported and configured, or is indicated with switchingPeriodFor2T when 2Tx-2Tx switching between the two bands in the band pair is supported and configured. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3C.3.5-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3C.3.5-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-1 and 6.3C.3.5-2 in a band pair with switchingOptionConfigForBandPair set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X µs indicated by UE capability uplinkTxSwitchingPeriodForBandPair on any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2, the requirements in Figure 6.3C.3.5-3 to 6.3C.3.5-5 are applicable when dualUL are supported on certain band pair(s) in the SUL band configuration with inter-band CA.

The switching time masks in Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4 are applicable when dualUL is supported for at least two uplink band pairs in the band configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	As baseline UE behaviour, the UE is not required to transmit on any of the three bands during time period T1 located on band X and band Z, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-4.

–	As optional UE behaviours, when the UE indicates band Z in the capability bandIndexUnaffected,

–	if the UE indicates maintainedUL-Trans for band pair of {band X and band Y}, UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-3;

–	otherwise, theUE is not required to transmit on any of the three bands during the switching period indicated by UE capability periodOnULBands [located on band X and band Z, as shown in Figure 6.3C.3.5-4.

In Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3C.3.5-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-3 in a band pair with switchingOptionConfigForBandPair set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching periods indicated by UE capability uplinkTxSwitchingPeriodForBandPair on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by uplinkTxSwitchingBandList is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3C.3.5-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3C.3.5-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3C.3.5-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour when UE additionally reports band pair of {band X and band Y} and band Z in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List, UE is not required to transmit on any of the three bands during time period indicated by UE capability switchingAdditionalPeriodDualUL.

In Figure 6.3C.3.5-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [7], and band Z is with the highest priority according to the RRC configuration uplinkTxSwitchingBandList.

Figure 6.3C.3.5-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3C.3.5-5 and with uplinkTxSwitchingOptionForBandPair set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by UE capability uplinkTxSwitchingPeriodForBandPair on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before T0 on any switched-to and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## 2.1	Test Environment

In NR test case General ON/OFF Time Mask measurement for SUL in FR1, test environment are defined as Normal, TL/VL, TL/VH, TH/VL, TH/VH. However this test case is only applicable to UEs which support uplink carrier switching but not to all UEs. So only testing under normal condition is enough to verifying the UE capability and meanwhile avoid costing too much time. And relative power control is used in test procedure. The requirements on power control accuracy apply under normal conditions. So in Time Mask for switching across three uplink bands for SUL with inter-band CA, only Normal condition is tested.

Proposal 1: Define Test Environment as Normal

## 2.2	Test Frequencies

In NR, General ON/OFF Time Mask for SUL is tested for Low, Mid and High range frequencies for SUL carrier. Considering the minimum requirement of Time Mask for switching across three uplink bands for SUL with inter-band CA is independent with test frequency range, to reduce the cost of testing time, selecting Mid range will be enough.

Proposal 2: Selecting Mid range for all CCs in Time Mask for switching across three uplink bands testing

## 2.3	Test Channel Bandwidths

In NR, General ON/OFF Time Mask for SUL is tested for Lowest, Mid and Highest channel bandwidth for SUL carrier based on the same selection in single-carrier test case. And Mid channel bandwidth is selected for NUL carrier.

In this test case, uplink is scheduled on NUL carrier and SUL carrier in different slot. And generally we consider highest channel bandwidth to be the worst case because of possible frequency selectivity and BW limitation issues. So testing Highest channel bandwidth for both NUL and SUL carrier will be enough to reduce testing time without losing test coverage.

Proposal 3: Select highest channel bandwidth for NUL carrier and SUL carrier

## 2.4	Test Subcarrier Spacing

For SUL test case, 15kHz for SUL carrier and lowest supported SCS for NUL carrier should be selected.

Proposal 4: Select 15kHz SCS for SUL carrier and Lowest supported SCS for Non-SUL carrier.

## 2.5	Uplink Modulation and RB allocation

In Time Mask for switching across three uplink bands testing, since 2-layer UL-MIMO would be scheduled on the carrier supporting 2Tx, CP-OFDM is chosen, with QPSK + Inner Full RB allocation to provide minimum MPR. In Time mask for switching between two uplink carriers testing, the same approach should be selected.

Proposal 5: Use CP-OFDM QPSK + Inner Full RB allocation for Time mask for switching across three uplink bands testing

## 2.6	Number of Test Points

Based on above proposals, the total test points of Time mask for switching between two uplink carriers testing are tabulated below.

Table 2.6-1 Total test points of Time mask for switching between two uplink carriers testing

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

## 2.7	Conclusion

Proposal 1: Define Test Environment as Normal

Proposal 2: Selecting Mid range for all CCs in Time Mask for switching across three uplink bands testing

Proposal 3: Select highest channel bandwidth for NUL carrier and SUL carrier

Proposal 4: Select 15kHz SCS for SUL carrier and Lowest supported SCS for Non-SUL carrier.

Proposal 5: Use CP-OFDM QPSK + Inner Full RB allocation for Time mask for switching across three uplink bands testing

- References

[1] 3GPP TS 38.521-1 V18.2.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 18)

[2] 	3GPP TS 38.101-1 v18.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)

[3]	R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.