3GPP TSG-RAN5 Meeting #103	R5-243218r1

# Fukuoka, Japan,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0341 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability for new test cases 12.23a and 12.24a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Two new test cases,12.23a and 12.24a, have been added. The corresponding applicability statements needs to be added as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability statements for the new test cases,12.23a and 12.24a, have been added. A new contidtion was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no applicability statements for test cases 12.23a and 12.24a |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Added note stating that if test cases 12.23a/12.24a are executed, test cases 12.23/12.24 are optional. |  |  |  |  |  |  |  |  |

## 4.1	Applicabilities for test cases specified in TS 34.229-1

Table 4.1: Applicability of TS 34.229-1 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| 6 | PDP Context Activation |  |  |  |
| 6.2 | General Purpose PDP Context Establishment (UE Requests for a Dedicated PDP Context) | Rel-8 | C04 | UE capable of being configured to initiate Dedicated PDP Context |
| 6.3 | Dedicated PDP Context Establishment | Rel-8 | C04 | UE capable of being configured to initiate Dedicated PDP Context |
| 7 | P-CSCF Discovery |  |  |  |
| 7.1 | P-CSCF Discovery via PDP Context | Rel-8 | C05 | UE capable of being configured to initiate P-CSCF Discovery via PCO |
| 7.2 | P-CSCF Discovery via DHCP - IPv4 | Rel-8 | C06 | UE supports IPv4 and capable of being configured to initiate P-CSCF Discovery via DHCPv4 |
| 7.3 | P-CSCF Discovery via DHCP - IPv4 (UE Requests P-CSCF discovery via PCO) | Rel-8 | C07 | UE supports IPv4, supports P-CSCF Discovery via PCO and DHCPv4 and capable of being configured to initiate P-CSCF Discovery via PCO |
| 7.4 | P-CSCF Discovery by DHCP - IPv6 | Rel-8 | C08 | UE capable of being configured to initiate P-CSCF Discovery via DHCPv6 |
| 7.5 | P-CSCF Discovery by DHCP-IPv6 (UE Requests P-CSCF discovery by PCO) | Rel-8 | C09 | UE supports P-CSCF Discovery via PCO and DHCPv6 and capable of being configured to initiate P-CSCF Discovery via PCO |
| 7.6 | P-CSCF Discovery by DHCP - IPv6 (UE does not Request P-CSCF discovery by PCO, SS includes P-CSCF Address(es) in PCO) | Rel-8 | C10 | UE supports P-CSCF Discovery via PCO and DHCPv6 and capable of being configured to initiate P-CSCF Discovery via DHCPv6 |
| 7.7 | Void |  |  |  |
| 7.8 | Void |  |  |  |
| 7.9 | Void |  |  |  |
| 8 | Registration |  |  |  |
| 8.1 | Initial registration | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 8.2 | User Initiated Re-Registration | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 8.3 | Mobile Initiated Deregistration | Rel-8 | C80 | UE supports IMS security and IMS deregistration and E-UTRA and (Support of CS/PS mode 2 or Support of PS mode 2) and not UE category M1 |
| 8.4 | Invalid Behaviour - 423 Interval Too Brief | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 8.5 | Void |  |  |  |
| 8.6 | Void |  |  |  |
| 8.7 | Void |  |  |  |
| 8.8 | Void |  |  |  |
| 8.9 | Void |  |  |  |
| 8.10 | Initial registration using GIBA | Rel-8 | C18 | UE supports GIBA only and E-UTRA and not UE category M1 |
| 8.11 | Initial registration using IMS AKA and GIBA against a network with GIBA support only | Rel-8 | C19 | UE supports IMS security and GIBA and E-UTRA and not UE category M1 |
| 8.12 | User initiated re-registration using GIBA | Rel-8 | C18 | UE supports GIBA only and E-UTRA and not UE category M1 |
| 8.13 | User initiated de-registration using GIBA | Rel-8 | C18 | UE supports GIBA only and E-UTRA and not UE category M1 |
| 8.14 | Void |  |  |  |
| 8.15 | Refresh for ISIM parameters | Rel-10 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 8.16 | User initiated re-registration- 423 Interval Too Brief | Rel-9 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 9 | Authentication |  |  |  |
| 9.1 | Invalid Behaviour - MAC Parameter Invalid | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 9.2 | Invalid Behaviour - SQN out of range | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 10 | Subscription |  |  |  |
| 10.1 | Invalid Behaviour - 503 Service Unavailable | Rel-8 | C151 | E-UTRA and not UE category M1 |
| 11 | Notification |  |  |  |
| 11.1 | Network-initiated deregistration | Rel-8 | C151 | E-UTRA and not UE category M1 |
| 11.2 | Network initiated re-authentication | Rel-8 | C17 | UE supports IMS security and E-UTRA and not UE category M1 |
| 12 | Call Control |  |  |  |
| 12.1 | Void |  |  |  |
| 12.2 | MO Call with preconditions at both originating UE and terminating UE - 503 Service Unavailable | Rel-8 | C160 | UE supports MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| 12.2a | MO Call with preconditions at both originating and terminating UE - 504 Server Time-out | Rel-8 | C160 | UE supports MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| 12.2b | MO Call without preconditions at both originating UE and terminating UE - 503 Service Unavailable | Rel-8 | C164 | UE supports MTSI and MTSI speech and initiating a session and no preconditions and E-UTRA and not UE category M1 |
| 12.2c | MO Call without preconditions at both originating and terminating UE - 504 Server Time-out | Rel-8 | C164 | UE supports MTSI and MTSI speech and initiating a session and no preconditions and E-UTRA and not UE category M1 |
| 12.3 | Void |  |  |  |
| 12.4 | Void |  |  |  |
| 12.5 | Void |  |  |  |
| 12.6 | Void |  |  |  |
| 12.7 | Void |  |  |  |
| 12.8 | Void |  |  |  |
| 12.9 | Void |  |  |  |
| 12.10 | Void |  |  |  |
| 12.11 | Void |  |  |  |
| 12.12 | MO MTSI Voice Call Successful with preconditions at both originating UE and terminating UE | Rel-8 | C160 | UE supports MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| 12.12a | MO MTSI Voice Call Successful without preconditions at both originating UE and terminating UE | Rel-14 | C164 | UE supports MTSI and MTSI speech and initiating a session and not preconditions and E-UTRA and not UE category M1 |
| 12.13 | MT MTSI Speech call | Rel-8 | C27 | UE supports MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 |
| 12.13a | MT MTSI speech cal with preconditions at both originating UE and terminating UEl when MO reserves resources before sending INVITE | Rel-8 | C27 | UE supports MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 |
| 12.13b | MT MTSI Speech call Successful without preconditions at both originating UE and terminating UE | Rel-8 | C165 | UE supports MTSI and MTSI speech and E-UTRA and not preconditions and not UE category M1 |
| 12.13c | MT MTSI speech call Successful with preconditions at originating UE and without preconditions at terminating UE | Rel-14 | C165 | UE supports MTSI and MTSI speech and E-UTRA and not preconditions and not UE category M1 |
| 12.15 | Void |  |  |  |
| 12.16 | Void |  |  |  |
| 12.17 | Void |  |  |  |
| 12.18 | Void |  |  |  |
| 12.18a | Void |  |  |  |
| 12.18b | Void |  |  |  |
| 12.19 | Void |  |  |  |
| 12.19a | Void |  |  |  |
| 12.19b | Void |  |  |  |
| 12.20 | Void |  |  |  |
| 12.20a | Void |  |  |  |
| 12.21 | MO MTSI Video call | Rel-8 | C71 | UE supports MTSI and initiating a session and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| 12.22 | MT MTSI Video call | Rel-8 | C70 | UE supports MTSI and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| 12.23 | MO MTSI speech call / EVS | Rel-12 | C83 | UE supports MTSI and MTSI speech EVS (see NOTE 3 below) and preconditions and E-UTRA and not UE category M1 (NOTE 8). |
| 12.23a | MO MTSI speech call / EVS / Default configuration | Rel-15 | Cxx | UE supports MTSI and MTSI speech and preconditions and EN-DC |
| 12.24 | MT MTSI speech call / EVS | Rel-12 | C83 | UE supports MTSI and MTSI speech EVS (see NOTE 3 below) and preconditions and E-UTRA and not UE category M1 (NOTE 8). |
| 12.24a | MT MTSI speech call / EVS / Default configuration | Rel-15 | Cxx | UE supports MTSI and MTSI speech and preconditions and EN-DC |
| 12.25 | MO MTSI speech call / EVS offered but not supported by remote UE / AMR-WB agreed | Rel-12 | C83 | UE supports MTSI and MTSI speech EVS (see NOTE 3 below) and preconditions and E-UTRA and not UE category M1 |
| 12.25a | MT MTSI speech call / EVS offered but not supported by UE / AMR-WB agreed | Rel-8 | C167 | MTSI and MTSI speech and MTSI speech, AMR wideband and not MTSI speech, EVS and preconditions and E-UTRA and not UE category M1 |
| 12.26 | MT MTSI speech call / EVS / AMR-WB IO mode | Rel-12 | C83 | UE supports MTSI and MTSI speech EVS (see NOTE 3 below) and preconditions and E-UTRA and not UE category M1 |
| 12.27 | MO MTSI speech call / SRVCC on MT side / Codec Change from AMR-WB to AMR-NB | Rel-8 | C160 | UE supports MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| 12.27a | MO MTSI speech call / SRVCC on MT side / Codec Change from EVS to AMR-NB | Rel-12 | C83 | UE supports MTSI and MTSI speech EVS (see NOTE 3 below) and preconditions and E-UTRA and not UE category M1 |
| 12.28 | MO MTSI speech call / MO UE cancels call establishment | Rel-8 | C160 | UE supports MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| 12.29 | MO MTSI Video call / MO UE cancels call establishment | Rel-8 | C71 | UE supports MTSI and initiating a session and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| 13 | SIP Compression (SigComp) |  |  |  |
| <<< ROWS SKIPPED >>> |  |  |  |  |

NOTE 1:	Applicability of test cases 13.2 and 13.3 are currently marked as FFS. The reason to this is that the contents of the specific messages sent by the SS (as currently specified within those Call Control test cases) do not match the contents of those messages as expected by any specific IMS application known. Further on the test specification apparently lacks support for certain application specific message exchanges which are however mandatory for a few specific IMS applications specified outside of TS 24.229. It is necessary to fully resolve the problem (by e.g. defining the applications for which the Call Control test cases would be applicable, possibly specifying the extensions to the test cases like required by those applications and creating the corresponding application profiles) before the applicability statements of Call Control test cases can be unambiguously defined.

NOTE 2:	Either one of the two adjacent test cases, i.e. (19.1.3 or 19.1.3a), (19.3.2 or 19.3.2a), shall be executed.

NOTE 3:	This TC can optionally be executed against Rel-8 and onwards UEs.

NOTE 4:	In case the UE supports both GBA and HTTP Digest, GBA shall be used as XCAP authentication.

NOTE 5:	In case the UE supports both GBA and HTTP Digest, HTTP Digest shall be used as XCAP authentication.

NOTE 6:	The TC contains multi-RAT branches not all mandatory in the scope of the TC. The E-UTRA/EPC branch will be executed always; the TC will go through any other RAT branch depending on the UE capability. Execution only of the E-UTRA/EPC branch regardless of the UE capabilities can also be imposed by setting the IXIT px_RATComb_Tested = EUTRA_only. For UEs supporting both UTRA and GERAN the TC should be executed once only for the E-UTRA/EPC and UTRA combination by setting the px_RATComb_Tested = EUTRA_UTRA.

NOTE 7:	This TC can optionally be executed against Rel-9 and onwards UEs.

NOTE 8:	If TC 12.23a is executed TC 12.23 is optional. If TC 12.24a is executed TC 12.24 is optional.

## <<< TEXT SKIPPED >>>

Table 4.1a: Applicability Conditions for TS 34.229-1 tests

| Condition | Test Case Selection Expression | Comment |
| --- | --- | --- |
| C00 | Void |  |
| C01 | Void |  |
| C02 | Void |  |
| C03 | Void |  |
| C04 | IF A.12/4 THEN R ELSE N/A | Dedicated PDP Context |
| C05 | IF A.12/5 THEN R ELSE N/A | P-CSCF Discovery via PCO |
| C06 | IF A.7/1 AND A.13/1 THEN R ELSE N/A | IPv4 and configured to initiate P-CSCF discovery via DHCPv4 |
| C07 | IF A.7/1 AND A.12/8 AND A.13/2 AND A.12/5 THEN R ELSE N/A | IPv4 and P-CSCF discovery via PCO and P-CSCF discovery via DHCPv4 and configured to initiate P-CSCF discovery via PCO |
| C08 | IF A.12/7 THEN R ELSE N/A | Configured to initiate P-CSCF discovery via DHCPv6 |
| C09 | IF A.12/8 AND A.12/10 AND A.12/5 THEN R ELSE N/A | P-CSCF Discovery via PCO and P-CSCF discovery via DHCPv6 and configured to initiate P-CSCF discovery via PCO |
| C10 | IF A.12/8 AND A.12/10 AND A.12/7 THEN R ELSE N/A | P-CSCF Discovery via PCO and P-CSCF discovery via DHCPv6 and configured to initiate P-CSCF discovery via DHCPv6 |
| C11 | Void |  |
| C12 | Void |  |
| C13 | Void |  |
| C14 | Void |  |
| C15 | Void |  |
| C16 | Void |  |
| C17 | IF A.6a/2 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS security and E-UTRA and not UE category M1 |
| C18 | IF A.6a/1 AND NOT A.6a/2 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | GIBA and not IMS security and E-UTRA and not UE category M1 |
| C19 | IF A.6a/2 AND A.6a/1 THEN R ELSE N/A | IMS security and GIBA |
| C20 | Void |  |
| C21 | Void |  |
| C22 | Void |  |
| C23 | IF A.3A/50 AND A.15/1 AND A.16/6 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Hold and preconditions and E-UTRA and not UE category M1 |
| C24 | IF A.3A/50 AND A.16/7 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and Incoming Communication Barring and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C25 | Void |  |
| C26 | Void |  |
| C27 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 |
| C28 | IF A.3A/50 AND A.4/2B AND A.15/1 AND A.15/2 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI speech, AMR wideband and preconditions and E-UTRA and not UE category M1 |
| C29 | IF A.3A/50 AND A.15/1 AND A.16/11 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Explicit Communication Transfer - consultative transfer and preconditions and E-UTRA and not UE category M1 |
| C30 | IF A.3A/50 AND A.16/5 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Communication Diversion and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C31 | IF A.3A/50 AND A.15/1 AND A.16/5 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Diversion and preconditions and E-UTRA and not UE category M1 |
| C32 | IF A.3A/50 AND A.15/1 AND A.16/9 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Conference and preconditions and E-UTRA and not UE category M1 |
| C33 | Void |  |
| C34 | Void |  |
| C35 | Void |  |
| C37 | Void |  |
| C38 | Void |  |
| C39 | Void |  |
| C40 | Void |  |
| C41 | Void |  |
| C42 | Void |  |
| C43 | IF A.3A/50 AND A.16/1 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Originating Identification Presentation and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C44 | IF A.3A/50 AND A.16/2 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Originating Identification Restriction and preconditions and E-UTRA and not UE category M1 |
| C45 | IF A.3A/50 AND A.16/7 AND A.16/12 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Communication Barring and MTSI Anonymous Communication Rejection and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C46 | Void |  |
| C47 | Void |  |
| C48 | IF A.3A/50 AND A.16/3 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and Terminating Identification Presentation and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C49 | IF A.3A/50 AND A.16/4 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Restriction and preconditions and E-UTRA and not UE category M1 |
| C50 | IF A.3A/50 AND A.16/8 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Message Waiting Indication and E-UTRA and not UE category M1 |
| C51 | Void |  |
| C52 | Void |  |
| C53 | Void |  |
| C54 | Void |  |
| C55 | IF A.3A/61 AND A.18/1 AND [73] A.4.4-2/32 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP sender and E-UTRA and UE configured to use SMS over IP and UE is not category M1 |
| C56 | IF A.3A/62 AND A.18/1 AND [73] A.4.4-2/32 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP receiver and E-UTRA and UE configured to use SMS over IP and UE is not category M1 |
| C57 | IF A.3A/50 AND A.15/1 AND A.16/13 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI communication waiting and preconditions and E-UTRA and not UE category M1 |
| C58 | IF A.6a/2 AND A.8/5 THEN R ELSE N/A | IMS security and Indicate Sigcomp |
| C59 | IF [73] A.4.2.1.1-1/4 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and E-UTRA and not UE category M1 |
| C60 | Void |  |
| C61 | IF A.3A/50 AND A.16/9 AND A.16/14 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Conference and MTSI three way session and preconditions and E-UTRA and not UE category M1 |
| C62 | IF [73] A.4.2.1.1-1/4 AND [34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7) THEN R ELSE N/A | EPS IMS emergency call and emergency speech call and (UTRAN or GERAN) |
| C63 | Void |  |
| C64 | IF [73] A.4.2.1.1-1/4 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | EPS IMS emergency call and E-UTRA and not UE category M1 |
| C65 | Void |  |
| C66 | IF [73] A.4.2.1.1-1/4 AND A.12/12 AND A.15/1 AND [34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7) THEN R ELSE N/A | EPS IMS emergency call and initiating bidirectional voice session over IMS and MTSI speech and emergency call and (UTRAN or GERAN) |
| C67 | Void |  |
| C68 | Void |  |
| C69 | Void |  |
| C70 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI Video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| C71 | IF A.3A/50 AND A.4/2B AND A.15/1 AND A.15/3 AND A.15/9 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and initiating a session and MTSI speech and MTSI video and MTSI Video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| C72 | IF [73] A.4.2.1.1-1/4 AND A.3A/50 AND A.15/1 AND A.12/33 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and MTSI and MTSI speech and MTSI Communication Hold during emergency call and preconditions and E-UTRA and not UE category M1 |
| C73 | IF [73] A.4.2.1.1-1/4 AND A.3A/50 AND A.15/1 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | EPS IMS emergency call and MTSI and MTSI speech and E-UTRA and not UE category M1 |
| C74 | IF [73] A.4.2.1.1-1/4 AND [3] A.2/2 AND [73] A.4.1-1/4 THEN R ELSE N/A | EPS IMS emergency call and emergency speech call and 1xRTT |
| C75 | IF A.12/12 AND A.15/1 AND [3] A.2/2 AND [73] A.4.1-1/4 THEN R ELSE N/A | initiating bidirectional voice session over IMS and MTSI Speech and IMS emergency call and emergency speech call and 1xRTT |
| C76 | IF [73] A.4.2.1.1-1/4 AND A.12/27 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | EPS IMS emergency call and capable of obtaining location Information and E-UTRA and not UE category M1 |
| C77 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.16/6 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI Video H.264 CBP Level 1.2 and MTSI Communication Hold and preconditions and E-UTRA and not UE category M1 |
| C78 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.16/9 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video H.264 CBP Level 1.2 and MTSI Conference and preconditions and E-UTRA and not UE category M1 |
| C79 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.16/9 AND A.16/14 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and MTSI Conference and MTSI three way session and preconditions and E-UTRA and not UE category M1 |
| C80 | IF A.6a/2 AND A.12/39 AND A.18/1 AND ([73] A.4.4-2/5 OR [73] A.4.4-2/8) AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS security and IMS deregistration and E-UTRA and (Support of CS/PS mode 2 or Support of PS mode 2) and not UE category M1 |
| C81 | IF A.3A/50 AND A.16/1 AND A.6a/3 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication. and E-UTRA and not UE category M1 |
| C82 | IF A.3A/50 AND A.15/1 AND A.12/45 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and early media and preconditions and E-UTRA and not UE category M1 |
| C83 | IF A.3A/50 AND A.15/10 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech EVS and preconditions and E-UTRA and not UE category M1 |
| C84 | IF A.3A/50 AND A.16/15 AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Outgoing Communication Barring and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C85 | Void |  |
| C86 | Void |  |
| C87 | Void |  |
| C88 | IF A.3A/50 AND A.16/2A AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Originating Identification Restriction Configuration and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C89 | IF A.3A/50 AND A.16/4A AND (A.6a/3 OR A.6a/4) AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Restriction - Configuration and (GBA or HTTP Digest) and E-UTRA and not UE category M1 |
| C90 | IF A.6a/5 AND A.18/3 THEN R ELSE N/A | SIP Digest without TLS and Fixed Broadband |
| C91 | IF A.6a/5 AND A.18/3 AND A.3A/50 AND NOT A.4/16 AND A.15/1 THEN R ELSE N/A | MTSI and MTSI speech and SIP Digest without TLS and no preconditions and Fixed Broadband |
| C92 | IF A.3A/50 AND A.16/1 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Originating Identification Presentation and WLAN |
| C93 | IF A.3A/50 AND A.16/2 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Originating Identification Restriction and preconditions and WLAN |
| C94 | IF A.3A/50 AND A.16/3 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Presentation and WLAN |
| C95 | IF A.3A/50 AND A.16/4 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Restriction and preconditions and WLAN |
| C96 | IF A.3A/50 AND A.16/1 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Originating Identification Presentation and Fixed Broadband |
| C97 | IF A.3A/50 AND A.16/2 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Originating Identification Restriction and Fixed Broadband |
| C98 | IF A.3A/50 AND A.16/3 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Presentation and Fixed Broadband |
| C99 | IF A.3A/50 AND A.16/4 AND NOT A.4/16 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Restriction and no preconditions and Fixed Broadband |
| C100 | IF A.6a/5 AND A.18/3 AND A.12/39 THEN R ELSE N/A | SIP Digest without TLS and Fixed Broadband and IMS deregistration |
| C101 | IF A.3A/50 AND A.16/5 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Communication Diversion and WLAN |
| C102 | IF A.3A/50 AND A.15/1 AND A.16/5 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Diversion and preconditions and WLAN |
| C103 | IF A.3A/50 AND A.16/5 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Communication Diversion and Fixed Broadband |
| C104 | IF A.3A/50 AND A.15/1 AND A.16/5 AND A.18/3 AND NOT A.4/16 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Diversion and Fixed Broadband and no preconditions |
| C105 | IF A.3A/50 AND A.6a/5 AND A.18/3 AND A.15/1 AND NOT A.4/16 THEN R ELSE N/A | MTSI and MTSI speech and SIP Digest without TLS and Fixed Broadband and no preconditions |
| C106 | IF A.3A/50 AND A.4/2B AND A.15/1 AND A.15/3 AND A.15/9 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and initiating a session and MTSI speech and MTSI video and MTSI Video H.264 CBP Level 1.2 and preconditions and WLAN |
| C107 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI Video H.264 CBP Level 1.2 and preconditions and WLAN |
| C108 | IF A.3A/50 AND A.18/4 THEN R ELSE N/A | MTSI and WLAN |
| C109 | IF A.3A/50 AND A.15/1 AND A.16/6 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Hold and preconditions and WLAN |
| C110 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.16/6 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI Communication Hold and preconditions and WLAN |
| C111 | IF A.3A/50 AND A.16/7 AND A.18/4 AND (A.6a/3 OR A.6a/4) THEN R ELSE N/A | MTSI and MTSI Incoming Communication Barring and WLAN and (GBA or HTTP Digest) |
| C112 | IF A.3A/50 AND A.16/15 AND A.18/4 AND (A.6a/3 OR A.6a/4) THEN R ELSE N/A | MTSI and MTSI Outgoing Communication Barring and WLAN and (GBA or HTTP Digest) |
| C113 | IF A.3A/50 AND A.16/8 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI Message Waiting Indication and WLAN |
| C114 | IF A.3A/50 AND A.15/1 AND A.16/9 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Conference and preconditions and WLAN |
| C115 | Void |  |
| C116 | IF A.3A/50 AND A.15/1 AND A.16/9 AND A.16/14 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Conference and MTSI three way session and preconditions and WLAN |
| C117 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.16/9 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and MTSI Conference and preconditions and WLAN |
| C118 | Void |  |
| C119 | IF A.3A/50 AND A.15/1 AND A.15/3 AND A.15/9 AND A.16/9 AND A.16/14 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and MTSI Conference and MTSI three way session and preconditions and WLAN |
| C120 | IF A.3A/50 AND A.15/1 AND A.16/13 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI communication waiting and preconditions and WLAN |
| C121 | IF A.12/65 AND A.18/4 THEN R ELSE N/A | IMS emergency call over WLAN and WLAN |
| C122 | IF A.3A/50 AND A.6a/5 AND A.18/3 AND A.15/1 AND A.15/3 AND NOT A.4/16 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and SIP Digest without TLS and Fixed Broadband and no preconditions |
| C123 | IF A.3A/50 AND A.16/5 AND A.16/18 AND A.18/3 THEN R ELSE N/A | MTSI and MTSI Communication Diversion and SIP-based configuration and Fixed Broadband |
| C124 | IF A.6a/2 AND A.6a/1 AND [73] A.4.2.1.1-1/4 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS security and GIBA and EPS IMS emergency call and E-UTRA and not UE category M1 |
| C125 | IF [73] A.4.2.1.1-1/4 AND A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and IMS emergency call without registration after rejection of emergency registration and E-UTRA and not UE category M1 |
| C126 | IF A.3A/50 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and preconditions and UE category M1 |
| C127 | IF A.3A/50 AND A.16/13 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI communication waiting and preconditions and UE category M1 |
| C128 | IF A.3A/50 AND A.16/8 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Message Waiting Indication and UE category M1 |
| C129 | IF A.3A/50 AND A.16/2 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Originating Identification Restriction and preconditions and UE category M1 |
| C130 | IF A.3A/50 AND A.16/4 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Terminating Identification Restriction and preconditions and UE category M1 |
| C131 | IF A.3A/50 AND A.16/5 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI Communication Diversion and preconditions and UE category M1 |
| C132 | IF [73] A.4.2.1.1-1/4 AND A.12/27 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and capable of obtaining location Information and UE category M1 |
| C133 | IF [73] A.4.2.1.1-1/4 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and UE category M1 |
| C134 | IF A.6a/2 AND NOT A.6a/1 AND [73] A.4.2.1.1-1/4 AND A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | IMS security and NOT GIBA and EPS IMS emergency call and IMS emergency call without registration after rejection of emergency registration. UE supports E-UTRA and not UE category M1 |
| C135 | IF A.20/1 THEN R ELSE N/A | Combined Registration E-UTRA |
| C136 | IF A.20/2 THEN R ELSE N/A | Dual Registration E-UTRA |
| C137 | IF A.20/3 THEN R ELSE N/A | Combined Registration WLAN |
| C138 | IF A.20/4 THEN R ELSE N/A | Dual Registration WLAN |
| C139 | IF A.20/5 THEN R ELSE N/A | Voice Call after combined registration E-UTRA |
| C140 | IF A.20/6 THEN R ELSE N/A | Voice call after dual registration E-UTRA |
| C141 | IF A.20/7 THEN R ELSE N/A | Voice Call after combined registration WLAN |
| C142 | IF A.20/8 THEN R ELSE N/A | Voice call after dual registration WLAN |
| C143 | IF A.20/9 THEN R ELSE N/A | RCS chat after combined registration E-UTRA |
| C144 | IF A.20/10 THEN R ELSE N/A | RCS chat after dual registration E-UTRA |
| C145 | IF A.20/11 THEN R ELSE N/A | RCS chat after combined registration WLAN |
| C146 | IF A.20/12 THEN R ELSE N/A | RCS chat after dual registration WLAN |
| C147 | IF A.12/54 AND [3] A.10/16 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and manual type of eCall |
| C148 | IF A.12/54 AND [3] A.10/17 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and automatic type of eCall |
| C149 | IF A.12/55 AND [3] A.10/16 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and manual type of eCall |
| C150 | IF A.12/55 AND [3] A.10/17 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and automatic type of eCall |
| C151 | IF A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | E-UTRA and not UE category M1 |
| C152 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/4 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and WLAN |
| C153 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/4 AND A.15/3 THEN R ELSE N/A | MTSI and MTSI speech and MTSI video and preconditions and WLAN |
| C154 | IF A.3A/61 AND [73] A.4.4-2/32 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP sender and UE configured to use SMS over IP and UE is category M1 |
| C155 | IF A.3A/62 AND [73] A.4.4-2/32 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | SM-over-IP receiver and UE configured to use SMS over IP and UE is category M1 |
| C156 | Void | - |
| C157 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND A.12/57 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 and Session Timer |
| C158 | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND A.12/57 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 and Session Timer |
| C159 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.18/1 AND A.16/19 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | UE supports MTSI and MTSI speech and initiating a session and E-UTRA and USSI and not UE category M1 |
| C160 | IF A.3A/50 AND A.15/1 AND A.4/2B AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and preconditions and E-UTRA and not UE category M1 |
| C161 | IF A.6a/5 AND A.18/3 AND A.3A/50 AND A.4/16 AND A.15/1 THEN R ELSE N/A | MTSI and MTSI speech and SIP Digest without TLS and preconditions and Fixed Broadband |
| C162 | IF [73] A.4.2.1.1-1/4 AND A.18/1 AND [3] A.20/90 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | EPS IMS emergency call and E-UTRA and no USIM test execution and not UE category M1 |
| C163 | IF A.3A/50 AND A.4/16 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and preconditions and UE category M1 |
| C164 | IF A.3A/50 AND A.15/1 AND A.4/2B AND NOT A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and initiating a session and no preconditions and E-UTRA and not UE category M1 |
| C165 | IF A.3A/50 AND A.15/1 AND A.18/1 AND NOT A.4/16 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | UE supports MTSI and MTSI speech and E-UTRA and no preconditions and not UE category M1 |
| C166 | Void |  |
| C167 | IF A.3A/50 AND A.15/1 AND A.15/2 AND NOT A.15/10 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | MTSI and MTSI speech and MTSI speech, AMR wideband and not MTSI speech, EVS and preconditions and E-UTRA and not UE category M1 |
| C168 | IF A.3A/64 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | Concatenated SM-over-IP sender and E-UTRA and UE is not category M1 |
| C169 | IF A.3A/65 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | Concatenated SM-over-IP receiver and E-UTRA and UE is not category M1 |
| Cxx | IF A.3A/50 AND A.15/1 AND A.4/16 AND [93] A.4.1-3/2 THEN R ELSE N/A | UE supports MTSI and MTSI speech and preconditions and EN-DC |