3GPP TSG-RAN5 Meeting #103	R5-243268r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0060 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.5.2 Location Report triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no existing test case for MCData Location Report Triggering |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case On-network / Location / Report triggering |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices could fail in the commercial field without testing the capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: In the test procedures, for consistency with the rest of the test cases, "EXCEPTION: SS releases the E-UTRA connection" was replaced by "The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection." The step numbers were adjusted accordingly. |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.5.2	On-network / Location / Report triggering

6.5.2.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message providing configuration parameters for NonEmergencyLocationInformation requesting indication of ESGIs and location information, and, TriggeringCriteria set to “CellChange”, and, a none zero minimumIntervalLength timer }

ensure that {

when { UE moves to a different cell }

then { UE (MCData Client) sends location report obeying the set in the location configuration parameters for NonEmergencyLocationInformation including waiting for the expiry of minimumReportInterval timer for before reporting }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message providing configuration parameters for NonEmergencyLocationInformation requesting indication of ESGIs and location information, and, TriggeringCriteria including “PlmnChange” and “PeriodicReport”, and, a none zero minimumIntervalLength timer }

ensure that {

when { UE moves to a cell belonging to a different PLMN however allowed for communication }

then { UE (MCData Client) sends location report obeying the set in the location configuration parameters for NonEmergencyLocationInformation and resets all triggers }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message providing configuration parameters for NonEmergencyLocationInformation requesting indication of ESGIs and location information, and, TriggeringCriteria including “PlmnChange” and “PeriodicReport”, and, a none zero minimumIntervalLength timer }

ensure that {

when { when the PeriodicReport trigger fires }

then { UE (MCData Client) sends location report obeying the set in the location configuration parameters for NonEmergencyLocationInformation }

}

6.5.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 17.3.1, 17.3.2, 17.3.3, 17.3.4.1, 17.3.4.2. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 17.3.1]

The MCData client sends a location report when one of the trigger criteria is fulfilled or when it receives a request from the participating MCData function to send a location report. To send the location report the MCData client can use an appropriate SIP message that it needs to send for other reasons, or it can include the location report in a SIP MESSAGE request.

To send a location report, the MCData client includes in the SIP MESSAGE request an application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in clause D.4. The MCData client populates the elements in accordance with its reporting configuration. Further location information may also be included in the P-Access-Network-Info header field.

[TS 24.282 clause 17.3.2]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata";

2)	a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml"; and

3)	an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Configuration> root element included in the <location-info> root element;

the MCData client:

1)	shall store the contents of the <Configuration> elements;

2)	shall set the location reporting triggers accordingly; and

3)	shall start the minimumReportInterval timer.

[TS 24.282 clause 17.3.3]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata";

2)	a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml"; and

3)	an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Request> element included in the <location-info> root element;

the MCData client:

1)	shall send a location report as specified in subclause 17.3.4; and

2)	shall reset the minimumReportInterval timer.

[TS 24.282 clause 17.3.4.1]

If a location reporting trigger fires, the MCData client checks if the minimumReportInterval timer is running. If the timer is running the MCData client waits until the timer expires. When the minimumReportInterval timer expires, the MCData client:

1)	shall, if any of the reporting triggers are still true, send a location information report as specified in subclause 17.3.4.2.

If the MCData client receives a location information request as specified in subclause 17.3.3, the MCData client shall send a location report as specified in subclause 17.3.4.2.

[TS 24.282 clause 17.3.4.2]

If the MCData client needs to send a SIP request anyway (i.e. for reasons other than explicit location reporting request or the firing of a configured location trigger), the MCData client:

1)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body and in the <location-info> root element the MCData client shall include:

a)	a <Report> element and, if the Report was triggered by a location request, include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, if triggers have fired, where each element is set to the value of the <Trigger-Id> attribute associated with the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired, if at least one trigger has fired;

2)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer; and

3)	shall reset all triggers.

If the MCData client does not need to send a SIP request for reasons other than explicit location reporting request or the firing of a configured location trigger, the MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6]. The MCData client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml";

3)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, if triggers have fired, where each element is set to the value of the <Trigger-Id> attribute associated with the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired, if at least one trigger has fired;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcdata along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [8];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

6)	shall reset all triggers; and

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

6.5.2.3	Test description

6.5.2.3.1	Pre-test conditions

Test configuration:

-	Multi-cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.2 with 3 cells according to the initial conditions of procedure 'MCX communication in E-UTRA / Change of cells' as described in TS 36.579-1 [2] clause 5.4.9.

-	GNSS simulator to simulate a location.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 to provide a location, as defined in TS 36.508 [24] Table 4.11.2-2, step 1 of scenario #4.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.5.2.3.2	Test procedure sequence

Table 6.5.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 2-4 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to configure location reporting with the non-emergency minimumIntervalLength timer set to 25s. NOTE: The SS sends a location reporting configuration containing the cell change trigger criteria and the minimum report interval. Provision of serving and neighbouring cells ECGIs is requested in addition to the Location Information. | - | - | - | - |
| 5 | SS starts timer=25 sec (the non-emergency minimumReportInterval interval length defined in the SIP MESSAGE sent in step 3 as minimumIntervalLength). | - | - | - | - |
| 6 | The procedure ‘MCX communication release’ as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 7 | Trigger the GNSS simulator to start step 2 of Scenario #4 to simulate the UE moving to location #7 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 8 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #7. | - | - | - | - |
| 9 | Make the SS simulate cell change (same PLMN) as described in TS 36.579-1 [2], clause 5.4.9 'MCX communication in E-UTRA / Change of cells' steps 1 and 2. The test sequence below shows only the MCData relevant messages exchanged. NOTE: Cell 2 and Cell 1 will be active. | - | - | - | - |
| 10 | Timer=25 sec expires. | - | - | - | - |
| 11-13 | Check: Does the UE (MCData Client) correctly perform steps 1a1-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to send a SIP MESSAGE containing the Cell change trigger identification along with the location information report, the current location coordinates as specified in step 3, and, the ECGIs of Cells 1 and 2? | - | - | 1 | P |
| 14-15 | Steps 2-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to configure location reporting. NOTE: The SS sends a location reporting configuration containing the PLMN change trigger criteria, and, periodic trigger criteria with PeriodicReport reporting interval set to 60 sec and the minimum report interval. Provision of cells ECGIs is requested in addition to the Location Information. The minimumIntervalLength time is set to 10 sec. | - | - | - | - |
| 16 | SS starts timer=60 sec (the PeriodicReport interval defined in the SIP MESSAGE sent in step 14). NOTE: The value of 60 sec has been chosen arbitrary however if the behaviour described in steps 18-23 takes longer than 60 sec then this value needs to be adjusted. | - | - | - | - |
| 17 | The procedure ‘MCX communication release’ as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 18 | Trigger the GNSS simulator to start step 4 of Scenario #4 to simulate the UE moving to location #8 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 19 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #8. | - | - | - | - |
| 20 | Make the SS simulate PLMN change as described in TS 36.579-1 [2], clause 5.4.9 'MCX communication in E-UTRA / Change of cells' steps 3 and 4. The test sequence below shows only the MCData relevant messages exchanged. NOTE: Cell 4 only will be active. | - | - | - | - |
| 21-23 | Check: Does the UE (MCData Client) correctly perform steps 1a1-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to send a SIP MESSAGE containing the PLMN change trigger identification along with the location information report, the current location coordinates as specified in step 14, and the ECGI of cell 4 (the only cell being active at this moment of time)? NOTE: The minimumReportInterval time (set in step 14 as minimumIntervalLength = 10 sec.) has already expired latest at the end of step 18. | - | - | 2 | P |
| 24 | SS cancels Timer=60 sec. | - | - | - | - |
| 25 | SS starts Timer=60 sec. | - | - | - | - |
| 26 | The procedure ‘MCX communication release’ as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 27 | Timer=60 sec expires | - | - | - | - |
| 28 | Check: Does the UE (MCData Client) correctly perform the procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to send a SIP MESSAGE containing the periodic trigger identification along with location information report after the non-emergency minimum interval length expired? NOTE: The minimumReportInterval time (set in step 14 as minimumIntervalLength = 10 sec.) has already expired before step 27. | - | - | 3 | P |

6.5.2.3.3	Specific message contents

Table 6.5.2.3.3-1: SIP MESSAGE from the SS (Step 3, Table 6.5.2.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body | MIME body not including MCPTT-Info and not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type being "application/vnd.3gpp.mcptt-info+xml" or "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.2.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.2.3.3-3 |  |  |  |

Table 6.5.2.3.3-2: MCData-Info in SIP MESSAGE (Table 6.5.2.3.3-1, Table 6.5.2.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |

Table 6.5.2.3.3-3: Location-Info in SIP MESSAGE (Table 6.5.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| MbmsSaId | not present | Provision of information in the report is not requested. |  |  |
| MbsfnArea | not present | Provision of information in the report is not requested. |  |  |
| minimumIntervalLength | "25" | the value in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| MbmsSaId | not present | Provision of information in the report is not requested. |  |  |
| MbsfnArea | not present | Provision of information in the report is not requested. |  |  |
| minimumIntervalLength | "30" | the value in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange |  |  |  |  |
| AnyCellChange |  |  |  |  |
| TriggerID | "CELLCHANGE" |  |  |  |

Table 6.5.2.3.3-4: SIP MESSAGE from the UE (Step 12, Table 6.5.2.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.2.3.3-5 |  |  |  |

Table 6.5.2.3.3-5: Location-Info in SIP MESSAGE (Table 6.5.2.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType | "NonEmergency" |  |  |  |
| TriggerID | "CELLCHANGE" |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | Encrypted - The E-UTRAN Cell Global Identification (ECGI) of Cell 2 | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| NeighbouringEcgi | Encrypted - The ECGI of Cell 1 | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| CurrentCoordinate |  | The location simulated by the GNSS simulator at the time of transmission of the message. |  |  |
| longitude | Encrypted - The longitude value as specified for location number #7 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00016 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted - The latitude value as specified for location number #7 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00013 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |

Table 6.5.2.3.3-6: SIP MESSAGE from the SS (Step 14, Table 6.5.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.2.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.2.3.3-7 |  |  |  |

Table 6.5.2.3.3-7: Location-Info in SIP MESSAGE (Table 6.5.2.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| MbmsSaId | not present | Provision of information in the report is not requested. |  |  |
| MbsfnArea | not present | Provision of information in the report is not requested. |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| MbmsSaId | not present | Provision of information in the report is not requested. |  |  |
| MbsfnArea | not present | Provision of information in the report is not requested. |  |  |
| minimumIntervalLength | "70" | the value in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| PLMNChange |  |  |  |  |
| AnyPLMNChange |  |  |  |  |
| TriggerID | "ANY PLMN" |  |  |  |
| PeriodicReport |  |  |  |  |
| extension base | "60" | the value in seconds of the time which determines the periodic provision of the report |  |  |
| TiggerID | "PERIODIC" |  |  |  |

Table 6.5.2.3.3-8: SIP MESSAGE from the UE (Step 22, Table 6.5.2.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.2.3.3-9 |  |  |  |

Table 6.5.2.3.3-9: Location-Info in SIP MESSAGE (Table 6.5.2.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType | "NonEmergency" |  |  |  |
| TriggerID | "ANY PLMN" |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | Encrypted - The ECGI of Cell 4 | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| NeighbouringEcgi | not present | Only Cell 4 is active at this moment of time (see TS 36.579-1 [2], clause 5.4.9) |  |  |
| CurrentCoordinate |  | The location simulated by the GNSS simulator at the time of transmission of the message. |  |  |
| longitude | Encrypted - The longitude value as specified for location number #8 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00016 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted - The latitude value as specified for location number #8 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00013 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |

Table 6.5.2.3.3-10: SIP MESSAGE from the UE (Step 28, Table 6.5.2.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.2.3.3-11 |  |  |  |

Table 6.5.2.3.3-11: Location-Info in SIP MESSAGE (Table 6.5.2.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType | "NonEmergency" |  |  |  |
| TriggerID | "PERIODIC" |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | Encrypted - The ECGI of Cell 4 | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| NeighbouringEcgi | not present | Only Cell 4 is active at this moment of time (see TS 36.579-1 [2], clause 5.4.9) |  |  |
| CurrentCoordinate |  | The location simulated by the GNSS simulator at the time of transmission of the message. |  |  |
| longitude | Encrypted - The longitude value as specified for location number #8 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00016 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted - The latitude value as specified for location number #8 defined in TS 36.508 [24] Table 4.11.2-3 +/- 0.00013 degrees. | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |

<END OF CHANGES>