3GPP TSG-RAN5 Meeting #103	R5-243270r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0062 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 5.9 Location based functional alias status change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no existing test case for MCData Location based functional alias status change |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 5.9 Configuration / Functional Alias / Location based functional alias status change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices could fail in the commercial field without testing the capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: In step 7 and 13, "The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection." changed to "The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection." and NOTE 1 removed. |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 5.9	Configuration / Functional Alias / Location based functional alias status change

5.9.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service }

ensure that {

when { UE enters into or exits from a location area that triggers one or more functional aliases to be activated according to the MCData user profile document }

then { UE (MCData Client) sends a SIP PUBLISH message to activate a functional alias }

}

(2)

with { UE (MCData Client) registered and authorised for MCData Service and having already activated a functional alias }

ensure that {

when { UE enters into or exits from a location area that triggers one or more functional aliases to be deactivated according to the MCData user profile document }

then { UE (MCData Client) sends a SIP PUBLISH message to deactivate a functional alias }

}

5.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.282clause 22.2.1.4, 22.2.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.282, clause 22.2.1.4]

If a location criterion for functional alias activation or de-activation is met, the MCData client shall initiate the functional alias status change procedure as specified in subclause 22.2.1.2.

[TS 24.282, clause 22.2.1.2]

In order:

-	to indicate that an MCData user requests to activate one or more functional aliases;

-	to indicate that the MCData user requests to deactivate one or more functional aliases;

-	to refresh indication of an MCData user interest in one or more functional aliases due to near expiration of the expiration time of a functional alias with the status set to the "activated" state received in a SIP NOTIFY request in subclause 22.2.1.3;

-	to indicate that the MCData client entering into or exiting from a location area triggers one or more functional aliases to be activated;

-	to indicate that the MCData client entering into or exiting from a location area triggers one or more functional aliases to be deactivated; or

-	any combination of the above;

the MCData client shall generate a SIP PUBLISH request according to TS 24.229 [5], IETF RFC 3903 [34], and IETF RFC 3856 [39].

When the MCData user requests to deactivate a functional alias, the MCData client shall first check the <manual-deactivation-not-allowed-if-location-criteria-met> element within the <anyExt> element of the <entry> element corresponding to the functional alias within the <FunctionalAliasList> list element of the <anyExt> element of the <OnNetwork> element of the MCData user profile document (see the MCData user profile document in TS 24.484 [12]). If the functional alias has been activated due to a location area trigger and the <manual-deactivation-not-allowed-if-location-criteria-met> element is set to a value of "true", the MCData client shall suppress the MCData user’s request.

NOTE 1:	If the request is suppressed, a notification message can be displayed to the user.

In the SIP PUBLISH request, the MCData client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCData function serving the MCData user;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall include the <mcdata-request-uri> element set to the MCData ID of the MCData user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

4)	if the MCData client requests to activate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [34], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

5)	if the MCData client requests to deactivate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [34], to zero; and

NOTE 3:	Activation and deactivation of functional alias cannot be performed with the same PUBLISH request.

6)	shall include an application/pidf+xml MIME body indicating per-user functional alias information according to subclause 22.3.1. In the MIME body, the MCData client:

a)	shall include all functional aliases where the MCData user requests activation for the MCData ID;

b)	shall include the MCData client ID of the targeted MCData client;

c)	shall not include the "status" attribute and the "expires" attribute in the <functionalalias> element;

d)	if the MCData client has received an indication that take over of a functional alias is possible and intends to take over a functional alias, shall include a <take-over> child element set to "true"; and

e)	shall set the <p-id-fa> child element of the <presence> root element to a globally unique value.

The MCData client shall send the SIP PUBLISH request according to TS 24.229 [5].

5.9.3	Test description

5.9.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

-	GNSS simulator to simulate a location.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 to provide a location, as defined in TS 36.508 [24] Table 4.11.2-2, step 1 of scenario #4.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

5.9.3.2	Test procedure sequence

Table 5.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 2 | Trigger the GNSS simulator to start step 2 of Scenario #4 to simulate the UE moving to location #7 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 3 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #7. | - | - | - | - |
| 4-6 | Check: Does the UE (MCData Client) correctly perform steps 2a1-4 of procedure 'UE initiated MCX functional alias status change' as described in TS 36.579-1 [2] Table 5.3.37.3-1 to change the status of a functional alias to "activated"? NOTE: The UE (MCData Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message and then the RRC connection is released. | - | - | 1 | P |
| 7 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 8 | Trigger the GNSS simulator to start step 4 of Scenario #4 to simulate the UE moving to location #8 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 9 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #8. | - | - | - | - |
| 10-12 | Check: Does the UE (MCData Client) correctly perform steps 2a1-4 of procedure 'UE initiated MCX functional alias status change' as described in TS 36.579-1 [2] Table 5.3.37.3-1 to change the status of a functional alias to "not activated"? NOTE: The UE (MCData Client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message and then the RRC connection is released. | - | - | 2 | P |
| 13 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.9.3.3	Specific message contents

Table 5.9.3.3-1: SIP PUBLISH from the UE (step 11, Table 5.9.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3.37.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.3.37.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.282 [31] clause 22.2.1.2 |  |
| value | "0" |  |  |  |

<END OF CHANGES>