3GPP TSG-RAN5 Meeting #103	R5-243314r1

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | 0007 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of polynomial interpolation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_FR1_TRP_TRS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During RAN5#102, it was suggested to provide some visual clarifications of the polynomial interpolation approach |  |  |  |  |  |  |  |  |
| Summary of change: |  | A figure was added to visually clarify and explain the polynomial interpolation approach |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less clarity on the optional interpolation approach |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.3.1.1, A.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-243314r1: added a clarification note in A.4.2.6: |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

### A.3.3.3	TRS Test procedure

The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step in both theta () and phi () axes using any of the measurement grids and quadratures options outlined and applicable to TRS in Table A.4.2.12-1.

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [11]).

For TRS measurement, the evaluations shall be performed at maximum transmit power.

The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the positioning guideline defined in Clause 6.

2.	Connect the SS with the DUT through the measurement antenna.

3.	Follow steps 1 through 4 in section 7.3.2.4.2 of TS 38.521-1 [11], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

The TRS value is calculated using the equation outlined in Clause A.3.5.2. The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If an alternate EIS search procedure, detailed in A.3.3.3.1, is applied to assess the EIS at each grid point, no correction of TRS shall be applied.

This TRS test procedure is common to both SA and EN-DC measurements. The detailed UE configurations for TRS test in SA and EN-DC mode are specified in Clause A.2.

#### A.3.3.3.1	Alternate EIS Search Procedure

The following alternate EIS search procedure methods can be used given that the corresponding uncertainty term as described in A.4.2.6 is considered.

##### A.3.3.3.1.1	EIS linearization using polynomial interpolation

An approach to approximate the sensitivity threshold is to empirically determine the polynomial interpolation based on the DUT sensitivity response, which is characterized with a fine step size using following this procedure for each channel:

1.	At the first grid point, or at any arbitrary reference initial position, an EIS search is performed with a very fine power step size (e.g. 0.1dB) capturing the DL power vs. Throughput for a sufficient DL power range (e.g. 1dB) around the sensitivity threshold.

2.	Based on the results in step 1, curve fitting techniques are used to obtain the best fit n degree polynomial characterizing the DUT sensitivity response

3.	The EIS measurement per grid point is performed as described in clause 7.2.1.1.1.4.2 using the maximum power step size allowed (i.e. 0.5dB), but the final EIS value per grid point is interpolated using the polynomial coefficients calculated in step 2.

4.	All of the resulting EIS values per grid point, calculated as described in step 3, are integrated to TRS as defined in Annex A.4.5.2.

The polynomial coefficients calculated in step 2 shall be reported together with the TRS result.

The default and alternate approaches are visualized in Figure A.3.3.3.1.1-1 for a set of throughput measurement results with a downlink power step size of 0.5 dB near the sensitivity level. For the classical approach, the last passing downlink power level/EIS is recorded (X plotted with green dot). For the optional EIS linearization using polynomial interpolation approach, a polynomial curve, determined earlier, is applied to the last passing (green) and failing (red) DL power levels and the interpolated EIS at the 95% maximum throughput (blue star) is recorded.

Figure A.3.3.3.1.1-1: Discrete TP measurements and EIS linearization using polynomial interpolation

<<< Skip Unchanged Sections >>>

### A.4.2.6	Sensitivity measurement: output level step resolution

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, the final power step resolution represents an asymmetric uncertainty contribution that can be corrected since this uncertainty is device and test system independent. The lab shall correct the TRS results by -0.5·output level step resolution of the final power step search and note the correction in the test report.

Alternatively, if the alternate EIS search procedure is applied as described in A.3.3.3.1 to assess the EIS at each grid point, no correction of TRS shall be applied and the no uncertainty contribution shall be considered.

Please note that the digital error rater uncertainty is included in the random uncertainty, see A.4.2.13.

<<< END OF CHANGES >>>