# Title:	Test Tolerances for FR1 NR-U L1-RSRP absolute accuracy tests

# Source:	Qualcomm

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR1 NR-U L1-RSRP absolute measurement accuracy tests.

The test cases from TS 38.533 are:

- 10.5.4.1, EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA

This document describes the process to derive the Test Tolerances, taking 10.5.4.1 as basis for the analysis. The other test cases all have identical power settings and requirements. Thus, there is no need for a separate analysis. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v18.5.0 Annex A.

<< Extract from TS 38.133 >>

#### A.10.5.4.1	SSB based L1-RSRP measurement

##### A.10.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.10.5.4.1.1-1.

Table A.10.5.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

##### A.10.5.4.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 are given in Table A.10.5.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.10.5.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.10.5.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1,2 |  | freq1 | freq1 |
| DL CCA model |  | 1,2 |  | As specifieed in A.3.26.2.1 | As specifieed in A.3.26.2.1 |
| UL CCA model |  | 1,2 |  | As specified in A.3.26.2.2 | As specified in A.3.26.2.2 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD Configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| BWchannel |  | 1,2 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| PDSCH Reference measurement channel |  | 1,2 |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  | 1,2 |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  | 1,2 |  | CCR.1.1 CCA | CCR.1.1 CCA |
| SSB configuration for Semi-static channel access |  | 1,2 |  | SSB.3 CCA | SSB.3 CCA |
| SSB configuration for Dynamic channel access |  | 1,2 |  | SSB.4 CCA | SSB.4 CCA |
| OCNG Patterns |  | 1,2 |  | OP.1 | OP.1 |
| TRS configuration |  | 1,2 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1,2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1,2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration |  | 1,2 |  | DBT.1 | DBT.1 |
| reportConfigType |  | 1,2 |  | periodic | periodic |
| reportQuantity |  | 1,2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1,2 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1,2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1,2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  | 1,2 |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/15kHz | -94.65 | -113 |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -91.65 | -110 |
|  |  | 1,2 | dB | 10 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -81.65 | -113 |
| IoNote3 | NR_TDD_FR1_I | 1,2 | dBm/ 38.16MHz | -50.19 | -77.19 |
|  |  | 1,2 | dB | 10 | -3 |
| Propagation condition |  | 1,2 |  | AWGN | AWGN |
| Antenna configuration |  | 1,2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

##### A.10.5.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

# 3. Discussion

The test has 2 subtests: Test 1 and Test 2. To analyse the test case it is helpful to visualise these as pie-charts showing the distribution of power, as below:

The subtests are designed to test the UE at two points in the level range over which the absolute L1-RSRP accuracy requirement applies:

- Test 1 applies at the highest power where the reporting accuracy is +/-8.5dB, Io just below -50dBm

- Test 2 applies at the lowest power where the reporting accuracy is +/-5dB, just above Ref. sensitivity

In Test 2 the Ês/Iot is set near the bottom of the range of Ês/Iot   -3 dB given in the core requirement TS 38.133. The UE measures and reports the L1-RSRP.

There are two test configurations according to Table A.10.5.4.1.1-1. Test Configurations 1, 2 have identical power settings, so they will be analysed together.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.10.5.4.1.1-2 and A.10.1.33.1.1-1 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Signalled parameters including the threshold are also copied. For this test, the key parameters are:

- Noc at frequency 1

- Cell 1 Es / Noc

All other parameters such as SSB_RP, SSB Es/Iot, etc. are device and not independently settable by the test system.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The Test system provides 1 cell on 1 frequency, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency 1, Noc uncertainty: ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 ±0.3 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 ±0.8 dB for PRBs #RBJ-RBJ+19

The uncertainties appear in section c) of the accompanying spreadsheet.

In this test the UE measures the L1-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19.

In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

Two sets of parameters are therefore given. The set averaged over the configured bandwidth have similar values to those already used for other tests. The set averaged over PRBs # RBJ-RBJ+19 have wider values, but constraining the deviation enough not to widen the L1-RSRP reporting range too much.

We note also that the outcome of this test is a range of allowed L1-RSRP values reported by the UE. The UE reporting accuracy is therefore taken into account when determining the test limits, although it is not an uncertainty of the test system itself.

This choice forms a minimum set (separately for PRBs #RBJ-RBJ+19, and for “averaged over BWConfig“), so the superposition principle can be applied if necessary.

The absolute level of Noc directly affects the UE’s L1-RSRP measurement, and is specified for frequency bands up to 6GHz.

The UE measures the L1-RSRP and reports it. The absolute measurement accuracy is given in TS 38.133:

### 10.1.33	L1-RSRP accuracy requirements under CCA

#### 10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1

##### 10.1.33.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement under CCA.

The accuracy requirements in Table 10.1.33.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.10.1 for a corresponding Band for each relevant SSB.

Table 10.1.33.1.1-1: SSB based L1-RSRP absolute accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| ±5.0 | ±9.5 | -3 | NR_CCA_FR1_I | -117 | -114 | N/A | -70 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| ±8.5 | ±11.5 | -3 | NR_CCA_FR1_I | N/A | N/A | -70 | -50 |
|  |  |  | NR_CCA_FR1_J |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The Io is in different ranges for each subtest, so the accuracy levels is selected differently for each subtests as indicated in Section 3 of this document. The absolute accuracy requirement refers to L1-RSRP side conditions in TS 38.133 Annex B.2.11:

## B.2.11	Conditions for NR L1-RSRP reporting under CCA

### B.2.11.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs under CCA: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.11.1-1 for NR cells under CCA.

Table B.2.11.1-1: Conditions for SSB based L1-RSRP measurements under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -120 | -117 | -3 |
|  | NR_CCA_FR1_J | -119.5 | -116.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The SSB Ês/Iot side condition is ≥ -3dB. Band group NR_CCA_FR1_J gives the highest SSB_RP side condition of -116.5dBm/30kHz.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (L1-RSRP reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each cell they are the SSB Es/Iot range, the SSB_RP power range, and the Io power range, over which the UE meets the specified L1-RSRP reporting accuracy. The 4 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. In this RRM test case there is usually a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

For example, for Cell 1, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 SSB_RP, so the sensitivity factor is 1.000. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 1 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 1 Es/Noc has an effect on Freq 1 Io which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example,

- In Test 2, the effect of Cell 1 Es/Noc uncertainty on Freq 1 Io  is x 0.334

These factors can also be derived intuitively, by considering the fraction of Cell 1 power compared to the total power (Cell and AWGN) making up Freq 1.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE L1-RSRP absolute measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE L1-RSRP reporting accuracy are combined, the root-sum square of test system uncertainties is added arithmetically to the UE L1-RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

For Test Configuration 1, we observe that the SSB Es/Iot for Test 2 is very close to the lower limit of -3 dB, and that the test equipment uncertainties could take the UE outside the allowed range. Therefore, the SSB Es/Iot for Test 2 is increased by an amount sufficient to achieve SSB Es/Iot >= -3dB.

For Test Configuration 3, we observe that the nominal Io for Test 1 is very close to the -50dBm upper limit of the +/-8.5dB UE reporting accuracy range, and that the test equipment uncertainties could take the UE outside the allowed range. The Noc for Test 1 is therefore decreased by an amount sufficient to achieve Io <= -50dBm. Note that for the other Test 2 the Test system uncertainty will not take the minimum or maximum Io values outside their limits, so no offset is required. We also observe that the SSB Es/Iot for Test 2 is very close to the lower limit of -3 dB, and that the test equipment uncertainties could take the UE outside the allowed range. Therefore, the SSB Es/Iot for Test 2 is increased by an amount sufficient to achieve SSB Es/Iot >= -3dB.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

For test configuration 1:

The SSB Es/Iot for Test 2 is increased by an amount sufficient to achieve SSB Es/Iot >= -3dB. The actual offset is 0.8 dB. The value is found empirically by observing the minimum/maximum parameter values in step g).

For test configuration 3:

The Noc for Test 1 is decreased by an amount sufficient to achieve Io < -50dBm. The actual offset required is 1.35dB for Test 1 only. The value is found empirically by observing the minimum/maximum parameter values in step g).

The SSB Es/Iot for Test 2 is increased by an amount sufficient to achieve SSB Es/Iot >= -3dB. The actual offset is 0.8 dB. The value is found empirically by observing the minimum/maximum parameter values in step g).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (SSB Es/Iot range, SSB_RP power range, and the Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

For this test, the verdict is based on the L1-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (L1-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of L1-RSRP values that a conformant UE could report.