3GPP TSG-RAN5 Meeting #103	R5-243372r1

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3233 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annex E and F for NR-U RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several NR-U test cases have missing Annex E and F content, many of which have TT analysis available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Annex E and F for many NR-U test cases, both section 10 and section 11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.8, E.9, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: For TC 10.5.1.1 in Table F.1.3.2-10, corrected Test number. Also updated MTSU for TC10.5.3.1 TC10.5.3.3 TC10.5.3.2 and TC11.6.3.2 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

# E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10

Table E.8-1 defines the cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in chapter 10 of this test specification.

Table E.8-1: Cell configuration mapping for EN-DC FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 10.2.1.1 | EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.2.2.1 | EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.1.2 | EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.1.3 | EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.2.1 | EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.3.3.1 | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.3.3.2 | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.3.3.3 | EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.3.4.1 | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.4.2 | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.5.1 | EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.5.2.1 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.5.2.2 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.3.5.3.1 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.3.6.1 | EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.1.1 | EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.4.1.4 | EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.4.2.1 | EN-DC FR1 RSSI measurement reporting | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.2 | EN-DC FR1 Channel occupancy measurement reporting | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.3 | EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.4 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.5 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.6 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.7 | EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.8 | EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.9 | EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.2.10 | EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.4.3.1 | EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.3.2 | EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.3.3 | EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.3.4 | EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.4.1 | EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.4.2 | EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.4.3 | EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.4.4.4 | EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |
| 10.5.1.1 | EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.1.2 | EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.2.1 | EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.2.2 | EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.3.1 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.3.2 | EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.3.3 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.4.1 | EN-DC FR1 SSB based L1-RSRP measurement | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.5.1 | EN-DC FR1 RSSI measurement accuracy on PSCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.5.2 | EN-DC FR1 RSSI measurement accuracy on SCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.5.5.3 | EN-DC FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.6.1 | EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.6.2 | EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | Intra-band |
|  |  |  |  | NR Cell 10 |  | Inter-band |
| 10.5.6.3 | EN-DC FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |

# E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11

Table E.9-1 defines the cell configuration mapping for SA FR1 Shared Spectrum test cases in chapter 11 of this test specification.

Table E.9-1: Cell configuration mapping for SA FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 11.2.2.3.1 | NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.2.3.2 | NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.3.1.1 |  | NR SA FR1 UE Transmit Timing Test with PCell under DL CCA |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.3.2.1 |  | NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.4.1.2 | NA SA FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA | NR Cell 1 |  |  |  |  |
| 11.4.1.3 |  | NR SA FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.4.3.1 |  | NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | Intra-band |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | Inter-band |  |
| 11.4.3.2 |  | NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | Intra-band |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | Inter-band |  |
| 11.4.3.3 |  | NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | Intra-band |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | Inter-band |  |
| 11.4.4.1 | NR SA FR1 E-UTRAN – NR interruptions during SCell operations with CCA | NR Cell 1 |  |  |  |  |
| 11.4.4.2 | NR SA FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | NR Cell 1 |  |  |  |  |
| 11.4.5.1 | NR SA FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | NR Cell 1 |  |  |  |  |
| 11.4.5.2.1 | NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.4.5.2.2 | NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | NR Cell 1 |  |  |  |  |
| 11.4.5.3.1 | NR SA FR1 DL active BWP switch of Cell with non-DRX | NR Cell 1 |  |  |  |  |
| 11.5.4.1 |  | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.5.4.2 |  | NR SA FR1 SSB based L1-RSRP measurement when DRX is used |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.5.4.3 |  | NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.5.4.4 |  | NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 11.6.1.1 |  | NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 11.6.2.1 |  | NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 11.6.2.2 |  | NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |
| 11.6.3.2 |  | NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |

<< Skipped sections >>

### F.1.1.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.1.2-12: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases for shared spectrum

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 10.2.1.1 EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA | Same as 4.4.1.1 | Same as 4.4.1.1 |
| 10.2.2.1 EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA | Same as 4.4.3.1 | Same as 4.4.3.1 |
| 10.3.1.2 EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 10.3.1.3 EN-DC FR1 Radio link monitoring in-sync test for PSCell under CCAconfigured with SSB-based RLM RS in non-DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 10.3.3.1 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.3.2 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.3.3 EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.4.1 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 10.3.4.2 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 10.3.5.1 EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA | 1.5dB | Measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1 |
| 10.3.5.2.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.5.2.2 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.5.3.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.6.1 EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA | Same as 4.5.7.1 | Same as 4.5.7.1 |
| 10.4.1.1 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA without gaps under non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 10.4.1.4 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA with per-UE gaps under DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 10.4.2.3 EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.4 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.5 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.6 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.7 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.8 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.9 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.10 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.3.1 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.2 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.3 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.4 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.5.1.1 EN-DC FR1 Intra-frequency measurement accuracy on a CCA serving cell | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 10.5.1.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 10.5.2.1 EN-DC FR1 Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.2.1 | Same as 4.7.2.1 |
| 10.5.2.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.2.2.1 | Same as 4.7.2.2.1 |
| 10.5.3.1 EN-DC FR1 Intra-frequency measurement accuracy on PSCC under CCA | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.8 dB Meas PRB Ês3 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 10.5.3.2 EN-DC FR1 Inter-frequency measurement accuracy on PSCC under CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.8 dB Meas PRB Ês3 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 10.5.3.3 EN-DC FR1 Intra-frequency measurement accuracy on SCC under CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 10.5.4.1 EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |

Table F.1.1.2-13: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for shared spectrum

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | 1.5dB | Measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.2.1 | Same as 6.7.2.2.1 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.8 dB meas PRB Ês2 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

<< Skipped sections >>

### F.1.3.2	Measurement of RRM requirements

<< Skipped tables >>

Table F.1.3.2-10: Derivation of test requirements for EN-DC FR1 RRM tests for shared spectrum

| Test |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.2.1.1 EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA | Same as 4.4.1.1 | Same as 4.4.1.1 | Same as 4.4.1.1 |
| 10.2.2.1 EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA | Same as 4.4.1.1 | Same as 4.4.1.1 | Same as 4.4.1.1 |
| 10.3.1.2 EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 10.3.1.3 EN-DC FR1 Radio link monitoring in-sync test for PSCell under CCAconfigured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4A.2.1 |  |
| 10.3.3.1 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | During T1 to T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1 to T3: 0dB 0dB 0dB 0dB | During T1 to T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 10.3.3.2 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 10.3.3.3 EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 10.3.4.1 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in non-DRX mode |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4A.2.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 10.3.4.2 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in DRX mode |  | Same as 10.3.4.1 |  | Same as 10.3.4.1 |  | Same as 10.3.4.1 |  |
| 10.3.5.1 EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.5.2.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.5.2.2 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC |  | Same as 10.3.5.2.1 |  | Same as 10.3.5.2.1 |  | Same as 10.3.5.2.1 |  |
| 10.3.5.3.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.6.1 EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA | Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | -3.6dB 0dB -3dB | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |
| 10.4.1.1 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA without gaps under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 1.2dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +5.20dB Ês3 / Noc: +7.00dB |  |
| 10.4.1.4 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA with per-UE gaps under DRX |  | Same as 10.4.1.1 |  | Same as 10.4.1.1 |  | Same as 10.4.1.1 |  |
| 10.4.2.3 EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |
| 10.4.2.4 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.5 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.6 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.7 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.8 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.9 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.10 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.3.1 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 10.4.3.2 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 10.4.3.3 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 10.4.3.4 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 10.5.1.1 EN-DC FR1 Intra-frequency measurement accuracy on a CCA serving cell |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 2: Noc: -107dBm/30kHz Ês2 / Noc: +1.0dB Ês3 / Noc: 0dB Reported RSRP values: ±6dB |  | Test 1: 0dB 0dB +0.4dB Via mapping Test 2: -1.0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 2: Noc: -118dBm/30kHz Ês2 / Noc: +1.0dB Ês3 / Noc: +0.2dB RSRP_43 to RSRP_55 RSRP_43 to RSRP_55 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |
| 10.5.1.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -103dBm/15kHz Noc2: -111dBm/15kHz Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -103dBm/15kHz Noc1: -111dBm/15kHz Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_47 to RSRP_60 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |
| 10.5.2.1 EN-DC FR1 Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -110dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -110dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |
| 10.5.2.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -112dBm/15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -112dBm/15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_52 to RSRQ_62 depending on operating band |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 10.5.3.1 EN-DC FR1 Intra-frequency measurement accuracy on PSCC under CCA |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported SINR values: ±3.5dB Test 2: Noc: -112dBm/15kHz Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB SINR_31 to SINR_49 Test 2: Noc: -112dBm/15kHz Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 10.5.3.2 EN-DC FR1 Inter-frequency measurement accuracy on PSCC under CCA |  |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -115.5dBm/15kHz Noc2: -115.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -115.5dBm/15kHz Noc2: -115.5dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 10.5.3.3 EN-DC FR1 Intra-frequency measurement accuracy on SCC under CCA |  |  | Same as 10.5.3.1 |  | Same as 10.5.3.1 |  | Same as 10.5.3.1 |  |
| 10.5.4.1 EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA |  |  |  |  |  |  |  |  |
| Test Configuration 1,2 |  |  | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -2.2dB RSRP_38 to RSRP_51 |  |

Table F.1.3.2-11: Derivation of test requirements for SA FR1 RRM tests for shared spectrum

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 10.3.1.2 | Same as 10.3.1.2 | Same as 10.3.1.2 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM RS in non-DRX mode | Same as 10.3.1.3 | Same as 10.3.1.3 | Same as 10.3.1.3 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | Same as 10.3.5.1 | Same as 10.3.5.1 | Same as 10.3.5.1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch of Cell with non-DRX | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 10.5.1.1 | Same as 10.5.1.1 | Same as 10.5.1.1 |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.2 | Same as 10.5.2.2 | Same as 10.5.2.2 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.2 | Same as 10.5.3.2 | Same as 10.5.3.2 |

<< End of Changes >>