3GPP TSG-RAN5 Meeting #103	R5-243415r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0057 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F updates for NTN RF test on Tx on-off time mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Requirements for NTN Tx ON-OFF Time Mask test are incomplete due to missing test tolerances |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex F is updated with MU and TT values and test requirements is also updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN Tx ON-OFF Time Mask Test will stay incomplete due to missing MU and TT values |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3, Annex F.1.2, Annex F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 corrects 38.521-1 test case referred in Annex F table |  |  |  |  |  |  |  |  |

### 6.3.3	Transmit on/off time mask

#### 6.3.3.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3.3.5.

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power as defined in sub-clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

##### <Unchanged sections skipped>

#### 6.3.3.5	Test requirement

The requirement for the power measured in steps 2, 3 and 4 of the test procedure shall not exceed the values specified in Table 6.3.3.5-1.

Table 6.3.3.5-1: General ON/OFF time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz | 20MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 |
| Transmit ON power | Same as Table 6.2.1.5-1 |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth of OFF power is specified in Table 6.3.3.5-2 NOTE 2: TT for each frequency and channel bandwidth of ON power is specified in Table 6.2.1.5-2 |  |  |  |  |

Table 6.3.3.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 40MHz | 1.5 dB |

##### <Unchanged sections skipped>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.3.3 Transmit on/off time mask | Same as clause 6.3.3 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz |  |
| 6.4.1_1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.4.1_2 Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.5.3.1 General spurious emissions | Same as clause 6.5.3.1 in TS 38.521-1 [2]. |  |
| 6.5.3.2 Spurious emission for UE co-existence | Same as clause 6.5.3.2 in TS 38.521-1 [2]. |  |
| 6.5.3.3 Additional spurious emissions | Same as clause 6.5.3.3 in TS 38.521-1 [2]. |  |
| 6.5.4 Transmit intermodulation | Same as clause 6.5.4 in TS 38.521-1 [2]. |  |

##### <Unchanged sections skipped>

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.3.3 Transmit on/off time mask | Same as clause 6.3.3 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz | Minimum requirement + TT |
| 6.4.1_1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4.1_2 Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.5.3.1 General spurious emissions | Same as clause 6.5.3.1 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.3.2 Spurious emission for UE co-existence | Same as clause 6.5.3.2 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.3.3 Additional spurious emissions | Same as clause 6.5.3.3 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.4 Transmit intermodulation | Same as clause 6.5.4 in TS 38.521-1 [2]. | Minimum requirement + TT |

##### <End of changes>