3GPP TSG-RAN5 Meeting #103	R5-243430r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0029 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to nb-ntn demod test case to update min test time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum test time for the NPDSCH and NPDCCH test cases are not pointing to the correct table references.Moreover the legacy test cases from where the GSO specific test cases are defined requires a really long test time. Simulation results for these cases were generated to come up with a more reasonable test time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the simulation results for these test cases, updating the minimum test time table in Annex G.3.5 Also added missing RMC R.NB.6 FDD and R.NB.6-1 FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will remain untestable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3, Annex G.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on outcome of discussion paper R5-243429 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated the WIC |  |  |  |  |  |  |  |  |

Table A.3.12.1.1-1: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for FDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.1 FDD | R.NB.2 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 616 | 392 |
| For Sub-Frame 0,5 | Bits | N/A | 392 |
| For Sub-Frame 4,9 | Bits | Note 2 | 392 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 3 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 4 | 320 |
| Max. Average Throughput | Bps | Note 5 | Note 5 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table A.3.12.2.1-1A: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for FDD legacy TN requirements

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.6 FDD | R.NB.6-1 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 616 | 392 |
| For Sub-Frame 0,5 | Bits | N/A | 392 |
| For Sub-Frame 4,9 | Bits | Note 2 | 392 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 3 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 4 | 320 |
| Max. Average Throughput | Bps | Note 5 | Note 5 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

<<Several sections skipped>>

## G.3.5	Minimum Test time

If a pass fail decision in G.3.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of subframes for FDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS),

then adding inactive subframes to the active ones (e.g. subframe 5 contains no DL payload. For TDD additional subframes contain no DL payload)

then rounding up to full thousand and

then adding a bias of 1000 (BMNSF).

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30% or 70% of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function “final throughput versus level” into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions “final throughput versus level” and “preliminary throughput versus time” are simulation results, which are done individual for each demodulation scenario. HST-scenarios and scenarios with MNAS ≥ 50000 are derived differently.

Figure G.3.5-1: Simulation method to derive minimum test time

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Clause 8.12.1.1.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 3466000 | N/A |
| 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 3609 | 5544000 | N/A |

Table G.3.5-2: Minimum Test time for PDSCH for UE Category NB2

| Clause 8.12.1.1.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 44000 | N/A |

Table G.3.5-3: Minimum Test time for NPDSCH for UE Category NB1, NB2 – NGSO Configuration

| Clause 8.3.1.1.1 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.1 FDD (200kHz, QPSK,1/2, Rep 32) (1x1), NTN-TDLC5-200 |  |  | N/A |
| 2 | R.NB.2 FDD (200kHz, QPSK,1/3, Rep 128) (1x1) NTN-TDLA100-10 |  |  | N/A |

Table G.3.5-4: Minimum Test time for NPDSCH for UE Category NB1, NB2 – GSO Configuration

| Clause 8.3.1.1.2 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.6 FDD (200kHz, QPSK,1/2, Rep 32) (1x1), NTN-TDLC5-200 |  |  | N/A |
| 2 | R.NB.6-1 FDD (200kHz, QPSK,1/3, Rep 128) (1x1) ETU1 |  |  | N/A |

Table G.3.5-5: Minimum Test time for NPDSCH for UE Category NB2 – GSO Configuration

| Clause 8.3.1.1.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.7 FDD (200kHz, QPSK,1/2, Rep 1) (1x1), EPA5 |  |  | N/A |

Table G.3.5-6: Minimum Test time for NPDCCH for UE Category NB1, NB2 – GSO Configuration

| Clause 8.3.1.1.3 Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 1 | R.NB.3 FDD (200kHz, Rep 128) (1x1), EPA5 |  |  | N/A |
| 2 | R.NB.3 FDD (200kHz, Rep 1024) (1x1), ETU1 |  |  | N/A |