3GPP TSG-RAN5 Meeting #103	R5-243746r1

# Fukuoka, Japan, May 20th 2024 - May 24th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3165 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for RedCap RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add MU and TT for following RedCap RRM test cases: - FR1 1Rx RedCap SDT test: 16.2.1.1; - FR2 handover test: 17.3.1.1, 17.3.1.2 - FR2 redirection test: 17.3.2.3.1 - FR2 timing test: 17.4.1.1, 17.4.3.1 - FR2 TCI-state switch test: 17.5.4.1.1, 17.5.4.2.1; - FR2 spatial relation switch test: 17.5.5.1.1, 17.5.5.2.1; - FR2 SS-RSRP accuracy test: 17.7.1.1, 17.7.1.2; - FR2 SS-RSRQ accuracy test: 17.7.2.1, 17.7.2.2; - FR2 L1-RSRP accuracy test: 17.7.3.1, 17.7.3.2; - FR2 SS-SINR accuracy test: 17.7.4.1, 17.7.4.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MU and TT of the test cases mentioned above are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 0739,0740,0741 TS 38.533 CR 3127,3128,3140-3163 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT 38.903 CRs: R5-242906, R5-242907, R5-242908 38.533 TC CRs: R5-242863, R5-242864, R5-242880 - R5-242903 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Changes to 7.2.1.1 and 17.2.1.1 are reversed because corresponding changes are not agreed in RAN4. - Test tolerance for 6.2.1.1, 16.2.1.1 and 16.2.1.2 are updated based on the updated TT analysis. - Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table F.1.1.2-9: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| <Unchanged contents are omitted> |  |  |
| 16.1.2.6 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.2.1.2 NR SA FR1 CG-SDT for 2Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.3.1.1 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| <Unchanged contents are omitted> |  |  |
| 16.7.6.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |

Table F.1.1.2-10: Maximum test system uncertainty for RRM requirements for SA FR2 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |
| <Unchanged contents are omitted> |  |  |
| 17.1.1.3 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.3.1.1 NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |
| 17.3.1.2 NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 | Same as 7.4.1.1 | Same as 7.4.1.1 |
| 17.4.3.1 NR SA FR2 timing advance adjustment accuracy | Same as 7.4.3.1 | Same as 7.4.3.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode | Same as 7.5.5.4 | Same as 7.5.5.4 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.5 | Same as 7.5.5.5 |
| 17.5.4.1.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.1.1 | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.2.1 | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS | Same as 7.5.9.1.1 | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS | Same as 7.5.9.2.1 | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 Event triggered reporting test without gap under non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 |
| <Unchanged contents are omitted> |  |  |
| 17.6.3.4 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 17.7.1.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.1 | Same as 7.7.1.1 |
| 17.7.1.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 17.7.2.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 17.7.2.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.2 | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Same as 7.7.4.1 | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | Same as 7.7.4.2 | Same as 7.7.4.2 |
| 17.7.4.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.1 | Same as 7.7.3.1 |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 | Same as 7.7.3.2 |

<End of modified section 1>

<Start of modified section 2>

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| <Unchanged contents are omitted> |  |  |  |
| 6.2.1 NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) | During T1: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc = -100 dBm/15kHz Es/Noc = 0dB In Signaling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS | During T1: 0 dB 0 dB During T2: 0 dB 0 dB During T3: 0 dB 0 dB During T4: 0 dB 0 dB During T5: 0 dB 0 dB During T6: 0 dB 0 dB In Signaling 0 dB 1 dB for Config 1,2,4 0 dB for Config 3 | During T1: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12 dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14 dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14 dB During T6: Noc = -100 dBm/15kHz Es/Noc = 0 dB In Signaling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -111 dBm/SCS for Config 1,2,4 cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS for Config 3 |
| 6.3.1.1 NR SA FR1 handover with known target cell | TEST CONFIGURATION 1, 2 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
|  | TEST CONFIGURATION 3 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| <Unchanged contents are omitted> |  |  |  |
| 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |
| Test Configuration 3,6 | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |

<End of modified section 2>

<Start of modified section 3>

Table F.1.3.2-7: Derivation of test requirements for NR SA FR1 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -3.11dB Ês2 / Noc: +2.79dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +2.79dB Ês2 / Noc: -3.11dB Qoffsets, n = 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB In signalling -2dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -3.11dB Ês2 / Noc: +2.79dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +2.79dB Ês2 / Noc: -3.11dB Qoffsets, n = -2dB |
| <Unchanged contents are omitted> |  |  |  |
| 16.1.2.6 R SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE | During T1: Noc = -100 dBm/15kHz Es/Noc = 0dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc =-100 dBm/15kHz Es/Noc = 0dB In Signaling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS | During T1: 0 dB 0 dB During T2: 0 dB 0.43 dB During T3: 0 dB 0 dB During T4: 0 dB 0 dB During T5: 0 dB 0 dB During T6: 0 dB -0.43 dB In Signaling 0 dB -5 dB for Config 1,2,4 -2 dB for Config 3 | During T1: Noc = -100 dBm/15kHz Es/Noc = 0dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12.43dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc = -100 dBm/15kHz Es/Noc = -0.43dB In Signaling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -115 dBm/SCS for config 1,2 & 4 -112 dBm/SCS for config 3 |
| 16.2.1.2 NR SA FR1 CG-SDT for 2Rx UE | Same as 6.2.1 | Same as 6.2.1 | Same as 6.2.1 |
| 16.3.1.1 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 12 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 12 dB In signalling A3-Offset 0 dB | 0 dB 0 dB 0 dB 0 dB 0 dB -1.85dB 0 dB 0 dB -1.85dB -1.5dB | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 10.15 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 10.15 dB In signalling A3-Offset -1.5 dB |
| <Unchanged contents are omitted> |  |  |  |
| 16.7.6.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.6.1 | Same as 6.7.6.1 | Same as 6.7.6.1 |

Table F.1.3.2-8: Derivation of test requirements for NR SA FR2 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| <Unchanged contents are omitted> |  |  |  |  |  |
| 17.1.1.3 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.3 | Same as 7.1.1.3 |  | Same as 7.1.1.3 |  |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.5 | Same as 7.1.1.5 |  | Same as 7.1.1.5 |  |
| 17.3.1.1 NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 17.3.1.2 NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.3 | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 |  | Same as 7.3.2.3 |  | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 | Same as 7.4.1.1 |  | Same as 7.4.1.1 |  | Same as 7.4.1.1 |
| 17.4.3.1 NR SA FR2 timing advance adjustment accuracy | Same as 7.4.3.1 |  | Same as 7.4.3.1 |  | Same as 7.4.3.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.1 |  | Same as 7.5.1.1 |  | Same as 7.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.2 |  | Same as 7.5.1.2 |  | Same as 7.5.1.2 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 7.5.1.4 |  | Same as 7.5.1.4 |  | Same as 7.5.1.4 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.1 |  | Same as 7.5.5.1 |  | Same as 7.5.5.1 |
| 17.5.4.1.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 Event triggered reporting test without gap under non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |
| <Unchanged contents are omitted> |  |  |  |  |  |
| 17.6.3.4 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |
| 17.7.1.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.1 |  | Same as 7.7.1.1 |  | Same as 7.7.1.1 |
| 17.7.1.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |
| 17.7.2.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.1 |  | Same as 7.7.2.1 |  | Same as 7.7.2.1 |
| 17.7.2.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.2 |  | Same as 7.7.2.2 |  | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Same as 7.7.4.1 |  | Same as 7.7.4.1 |  | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | Same as 7.7.4.2 |  | Same as 7.7.4.2 |  | Same as 7.7.4.2 |
| 17.7.4.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.1 |  | Same as 7.7.3.1 |  | Same as 7.7.3.1 |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 |  | Same as 7.7.3.2 |  | Same as 7.7.3.2 |

<End of modified section 4>