3GPP TSG-RAN5 Meeting #104	R5-244158

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3271 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.7.17.1.1 for FR1 LTM inter L1-RSRP absolute accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LTM inter-frequency L1-RSRP absolute accuracy requirements, new test case 6.7.17.1.1 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 6.7.17.1.1 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.17.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0446 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### 6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1

##### 6.7.17.1.1	Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1

Editor's Note: This test case is incomplete in following aspects:

-    LTM specific configurations are missing.

-    Test applicability is FFS in TS 38.522.

-    TT analysis is missing.

6.7.17.1.1.1	Test purpose

To verify that the inter-frequency SSB based L1-RSRP absolute measurement accuracy on neighbor cell is within the limits for all bands specified in TS 38.133 [6] clause 10.1.19E.

6.7.17.1.1.2	Test applicability

This test applies to all types of NR UE supporting SSB based inter-frequency L1-RSRP measurements with measurement gaps from release 18 and forward.

6.7.17.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.17.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.17.1.

6.7.17.1.1.4	Test description

6.7.17.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.17.1.1.4.1-1.

Table 6.7.17.1.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.17.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.17.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.17.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations in each supported band Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.7.17.1.1.4.1-2.

Table 6.7.17.1.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.17.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.7.17.1.1.4.1-3.

2. Message contents are defined in clause 6.7.17.1.1.4.3.

3. There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.7.17.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | reportConfigType |  | periodic | Periodic L1-RSRP reporting configured |
|  | L1-RSRP reporting period | slot | 80 |  |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |

6.7.17.1.1.4.2	Test procedure

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically.

There are two tests in the test case, test 1 and test 2:

-	In test 1, test at the highest power level below maximum Io.

-	In test 2, test at the lowest power level above minimum SSB_RP.

Measurement gap pattern configuration defined in Table 6.7.17.1.1.4.1-3 is provided.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Test 1 in Table 6.7.17.1.1.5-1.

3. SS shall transmit an RRCReconfiguration message to configure periodical measurement report.

4. The UE shall transmit RRCReconfigurationComplete message.

5. The UE shall transmit a MeasurementReport message periodically including Cell 2 measurement report with SSB index.

6. SS shall transmit an RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7. The UE shall transmit RRCReconfigurationComplete message.

8. The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 80 slots.

9. After 10s wait from Step 6, the SS shall check the values in the periodic measurement report. The L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. If the value is outside the limits in Table 6.7.17.1.1.5-2 for test configurations 1, 2 and in Table 6.7.17.1.1.5-3 for test configuration 3, or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

10. The SS shall continue checking the L1-RSRP report transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

11. The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

13. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

14.	Set the parameters according to each test in Table 6.7.17.1.1.5-1 as appropriate and repeat steps 3-13.

6.7.17.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.7.17.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 with condition SSB Index Table H.3.1-6 with conditions gapUE and Pattern #0 Table H.3.1-7 with conditions INTER-FREQ, SSB Index |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.17.1.1.4.3-2: ReportConfigNR (periodical) (step3)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ms480 |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.17.1.1.5	Test requirement

Table 6.7.17.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.7.17.1.1.5-1: Cell specific test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell1 | Cell 2 | Cell 1 | Cell 2 |
| SSB GSCN |  | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |
|  |  | 2 |  | TDD |  | TDD |  |
|  |  | 3 |  | TDD |  | TDD |  |
| TDD Configuration |  | 1 |  | N/A |  | N/A |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | 2 |  | SR.1.1 TDD | - | SR.1.1 TDD | - |
|  |  | 3 |  | SR.2.1 TDD | - | SR.2.1 TDD | - |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  |  | 3 |  | CR.2.1 TDD | - | CR.2.1 TDD | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  |  | 3 |  | TRS.1.2 TDD | - | TRS.1.2 TDD | - |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65+TT |  | (Noc for Channel 2 +8dB) | -117+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5+TT |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65+TT |  | (Noc for Channel 2 +8dB) | -117+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65+TT |  | (Noc for Channel 2 +8dB) | -114+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -113.5+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -111.5+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -111+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -110.5+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -107.5+TT |
| EsIot |  | 1~3 | dB | 10+TT | 10+TT | 13+TT | -3+TT |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65+TT |  | (RSRP for Cell 2 +24dB) | -120+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -119.5+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -119+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -118.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -118+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -117.5+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -117+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -116.5+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -113.5+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65+TT |  | (RSRP for Cell 2 +24dB) | -117+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5+TT |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28+TT |  | (Io for Channel 2 +19.75dB) | -87.28+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -86.78+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -86.28+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -85.78+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -85.28+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -84.78+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -84.28+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -83.78+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -80.78+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19+TT |  | (Io for Channel 2 +19.75dB) | -81.19+TT |
|  | NR_FDD_FR1_B |  |  |  |  |  | -80.69+TT |
|  | NR_TDD_FR1_C |  |  |  |  |  | -80.19+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -79.69+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -79.19+TT |
|  | NR_FDD_FR1_F |  |  |  |  |  | -78.69+TT |
|  | NR_FDD_FR1_G |  |  |  |  |  | -78.19+TT |
|  | NR_FDD_FR1_H |  |  |  |  |  | -77.69+TT |
|  | NR_FDD_FR1_N |  |  |  |  |  | -74.69+TT |
| EsNoc |  | 1~3 | dB | 10+TT | 10+TT | 13+TT | -3+TT |
| Propagation condition |  | 1~3 |  | AWGN |  | AWGN |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51, and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

Each L1-RSRP measurement report of Cell 2 in each test in Table 6.7.17.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.17.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.17.1.1.5-3 for test configuration 3.

Table 6.7.17.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Highest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Highest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1. |  |  |  |

Table 6.7.17.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Highest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Highest reported value (Cell 2) | TBD | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | TBD |
|  |  | Bands NR_FDD_FR1_B | TBD |
|  |  | Bands NR_TDD_FR1_C | TBD |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | TBD |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | TBD |
|  |  | Bands NR_FDD_FR1_F | TBD |
|  |  | Bands NR_FDD_FR1_G | TBD |
|  |  | Bands NR_FDD_FR1_H | TBD |
|  |  | Bands NR_FDD_FR1_N | TBD |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1. |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<< End of changes >>